CISCO NSO Service Provisioning

Acceptance Test Procedure Document

Environment: CFS VIP 172.28.2.26 (172.28.2.11–13)   RFS VIP 172.28.2.27 (172.28.2.14–16)

LSA Deployment (CFS/RFS) — VIP-Based Cluster Access

Cisco Network Services Orchestrator (NSO v6.4.11)

CLI and Web UI Test Procedures — Tejas EMS Integration

Revision History

| Revision No. | Date of Submission | Description of Revision | Prepared By | Date of submission for Review | Review completed Date |
| --- | --- | --- | --- | --- | --- |
| 1.0 | 28th Jul26 | Initial version — Production Environment (3 CFS / 3 RFS) |  | 28-07-2026 |  |
| 1.1 | 16th Aug26 | Updated to reflect VIP-based cluster access — CFS VIP 172.28.2.26 (cfs-node-11/12/13) and RFS VIP 172.28.2.27 (rfs-node-1/2/3) |  | 16-08-2026 |  |
| 1.2 | 16th Aug26 | Removed RFS Lock/Unlock test cases — not applicable under VIP-based access; replaced with a single end-to-end Postman-to-NSO provisioning/deletion verification test case |  | 16-08-2026 |  |

| Prepared by | Reviewed By | Approved by |
| --- | --- | --- |

# Table of Contents

Table of Contents	3

1 Introduction	4

Site / Node Details	4

2 Test Cases summary	5

3. NSO Service Configuration and Lifecycle Management	6

Test Case 1: Provisioning a New Service Instance	6

Test Case 2: Committing the Service Configuration	6

Test Case 3: Deleting a Service Instance	7

4. The Cluster Architecture — Nodes, Node Type, IP, VIP	8

4.1 Cluster Node Reference Table	8

5 User Interface and System Verification	9

5.1 Web Portal and EMS Verification	9

Test Case 4: Postman Request Submission to NSO	9

Test Case 5: CFS VIP — Service Visibility	10

Test Case 6: CFS Cluster — High Availability Verification	11

Test Case 7: RFS VIP — Service Visibility	11

Test Case 8: RFS Cluster — High Availability Verification	12

Test Case 9: Tejas EMS — ONT Provisioning Verification	13

Test Case 10: Tejas EMS — ONT Provisioning Detail Verification	14

Test Case 11: CFS VIP — Service Deletion Request	14

Test Case 12: Deleted in CFS	15

Test Case 13: Deleted in RFS	15

Test Case 14: Unprovisioned in Tejas EMS	16

# 1 Introduction

This acceptance test plan provides the tests performed on the Cisco NSO (Network Services Orchestrator) platform to evaluate the automation, provisioning, and management functionalities offered for the T-Fiber network, covering the LSA (Layered Service Architecture) deployment across RFS (Resource Facing Service) and CFS (Customer Facing Service) layers.

The acceptance testing included verification of NSO system status, user access, device inventory, package operational status, service configuration, service commit operations, service deletion, and service visibility across CFS, RFS, and Tejas EMS.

In this production environment, the CFS layer is deployed as a 3-node cluster (cfs-node-11, cfs-node-12, cfs-node-13 at 172.28.2.11–13) sitting behind a Virtual IP, CFS VIP 172.28.2.26, replacing the single-node CFS used in the test environment. Similarly, the RFS layer is deployed as a 3-node cluster (rfs-node-1, rfs-node-2, rfs-node-3 at 172.28.2.14–16) behind RFS VIP 172.28.2.27. All client and integration traffic (SCCM, Postman/RESTCONF, Web UI) is directed at the VIP address, which redirects to the currently active node in each cluster, providing high availability without requiring clients to target individual node IPs.

## Site / Node Details

| CFS VIP (Virtual IP) | 172.28.2.26 — front-facing address for the CFS cluster |
| --- | --- |
| CFS Cluster Nodes | cfs-node-11 (172.28.2.11), cfs-node-12 (172.28.2.12), cfs-node-13 (172.28.2.13) — all redirect via VIP 172.28.2.26 |
| RFS VIP (Virtual IP) | 172.28.2.27 — front-facing address for the RFS cluster |
| RFS Cluster Nodes | rfs-node-1 (172.28.2.14), rfs-node-2 (172.28.2.15), rfs-node-3 (172.28.2.16) — all redirect via VIP 172.28.2.27 |
| Deployment Type | LSA (Layered Service Architecture) - RFS/CFS, 3-Node CFS Cluster with VIP Failover / 3-Node RFS Cluster with VIP Failover |
| Field Management System | Tejas EMS |

# 2 Test Cases summary

The table below summarizes each verification step performed. Detailed procedure, expected result, and supporting evidence (screenshots) are documented against each test case.

| # | Checkpoint | What It Confirms | Type | Result |
| --- | --- | --- | --- | --- |
| 1 | Provisioning a New Service Instance | The required service parameters (device, OLT, ONT, customer ID, VLANs, bandwidth profile) can be successfully configured in NSO. | CLI | PASS |
| 2 | Committing the Service Configuration | The configured service transaction commits successfully in NSO with no errors. | CLI | PASS |
| 3 | Deleting a Service Instance | An existing service instance can be removed from NSO and the deletion commits successfully. | CLI | PASS |
| 4 | Postman Request Submission to NSO | A service provisioning request submitted directly from Postman to NSO via RESTCONF (CFS VIP) is accepted. | Postman | PASS |
| 5 | CFS VIP — Service Visibility | The provisioned service instance is visible in the CFS Services list via the CFS VIP. | Web UI | PASS |
| 6 | CFS Cluster — High Availability | cfs-node-11/12/13 report correct primary/secondary roles via the CFS VIP. | Web UI | PASS |
| 7 | RFS VIP — Service Visibility | The provisioned service instance is visible in the RFS Services list via the RFS VIP. | Web UI | PASS |
| 8 | RFS Cluster — High Availability | rfs-node-1/2/3 report correct primary/secondary roles via the RFS VIP. | Web UI | PASS |
| 9 | Tejas EMS — ONT Provisioning | The ONT associated with the service is visible and correctly activated in Tejas EMS. | Web UI | PASS |
| 10 | Tejas EMS — ONT Provisioning Detail | Detailed ONT attributes in Tejas EMS match the values submitted via Postman. | Web UI | PASS |
| 11 | CFS VIP — Service Deletion Request | A provisioned service instance can be selected and deleted via the CFS VIP. | Web UI | PASS |
| 12 | Deleted in CFS | The service instance no longer appears in the CFS Services list after deletion. | Web UI | PASS |
| 13 | Deleted in RFS | The service instance no longer appears in the RFS Services list after deletion. | Web UI | PASS |
| 14 | Unprovisioned in Tejas EMS | The ONT record is fully removed from Tejas EMS following service deletion. | Web UI | PASS |

# 3. NSO Service Configuration and Lifecycle Management

This section covers a service instance within NSO — from initial configuration through commit and eventual deletion, all performed via the command line interface.

### Test Case 1: Provisioning a New Service Instance

| Description: | Verify that a service instance can be configured in NSO by supplying the required service parameters — device name, OLT details, ONT details, customer ID, VLANs, and bandwidth profile. |
| --- | --- |
| Procedure: | 1. Enter the service configuration mode: services SERVICE_TEMPLATE SERVICE_INSTANCE_ID 2. Configure the required service parameters, including: - Device name and package - OLT IP address - ONT serial number, model, and port - C-VLAN and S-VLAN - Customer ID - Bandwidth profile - Provisioning mode and VLAN tagging - PON port details |
| Expected Result: | The configuration should accept and display the provided parameters within the NSO configuration session. |
| Result: | ☐ OK ☐ Not OK |
| Logs/Snaps: (if Any) | image |

### Test Case 2: Committing the Service Configuration

| Description: | Verify that a configured service transaction can be committed in NSO. |
| --- | --- |
| Procedure: | 1. Execute: commit |
| Expected Result: | The service configuration transaction should be committed successfully. |
| Result: | ☐ OK ☐ Not OK |
| Logs/Snaps: (if Any) | image |

### Test Case 3: Deleting a Service Instance

| Description: | Verify that an existing service instance can be removed from NSO and that the deletion is committed. |
| --- | --- |
| Procedure: | 1. Enter configuration mode: config terminal 2. Delete the target service instance: no services SERVICE_TEMPLATE SERVICE_INSTANCE_ID 3. Commit the change: commit |
| Expected Result: | The specified service instance is deleted, and the commit completes successfully. |
| Result: | ☐ OK ☐ Not OK |
| Logs/Snaps: (if Any) | image |

# 4. The Cluster Architecture — Nodes, Node Type, IP, VIP

## 4.1 Cluster Node Reference Table

| Node | Node Type | Node IP Address | Cluster VIP |
| --- | --- | --- | --- |
| cfs-node-11 | CFS | 172.28.2.11 | 172.28.2.26 (CFS VIP) |
| cfs-node-12 | CFS | 172.28.2.12 | 172.28.2.26 (CFS VIP) |
| cfs-node-13 | CFS | 172.28.2.13 | 172.28.2.26 (CFS VIP) |
| rfs-node-1 | RFS | 172.28.2.14 | 172.28.2.27 (RFS VIP) |
| rfs-node-2 | RFS | 172.28.2.15 | 172.28.2.27 (RFS VIP) |
| rfs-node-3 | RFS | 172.28.2.16 | 172.28.2.27 (RFS VIP) |

This chapter confirms correct VIP redirection for both clusters. All requests to the CFS cluster (cfs-node-11, cfs-node-12, cfs-node-13) are made against CFS VIP 172.28.2.26; all requests to the RFS cluster (rfs-node-1, rfs-node-2, rfs-node-3) are made against RFS VIP 172.28.2.27. Because all client and integration traffic is directed at the VIP rather than individual node addresses, per-node administrative Lock/Unlock state does not apply as a test dimension at this layer — the VIP always transparently routes to the currently active node in each cluster.

# 5 User Interface and System Verification

These test cases run against the web-facing systems — the CFS dashboard (via VIP 172.28.2.26) and RFS dashboard (via VIP 172.28.2.27) — along with the Tejas EMS field management application, confirming that CLI-driven and RESTCONF-driven automated changes are correctly reflected in the systems used daily by business and field teams.

| Component | Role | IP Address / Identifier |
| --- | --- | --- |
| CFS (Customer Facing Service) | Central orchestration cluster used for order submission & verification; all client requests target the CFS VIP, which routes to the active node among cfs-node-11/12/13 | 172.28.2.26 (VIP) |
| RFS (Resource Facing Service) Web Portal | Field-facing service cluster used for order verification; all client requests target the RFS VIP, which routes to the active node among rfs-node-1/2/3 | 172.28.2.27 (VIP) |
| Tejas EMS | Third-party Element Management System for OLT field device verification | Integrated via NBI |

## 5.1 Web Portal and EMS Verification

### Test Case 4: Postman Request Submission to NSO

| Description: | Verify that a service provisioning request submitted directly from Postman to NSO via the RESTCONF API (targeting CFS VIP 172.28.2.26) is accepted. |
| --- | --- |
| Procedure: | 1. Open Postman and construct a PATCH request to the CFS VIP RESTCONF endpoint: http://172.28.2.26:8080/restconf/data/tailf-ncs:services/tejas-nms-nso-cfs:tejas-nms-nso-cfs 2. Populate the request body with the required service parameters — device, service-id, package, OLT IP, PON port, ONT serial/model/port, C-VLAN/S-VLAN, customer ID, bandwidth profiles, provisioning mode, and VLAN tagging. 3. Send the request. |
| Expected Result: | NSO returns a success response (204 No Content), confirming the provisioning request was accepted. |
| Result: | ☐ OK ☐ Not OK |
| Logs/Snaps: (if Any) | image |

### Test Case 5: CFS VIP — Service Visibility

| Description: | Verify that the newly provisioned service instance is visible in the CFS Services list via the CFS VIP. |
| --- | --- |
| Procedure: | 1. Log in to the CFS Web UI via VIP (172.28.2.26). 2. Navigate to Services. 3. Search for the Service ID. |
| Expected Result: | The service instance (e.g., Tik10012-GPON-300626022237) appears in the CFS Services list. |
| Result: | ☐ OK ☐ Not OK |
| Logs/Snaps: (if Any) | image |

### Test Case 6: CFS Cluster — High Availability Verification

| Description: | Verify that the CFS cluster's High Availability page (via CFS VIP) correctly reports node roles. |
| --- | --- |
| Procedure: | 1. Navigate to Tools > High Availability on the CFS Web UI (via VIP 172.28.2.26). 2. Review the running and configured role of each node. |
| Expected Result: | cfs-node-11 is shown as primary (current); cfs-node-12 and cfs-node-13 are shown as secondary. |
| Result: | ☐ OK ☐ Not OK |
| Logs/Snaps: (if Any) | image |

### Test Case 7: RFS VIP — Service Visibility

| Description: | Verify that the same provisioned service instance is visible in the RFS Services list via the RFS VIP. |
| --- | --- |
| Procedure: | 1. Log in to the RFS Web UI via VIP (172.28.2.27). 2. Navigate to Services. 3. Search for the Service ID. |
| Expected Result: | The service instance appears in the RFS Services list. |
| Result: | ☐ OK ☐ Not OK |
| Logs/Snaps: (if Any) | image |

### Test Case 8: RFS Cluster — High Availability Verification

| Description: | Verify that the RFS cluster's High Availability page (via RFS VIP) correctly reports node roles. |
| --- | --- |
| Procedure: | 1. Navigate to Tools > High Availability on the RFS Web UI (via VIP 172.28.2.27). 2. Review the running and configured role of each node. |
| Expected Result: | rfs-node-1 is shown as primary (current); rfs-node-2 and rfs-node-3 are shown as secondary. |
| Result: | ☐ OK ☐ Not OK |
| Logs/Snaps: (if Any) | image |

### Test Case 9: Tejas EMS — ONT Provisioning Verification

| Description: | Verify that the ONT associated with the provisioned service is visible and correctly configured in Tejas EMS. |
| --- | --- |
| Procedure: | 1. Log in to Tejas EMS. 2. Navigate to Configuration > Manage ONTs. 3. Filter or search by the ONT Serial Number. |
| Expected Result: | The ONT record appears with EMS State = ACTIVATE and Admin State = Enable. |
| Result: | ☐ OK ☐ Not OK |
| Logs/Snaps: (if Any) | image |

### Test Case 10: Tejas EMS — ONT Provisioning Detail Verification

| Description: | Verify that the detailed provisioning attributes of the ONT in Tejas EMS match the values submitted in the service request. |
| --- | --- |
| Procedure: | 1. From the Manage ONTs view in Tejas EMS, scroll to view the extended columns. 2. Confirm the Customer ID, Upstream Mapping, and related fields. |
| Expected Result: | Customer ID and other fields match the values submitted via Postman (e.g., Customer Id = GOV/ITC/001). |
| Result: | ☐ OK ☐ Not OK |
| Logs/Snaps: (if Any) | inage |

### Test Case 11: CFS VIP — Service Deletion Request

| Description: | Verify that a provisioned service instance can be selected and deleted via the CFS VIP. |
| --- | --- |
| Procedure: | 1. In the CFS Web UI (via VIP 172.28.2.26), select the target service instance. 2. Click Delete, then confirm "Commit changes directly (skip commit manager)". |
| Expected Result: | The Delete Service Instance confirmation dialog correctly displays the selected service instance path. |
| Result: | ☐ OK ☐ Not OK |
| Logs/Snaps: (if Any) | image |

### Test Case 12: Deleted in CFS

| Description: | Verify that the service instance no longer appears in the CFS Services list after deletion. |
| --- | --- |
| Procedure: | 1. After confirming deletion, refresh the CFS Services list (via VIP 172.28.2.26). 2. Search for the deleted Service ID. |
| Expected Result: | The service instance is no longer present in the CFS Services list (result count decreases by one). |
| Result: | ☐ OK ☐ Not OK |
| Logs/Snaps: (if Any) | image |

### Test Case 13: Deleted in RFS

| Description: | Verify that the service instance no longer appears in the RFS Services list after deletion. |
| --- | --- |
| Procedure: | 1. Refresh the RFS Services list (via VIP 172.28.2.27). 2. Search for the deleted Service ID. |
| Expected Result: | The service instance is no longer present in the RFS Services list. |
| Result: | ☐ OK ☐ Not OK |
| Logs/Snaps: (if Any) | image |

### Test Case 14: Unprovisioned in Tejas EMS

| Description: | Verify that the ONT record is removed from Tejas EMS following service deletion. |
| --- | --- |
| Procedure: | 1. In Tejas EMS, navigate to Manage ONTs. 2. Search by the ONT Serial Number used earlier. |
| Expected Result: | No matching ONT record is found (0 of 0 results), confirming the service is fully unprovisioned. |
| Result: | ☐ OK ☐ Not OK |
| Logs/Snaps: (if Any) | image |