Short----------
1.  Service provisioning
3.  NSO VLAN Assignment Models
4.  IP Pool Assignment Models
5.  Traffic Priority for QoS in GPON
2.  ONT LAN Ports — Enable (Unshut) During Service Configuration
7. Re-Try and provision mechanism parameter to check , number of retries.
8. Additionally Second configuration of PON port not working to be resolved while already existing customer is in use, We cant go live without this option. This was working when Cisco has done the Demo now its not working.
6.  GI ILL — TDIA Service Configuration

1.  TCell Point-to-Point (P2P) L2VPN Service Configuration

For point-to-point cellular connectivity, the following scenarios are defined:

Scenario 1 — DCWAN/ZCR (Headend or Site A) and MAG/GP (Spoke or Site B)
Scenario 2 — MAG/GP (Headend or Site A) and MAG/GP (Spoke or Site B)

Under Scenario 2, three possible sub-cases exist:

- MAG as Site A and GP as Site B

- MAG as Site A and another MAG as Site B

- GP as Site A and another GP as Site B

NOTE: Considering BGP-LU, end-to-end L2VPN can be created between other ring MAG & GP nodes.

Both scenarios are supported with two transport options:

- Service Configuration without BGP-LU Transport

- Service Configuration with BGP-LU Transport

Configuration Details

Scenario 1 — DCWAN/ZCR (Site A) and MAG/GP (Site B)
DCWAN/ZCR Configuration — Ingress Interface

interface Bundle-Ether<As per bundle number>.<As per VLAN-ID> l2transport

service-policy input <As per policy-map_IN>

service-policy output <As per policy-map_OUT>

description <As per naming format>

encapsulation dot1q <As per customer VLAN>

!

DCWAN/ZCR Configuration — L2VPN

L2VPN

!

bridge group <As per bridge group name>

bridge-domain <As per Bridge domain ID>

mtu 9100

interface Bundle-Ether <As per bundle number>.<As per VLAN-ID>

!

vfi <As per VFI name>

neighbour <As per Neighbor IP> <As per pseudowire ID> ! Add Mandal/GP Router MAG/GP Loopback 0

pw-class <As per name of the pseudowire class>

!

MAG/GP Configuration — Egress Interface (UNI)

interface Gix/x/x ! UNI Interface

service-policy input <As per policy-map_IN>

service-policy output <As per policy-map_OUT>

service instance <As per service instance ID> ethernet

encapsulation dot1q <As per customer VLAN IDs>

rewrite ingress tag push dot1q <As per Bridge domain ID> symmetric

bridge-domain <As per Bridge domain ID>

!

MAG/GP Configuration — L2VPN

l2 vfi <As per VFI name> manual vpn id <As per VPN-ID> mtu 9100 bridge-domain <As per Bridge domain ID> neighbor <As per Neighbor IP> encapsulation mpls ! Add Headend Router DC-WAN/ZCR Loopback 0 !

NOTE: The UNI interface may differ — it can be on a port-channel, in which case the port-channel UNI deployment configurations apply with Active/Standby LACP.

Scenario 2 — MAG/GP (Site A) and MAG/GP (Site B)
MAG Configuration (Site A) — Ingress Interface

interface Gix/x/x ! UNI Interface

service-policy input <As per policy-map_IN>

service-policy output <As per policy-map_OUT>

service instance <As per service instance ID> ethernet

encapsulation dot1q <As per VLAN-ID list>

rewrite ingress tag push dot1q <As per Bridge domain ID> symmetric

bridge-domain <As per Bridge domain ID>

!

MAG Configuration — L2VPN

l2 vfi <As per VFI name> manual

vpn id <As per VPN-ID>

mtu 9100

bridge-domain <As per Bridge domain ID>

neighbor <As per Neighbor IP> encapsulation mpls ! Add Headend Router DC-WAN/ZCR Loopback 0

!

GP Configuration (Site B) — Ingress Interface

interface Gix/x/x ! UNI Interface

service-policy input <As per policy-map_IN>

service-policy output <As per policy-map_OUT>

service instance <As per service instance ID> ethernet

encapsulation dot1q <As per VLAN-ID list>

rewrite ingress tag push dot1q <As per Bridge domain ID> symmetric

bridge-domain <As per Bridge domain ID>

!

GP Configuration — L2VPN

l2 vfi <As per VFI name> manual

vpn id <As per VPN-ID>

mtu 9100

bridge-domain <As per Bridge domain ID>

neighbor <As per Neighbor IP> encapsulation mpls ! Add Headend Router DC-WAN/ZCR Loopback 0

!

NOTE: The UNI interface may differ — it can be on a port-channel. Refer to port-channel UNI deployments with SDM(Switch Database Management) commands and Active/Standby LACP configurations.

Additional P2P Scenarios — Routers Not Yet Integrated with MPLS (Managed via TSWAN)

Where Mandal/GP routers are not yet integrated into the MPLS network, the following additional P2P scenarios apply. Router management is via TSWAN network until MPLS integration is complete.

Additional Scenario 1 — Both Sites on Same Mandal Router

- Cell Tower Site-1 connected to Mandal router on fiber last mile (single or dual), Port-Channel A configured with 1 or 2 links.

- Cell Tower Site-2 also connected to the same Mandal router on fiber last mile, Port-Channel B configured with 1 or 2 links.

- Port-Channel A and Port-Channel B are configured under the same bridge domain to establish direct P2P connectivity between Site-1 and Site-2.

- CIR and QoS configured as per requirement and subscription.

Mandal Router Configuration

interface Gi<x/x/x>

description <As per site description>

no ip address

negotiation auto

service-policy input <As per policy-map_IN>

service-policy output <As per policy-map_OUT>

service instance <As per service instance ID> ethernet

description <As per site description>

encapsulation dot1q <As per VLAN-ID list>

bridge-domain <As per Bridge domain ID>

interface Gi<x/x/x>

description <As per site description>

no ip address

negotiation auto

service-policy input <As per policy-map_IN>

service-policy output <As per policy-map_OUT>

service instance <As per service instance ID> ethernet

description <As per site description>

encapsulation dot1q <As per VLAN-ID list>

bridge-domain <As per Bridge domain ID>

Additional Scenario 2 — Site-1 on Mandal Router, Site-2 on GP Router (Not in MPLS)

- Mandal and GP routers are not integrated with MPLS network, but connectivity between them is ready.

- Cell Tower Site-1 connected to Mandal router on fiber last mile, Port-Channel A with 1 or 2 links.

- Cell Tower Site-2 connected to GP router on fiber last mile, Port-Channel A with 1 or 2 links.

- QinQ configured on router ports facing Cell Tower.

- L2VPN configured between Mandal and GP routers to establish P2P connectivity.

- CIR and QoS configured on the LM port-channel per requirement.

Mandal Router Configuration

interface Gi<x/x/x>

description <As per site description>

no ip address

negotiation auto

service-policy input <As per policy-map_IN>

service-policy output <As per policy-map_OUT>

service instance <As per service instance ID> ethernet

description <As per site description>

encapsulation dot1q <As per VLAN-ID list>

rewrite ingress tag push dot1q <As per Bridge domain ID> symmetric

bridge-domain <As per Bridge domain ID>

! L2VPN:

l2 vfi <As per VFI name> manual

vpn id <As per VPN-ID>

mtu 9100

bridge-domain <As per Bridge domain ID>

neighbor <As per Neighbor IP> encapsulation mpls ! L2VPN to GP router

GP Router Configuration

interface Gi<x/x/x>

description <As per site description>

no ip address

negotiation auto

service-policy input <As per policy-map_IN>

service-policy output <As per policy-map_OUT>

service instance <As per service instance ID> ethernet

description <As per site description>

encapsulation dot1q <As per VLAN-ID list>

rewrite ingress tag push dot1q <As per Bridge domain ID> symmetric

bridge-domain <As per Bridge domain ID>

! L2VPN:

l2 vfi <As per VFI name> manual

vpn id <As per VPN-ID>

mtu 9100

bridge-domain <As per Bridge domain ID>

neighbor <As per Neighbor IP> encapsulation mpls ! L2VPN to Mandal router

Troubleshooting Commands — P2P (Spoke/MAG/GP Routers)

| Command | Purpose |
| --- | --- |
| show run ¦ s l2 vfi | Check and verify the L2 VFI configuration on MAG & GP router |
| show mpls l2transport vc vcid <Number> | Check and verify the L2VPN status on MAG & GP |
| show l2vpn vfi name <name> detail | Check and verify the L2VPN status on MAG & GP |
| show bridge-domain <Number> | Check and validate the MAC addresses |
| show policy-map interface <interface-id> {input/output} | Validate the QoS policy status on MAG & GP |

QoS Policy — TCell L2 VPN Point to Point (P2P)

Traffic Class Definition

| Traffic Class | CoS Match | CIR % | Egress Priority | Purpose |
| --- | --- | --- | --- | --- |
| Signalling (SIGN) | CoS 7 | 5% | Priority Level 1 — Strict | Control / Signalling |
| Circuit Switched (CS) | CoS 6 | 10% | Priority Level 2 — Strict | Voice / CS Bearer |
| OAM | CoS 5 | 5% | Bandwidth (best-effort) | Operations & Mgmt |
| Data | CoS 4 | 80% | Bandwidth (best-effort) | User Data / Internet |

NOTE: Signalling and CS use Strict Priority — always served first regardless of queue depth. OAM and Data use bandwidth queuing with queue-limit. Aggregate CIR on MAG/GP is 200 Mbps (parent police).

SDC Router — QoS Policy

Classifies CoS bits into internal qos-group values. No CIR policing at SDC — classification and remarking only.

| Ingress — Child Policy Map | Egress — Policy Map |
| --- | --- |
| class-map match-all AIRTEL_CELL_SIGN_IN match cos 7 ! class-map match-all AIRTEL_CELL_CS_IN match cos 6 ! class-map match-all AIRTEL_CELL_OAM_IN match cos 5 ! class-map match-all AIRTEL_CELL_DATA_IN match cos 4 ! policy-map AIRTELCELL_CHILD-IN class AIRTEL_CELL_SIGN_IN set qos-group 7 ! class AIRTEL_CELL_CS_IN set qos-group 6 ! class AIRTEL_CELL_OAM_IN set qos-group 2 ! class AIRTEL_CELL_DATA_IN set qos-group 1 ! Policy-Map AIRTELCELL_PARENT-IN class class-default service-policy AIRTELCELL_CHILD-IN ! | policy-map AIRTELCELL-OUT class AIRTEL_CELL_SIGN_OUT ! qos-group 7 set cos 5 ! class AIRTEL_CELL_CS_OUT ! qos-group 6 set cos 6 ! class AIRTEL_CELL_OAM_OUT ! qos-group 2 set cos 5 ! class AIRTEL_CELL_DATA_OUT ! qos-group 1 set cos 4 ! |

DCWAN / ZCR Router (HUB) — QoS Policy

Classifies CoS, marks MPLS EXP bits for core transport, and applies strict priority queuing at egress toward SDC.

| Ingress — Child Policy Map (with MPLS EXP) | Egress — Priority Queuing |
| --- | --- |
| policy-map AIRTELCELL_CHILD-IN class AIRTEL_CELL_SIGN_IN set qos-group 5 set mpls experimental imposition 5 ! class AIRTEL_CELL_CS_IN set qos-group 6 set mpls experimental imposition 6 ! class AIRTEL_CELL_OAM_IN set qos-group 2 set mpls experimental imposition 2 ! class AIRTEL_CELL_DATA_IN set qos-group 1 set mpls experimental imposition 1 ! class class-default set qos-group 0 ! Policy-Map AIRTELCELL_PARENT-IN class class-default service-policy AIRTELCELL_CHILD-IN ! | policy-map AIRTELCELL-OUT class AIRTEL_CELL_SIGN_OUT set cos 7 priority level 1 ! class AIRTEL_CELL_CS_OUT set cos 6 priority level 2 ! class AIRTEL_CELL_OAM_OUT set cos 5 ! class AIRTEL_CELL_DATA_OUT set cos 4 ! ! Apply to interface: interface Bundle-Ether15.2700 l2transport service-policy input AIRTELCELL_PARENT-IN service-policy output AIRTELCELL-OUT ! |

MAG — Mandal Router (Spoke, ASR907) — QoS Policy

CIR policing per class at ingress. Parent policy enforces 200 Mbps aggregate. Egress applies hierarchical shaping (200 Mbps) with priority queuing.

| Ingress — CIR Policing + MPLS EXP | Egress — Priority + BW + 200 Mbps Shaper |
| --- | --- |
| policy-map AIRTELCELL_CHILD-IN class AIRTEL_CELL_SIGN_IN police cir percent 5 set qos-group 5 set mpls experimental imposition 5 ! class AIRTEL_CELL_CS_IN police cir percent 10 set qos-group 6 set mpls experimental imposition 6 ! class AIRTEL_CELL_OAM_IN police cir percent 5 set qos-group 2 set mpls experimental imposition 2 ! class AIRTEL_CELL_DATA_IN police cir percent 80 set qos-group 1 set mpls experimental imposition 1 ! class class-default set qos-group 0 set mpls experimental imposition 0 ! Policy-Map AIRTELCELL_PARENT-IN class class-default police cir 200000000 bc 4161500 conform-action transmit exceed-action drop service-policy AIRTELCELL_CHILD-IN ! | policy-map AIRTELCELL-OUT class AIRTEL_CELL_SIGN_OUT priority level 1 percent 5 set cos 7 ! class AIRTEL_CELL_CS_OUT priority level 2 percent 10 set cos 6 ! class AIRTEL_CELL_OAM_OUT bandwidth percent 5 set cos 4 ! class AIRTEL_CELL_DATA_OUT bandwidth percent 80 set cos 3 queue-limit 4000000 bytes ! class class-default set cos 0 ! Policy-Map AIRTELCELL_PARENT-OUT class class-default shape average 200000000 4000000 service-policy AIRTELCELL-OUT ! |

ASR907 — Egress QoS on Port-Channel (SDM Template — One-Time)

! Step 1 — Enable SDM template (box will reload after this) sdm prefer enable_portchannel_qos_multiple_active reload location all ! Step 2 — Post-reload: enable platform CLI BEFORE creating port-channel platform qos-port-channel_multiple_active <port-channel#> ! Step 3 — Apply policy on port-channel interface Port-channel <Number> service-policy input AIRTELCELL_PARENT-IN service-policy output AIRTELCELL_PARENT-OUT

GP — Gram Panchayat Router (Spoke, ASR920) — QoS Policy

Ingress is identical to MAG. Egress differs — ASR920 does NOT support egress shaping on port-channel. Apply egress on member links; ingress on port-channel service instance.

| Egress — Priority + BW + 200 Mbps Shaper | ASR920 — Apply Points (Port-Channel Limitation) |
| --- | --- |
| policy-map AIRTELCELL-OUT class AIRTEL_CELL_SIGN_OUT ! qos-group 5 priority level 1 percent 5 set cos 7 ! class AIRTEL_CELL_CS_OUT ! qos-group 6 priority level 2 percent 10 set cos 6 ! class AIRTEL_CELL_OAM_OUT ! qos-group 2 bandwidth percent 5 set cos 4 ! class AIRTEL_CELL_DATA_OUT ! qos-group 1 bandwidth percent 80 queue-limit 2000000 bytes set cos 3 ! class class-default set cos 0 ! Policy-Map AIRTELCELL_PARENT-OUT class class-default shape average 200000000 2000000 service-policy AIRTELCELL-OUT ! | ! Egress — on MEMBER LINKS only (NOT port-channel) interface <member-link-1> service-policy input AIRTELCELL_PARENT-IN service-policy output AIRTELCELL_PARENT-OUT ! interface <member-link-2> service-policy input AIRTELCELL_PARENT-IN service-policy output AIRTELCELL_PARENT-OUT ! ! Ingress — on port-channel SERVICE INSTANCE interface <port-channel> service instance <XX> ethernet service-policy input AIRTELCELL_PARENT-IN service-policy output AIRTELCELL_PARENT-OUT bridge-domain 2700 ! ! LACP Active/Standby interface <port-channel> lacp fast-switchover lacp max-bundle 1 ! |

QoS Summary — All Routers

| Router | Ingress Policing | MPLS EXP Mark | Egress Queuing | Aggregate Shape |
| --- | --- | --- | --- | --- |
| SDC | No — CoS classify only | No | CoS remarking only | None |
| DCWAN/ZCR (HUB) | No — CoS classify only | EXP 1/2/5/6 | Priority L1/L2 + BW | None |
| MAG — ASR907 | Yes — CIR% per class | EXP 1/2/5/6 | Priority L1/L2 + BW (burst 4M) | 200 Mbps |
| GP — ASR920 | Yes — CIR% per class | EXP 1/2/5/6 | Priority L1/L2 + BW (burst 2M) | 200 Mbps |

ASR907 vs ASR920 — Key QoS Difference

| Feature | ASR907 (MAG) | ASR920 (GP) |
| --- | --- | --- |
| Egress QoS on Port-Channel | Supported — SDM template + platform CLI | NOT supported — apply on member links |
| SDM Template / Box Reload | Required — box reloads | Not required |
| Ingress QoS Apply Point | Port-channel interface | Port-channel service instance |
| Egress Burst Size | shape average 200000000 4000000 | shape average 200000000 2000000 |

IMPORTANT: For ASR907: The SDM template (sdm prefer enable_portchannel_qos_multiple_active) is a one-time operation that requires a box reload. The platform qos-port-channel_multiple_active command must be applied after reload and BEFORE creating the port-channel.

SERVICE PROVISIONING

End-to-End Service Provisioning — BSS | OSS | SCCM | NSO | Network

This document provides a complete overview of the automated service provisioning system, describes the end-to-end provisioning flow, explains the role of each system involved, and highlights the exact API calls that enable seamless integration.
Service connects government institutions — schools, Gram Panchayats, health centres — to high-speed broadband internet using T-FIBER's optical fiber network. The entire provisioning process, from accepting a customer order to activating the service on the network, is automated through a chain of four interconnected software systems.

Four Systems in the Chain

| System | Full Name | Plain-English Role |
| --- | --- | --- |
| BSS | Business Support System | The customer-facing portal. A government institution places a broadband order here. BSS validates the customer, creates the order, and triggers the provisioning chain. It also receives a final notification when the service is live. |
| OSS | Operations Support System | The network inventory system. OSS keeps track of every device, service, and circuit in the T-FIBER network. It queries SCCM to maintain an up-to-date picture of all active services — used for fault management, audits, and capacity planning. |
| SCCM | Service & Config Control Manager | The orchestration middleware. SCCM sits between BSS and the network. It receives orders from BSS, figures out which devices need configuration, builds the service, sends it to NSO, and notifies BSS when done. It also exposes inventory APIs to OSS. |
| NSO | Network Services Orchestrator (Cisco) | The network configuration engine. NSO receives service templates from SCCM and pushes the actual router and OLT configurations. It is the only system that directly touches network devices. If a push fails on any device, NSO rolls back all changes automatically. |

NOTE: T-FIBER's network is built on IP/MPLS core (Cisco ASR routers) with GPON access (Nokia and Tejas OLTs). GI-BB is delivered as an L2VPN Hub & Spoke service over this infrastructure.

BSS Flowchart diagram

1.2.  Device Inventory

1.2.1  Physical Network Devices

- The following hardware devices are part of the T-FIBER Service provisioning environment

| Device Type | Hostname | Management IP | Platform | Role in GI-BB |
| --- | --- | --- | --- | --- |
| GP Router | NOCTESTGAC001 | 172.17.128.1 | Cisco ASR 920 | Spoke — GP Level PE |
| Mandal Router | NOCTESTMAR001 | 172.17.224.1 | Cisco ASR 907 | Spoke — Mandal PE |
| Nokia OLT | NOCTESTOLTN001 | 172.17.128.30 | Nokia 7360 (PizzaBox) | GPON OLT — GP Zone |
| Nokia OLT | NOCTESTOLTN003 | 172.17.224.27 | Nokia 7362 (Chassis) | GPON OLT — Mandal Zone |
| HUB Router | NOC507RTRN00001 | 172.17.236.1 | Cisco ASR 9000 (ZCR) | L2VPN Hub Head-end |
| Tejas ONT | Serial: TJNW95D01B1D | — | TJ2100N Series | Customer Premises |
| Nokia ONT | Serial: ALCLFBDC7DCD | — | G-240G-E / G-2425G-A | Customer Premises |
| SCCM Server | -- | 172.28.2.101 | VM (PHP/Apache) | Orchestration Engine |
| BSS Server | -- | 172.28.10.9 | Application Server | Customer Order Portal |
| Nokia EMS | Nokia AMS (APC) | 172.28.39.12 | Nokia 5520 AMS v9.7.08 | Nokia OLT EMS |
| Tejas EMS | Tejas NMS | 172.28.7.85 | Tejas EMS v8_1_0 | Tejas OLT EMS |

PHYSICAL INVENTORY VISIBILITY IS THROUGH Below API CALL

https://<SCCM_SERVER>/sccm/sccm_api/web/api/v1/commited_service_orders Response: { "status_code": "<HTTP Status Code>", "sub_status_code": "<Application Specific Status Code>", "system": "<Source System Identifier>", "message": "<Execution Status Message>", "data": [ { "id": "<Service Order ID>", "service_id": "<Service Identifier>", "circuit_id": "<Circuit Identifier>", "devices_list": { "ONT": "<Optical Network Terminal>", "OLT": "<Optical Line Terminal>", "EMS": "<Element Management System>", "MAG": "<Aggregation Device>", "GAC": "<Gateway Access Controller>", "ont-serial-number": "<ONT Serial Number>", "connected-router": "<Connected Router Name>", "connected-router-ip": "<Connected Router IP Address>" }, "service_start_point": "<Service Start Point>", "service_end_point": "<Service End Point>", "gateway-ip": "<Gateway IP Address>", "customer-ip": "<Customer Assigned IP Address>", "interfaceID": "<Router Interface Identifier>", "service_type": "<Service Category>" } ], "app_data": [] }

1.2.2  Logical Inventory

Logical resources assigned to GI-BB services:

| Logical Element | Value / Range | Description |
| --- | --- | --- |
| S-Tag (SVLAN) | 3291 – 3300 | GI-BB outer VLAN on L3VPN Hub & Spoke (ISP VLAN range) |
| C-Tag (CVLAN) | 2537 – 2699 | Customer VLAN assigned per ONT port (from VLAN Master Plan) |
| OLT Network VLAN | 3301 | L2 forwarding VLAN inside Nokia OLT (l2fwder-vlan). Same across all GI-BB services. |
| Bridge Domain | 3301 | L2 bridge domain on GP/Mandal router where pseudowires and interfaces meet |
| VPN ID | 3301 | Shared VPN identifier across HUB and all Spoke routers in the same L2VPN |
| MPLS EXP (QoS) | 2 | MPLS Experimental bits marking for GI-BB data traffic in the MPLS core |
| GI-BB IP Pool A | 10.10.0.0/16 → /28 subnets | Subscriber IP pool — Zone A. Each customer gets one /28 (14 usable IPs). Pool A — unassigned to router. |
| GI-BB IP Pool B | 10.11.0.0/16 → /28 subnets | Subscriber IP pool — Zone B. Mapped to GP routers (172.16.x.x management IPs). PKG: B/C assigned. |
| GI-BB IP Pool C | 10.12.0.0/16 → /28 subnets | Subscriber IP pool — Zone C. Mapped to GP routers (172.16.x.x). PKG: A assigned. |
| Subnet Mask | 255.255.255.240 (/28) | Each GI-BB customer gets a /28 block. First IP = Network, Second = Gateway, Last = Broadcast. |
| Internal IP Range | 172.24.0.0/16 – 172.25.0.0/16 | Internal T-FIBER service addressing (management / L3VPN internal use) |
| External IP Range | 172.22.0.0/16 – 172.23.0.0/16 | External service addressing |
| Route Target (Internal) | RT 1–4999 | RT 1–100: Internal services. RT 101–4999: Government/internal customers (G-to-G, G-to-B) |
| Route Target (External) | RT 5000–9999 | External VPN customers (C-to-C, G-to-C) |
| SCCM Base URL | https://172.28.2.101/sccm/sccm_api/web/api/v1/ | All SCCM REST API calls use this base URL |
| BSS Callback URL | https://172.28.10.9:30098/rest/thirdparty/notification/servicecompletion | SCCM calls this URL to notify BSS of service completion |
| Nokia AMS SOAP URL | http://172.28.39.12:8080/soap/services/ApcRemotePort/9.7 | NSO uses this to push OLT/ONT configurations via SOAP |
| Tejas EMS REST URL | https://172.28.7.85:446/nmsnbi-rest/tapi/data/ | NSO uses this for Tejas OLT provisioning via REST |

LOGICAL INVENTORY VISIBILITY IS THROUGH Below API CALL

https://<SCCM_API_HOST>/sccm/sccm_api/web/api/v1/service_provisional_details?service_order_id=L3VPN-SER-0002 { "status_code": "<HTTP Status Code>", "sub_status_code": "<Application Status Code>", "system": "<Source System>", "message": "<Execution Status>", "data": [ { "id": "<Transaction ID>", "service_order_id": "<Service Order ID>", "customer_id": "<Customer ID>", "service_model_id": "<Service Model ID>", "endpoints": "<Endpoint Count>", "port_number": "<Port Information>", "qos": "<QoS Profile>", "bandwidth": "<Bandwidth Profile>", "profile": "<Service Profile>", "user_defined_data": { "vlan-id": "<Customer VLAN ID>", "bfd-required": "<Yes/No>", "bandwidth-profile": "<Bandwidth Profile>", "service-profile": "<Service Profile>", "routing-protocol": "<Routing Protocol>", "topology": "<Service Topology>" }, "system_defined_data": { "transaction_id": "<Transaction ID>", "service_instance_id": "<Service Instance ID>", "customer_identifier": "<Customer Identifier>", "service_model_reference": "<Service Model Reference>", "creation_timestamp": "<Creation Timestamp>", "generated_by": "<System Component>" }, "device_list": [ "<Target Device 1>", "<Target Device 2>" ], "final_nso_payload": { "service-id": "<Service ID>", "service-type": "<Service Type>", "topology": "<Topology>", "routing-protocol": "<Routing Protocol>", "vrf-definitions": [ { "vrf-name": "<VRF Name>", "rd": "<Route Distinguisher>", "rt-import": "<Route Target Import>", "rt-export": "<Route Target Export>" } ], "interfaces": [ { "interface-id": "<Interface ID>", "vlan-id": "<Customer VLAN>", "ip-type": "<IPv4/IPv6/Both>", "device-mode": "<Primary/Secondary>" } ], "prefix-sets": [ { "prefix-name": "<Prefix Set Name>", "ip-type": "<IPv4/IPv6>", "prefix": "<Prefix Value>" } ], "route-maps": [ { "route-map-name": "<Route Map Name>", "sequence": "<Sequence Number>" } ] }, "scheduled_status": "<Schedule Status>", "service_instance_status": "<Service Status>", "type_of_change": "<Provisioning Action>", "topology": "<Topology Type>", "order_type": "<Order Type>", "dryrun_output": { "dryrun-result": { "native": { "device": [ { "name": "<Device Name>", "data": "<Generated Configuration Preview>" } ] } } }, "commit_output": { "transaction_id": "<NSO Transaction ID>", "status": "<SUCCESS/FAILED>", "commit_time": "<Commit Timestamp>", "affected_devices": [ "<Device Name>" ], "message": "<Commit Status Message>" }, "variables": { "<Device ID>": { "<Service Component>": { "<Attribute ID>": { "value": "<Attribute Value>", "module_id": "<Module Identifier>", "device_mode": "<Primary/Secondary>", "device_type": "<Device Type>" } } } }, "service_specific_id": "<Service Specific ID>" } ], "app_data": [] }

1.2.3  Sub-Service ID Reference (Service type Variants (Gi-bb))

Each GI-BB service variant has a unique sub_service_id used in BSS → SCCM API calls:

| Sub Service ID | Router | Mode | VLAN Type | Description |
| --- | --- | --- | --- | --- |
| 9.5.2.1 | ASR 920 | Bridge | Untagged / Tagged | Nokia OLT → ASR920. Bridge mode. CPE sends untagged or tagged frames. |
| 9.5.2.2 | ASR 907 | Bridge | Untagged / Tagged | Nokia OLT → ASR907. Bridge mode. OLT adds PVID or single-tag. |
| 9.5.2.4 | ASR 920 | Route | Tagged | Nokia OLT → ASR920. Route mode. ONT acts as IP router. DHCP enabled on WAN. |
| 9.5.2.5 | ASR 907 | Route | Tagged | Nokia OLT → ASR907. Route mode. ONT acts as IP router. DHCP enabled on WAN. |

NOTE: Each entry has a Sub Service Id (like 9.5.2.6) And Its Naming convention is as given below (like GI-BB-NOKIA-SINGLE-ASR9k-ROUTE).

- First digit (9) → Refers to the main service family (here, GI-BB = “Gigabit Backbone” service).

- Second digit (5) → Indicates the vendor/device type (e.g., Nokia).

- Third digit (2) → Defines the deployment model (e.g., “Single” device setup vs. dual/cluster).

- Fourth digit (6) → Specifies the device model and function (e.g., ASR9k with routing).

1.2.4  GI-BB Test Scenarios

The following 12 test scenarios were executed from BSS-WFM-SCCM(NSO).

| Sr | OLT ¦ ONT | Router | Mode | VLAN Tag | Sub Svc ID | Service Order |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Nokia ¦ Tejas | ASR 907 | Bridge | Untagged | 9.5.2.2 | S101000587 |
| 2 | Nokia ¦ Nokia | ASR 907 | Bridge | Untagged | 9.5.2.2 | S101000582 |
| 3 | Nokia ¦ Tejas | ASR 907 | Route | Tagged | 9.5.2.5 | S101000592 |
| 4 | Nokia ¦ Nokia | ASR 907 | Route | Tagged | 9.5.2.5 | S101000594 |
| 5 | Nokia ¦ Tejas | ASR 920 | Bridge | Untagged | 9.5.2.1 | S101000547 |
| 6 | Nokia ¦ Nokia | ASR 920 | Bridge | Untagged | 9.5.2.1 | S101000607 |
| 7 | Nokia ¦ Tejas | ASR 920 | Route | Tagged | 9.5.2.4 | S101000604 |
| 8 | Nokia ¦ Nokia | ASR 920 | Route | Tagged | 9.5.2.4 | S101000601 |
| 9 | Nokia ¦ Nokia | ASR 920 | Bridge | Tagged | 9.5.2.1 | S101000543 |
| 10 | Nokia ¦ Tejas | ASR 920 | Bridge | Tagged | 9.5.2.1 | S101000635 |
| 11 | Nokia ¦ Tejas | ASR 907 | Bridge | Tagged | 9.5.2.2 | S101000638 |
| 12 | Nokia ¦ Nokia | ASR 907 | Bridge | Tagged | 9.5.2.2 | S101000639 |

1.3.  End-to-End Service Provisioning Flow

The diagram below shows the complete lifecycle of a service order —

| STEP 1–2 BSS Customer Order | STEP 3–5 SCCM Orchestration | STEP 6–7 NSO + NETWORK Config Push | STEP 8–9 CALLBACK + OSS Completion |
| --- | --- | --- | --- |

PHASE 1 BSS — Customer Order & Authentication

What BSS Does —

BSS is the starting point. A government institution (school, Gram Panchayat, clinic, etc.) places a broadband order through the BSS portal or CRM system. BSS is responsible for making sure the customer is valid, the service is available, and then handing the order to SCCM with all the technical parameters SCCM needs to build the connection.

BSS does NOT configure any network device. Its job is to collect the order, validate it, pass it correctly to SCCM, and then wait for SCCM to tell it the service is active.

| 1 | BSS | Customer places service order A government institution submits a GI-BB broadband request specifying: location, bandwidth plan, CPE type (Nokia/Tejas ONT), and service type (Bridge/Route mode). BSS generates a unique Service Order ID (e.g., S101000587). |
| --- | --- | --- |

| 2 | BSS → SCCM | Login — get access token POST /login with username and password. SCCM returns a JWT Bearer Token. BSS must include this token in the Authorization header of every subsequent API call to SCCM in this session. |
| --- | --- | --- |

| 2a | BSS → SCCM | Validate customer record GET /customers?customer_id= — BSS checks if the customer exists in SCCM. If the customer is new, BSS first creates them: POST /customers with customer_name, customer_id, short_name. |
| --- | --- | --- |

| 2b | BSS → SCCM | Fetch sub-service catalog GET /catalogitem — BSS retrieves the sub_service_id and list of required parameters (User Defined and System Defined) for the specific service type. This determines what fields must be sent in Step 3. |
| --- | --- | --- |

PHASE 2 BSS → SCCM — Three-Step Service Provisioning

How the 3-Step Provisioning Works

Once BSS has authenticated and fetched the catalog, it calls SCCM in three sequential steps. 
Step 1 is the cover sheet (service identity), Step 2 is the technical specifications (which device, which port, QoS requirements), and Step 3 is pressing the Submit button to execute everything.

Each step must succeed before the next begins. If Step 2 fails, Step 3 must not be called. SCCM validates each step before accepting the next.

| 3 | BSS → SCCM | Step 1 — Initiate Service Order POST /servicedeploy/step1. BSS sends the service order ID, customer ID, sub_service_id, device count, topology, location, service VLAN, and ISP name. SCCM creates a transaction record and returns a transaction_id (e.g., 2274). This ID links all three steps together. |
| --- | --- | --- |

| 4 | BSS → SCCM | Step 2 — Device & QoS Parameters POST /servicedeploy/step2. BSS sends the transaction_id plus device-level details: PE hostname (which router), service-specific ID, QoS config (pbit values and CIR % per traffic class: SIGN/CS/DATA/OAM), hub router hostname, interface ID, shaping value (200 Mbps), customer VLANs. |
| --- | --- | --- |

| 5 | BSS → SCCM | Step 3 — Commit & Execute POST /servicedeploy/step3. BSS sends the transaction_id with action=1 (Create). SCCM builds the full NSO payload, runs a dry-run validation, then commits the service. SCCM returns a unique_id (Circuit ID, e.g., CKT-GI-BB-SCCM-S000754). Network provisioning begins in the background. |
| --- | --- | --- |

Note: Step 1,2,3 For Api’s information Refer: Below Phase 4- Point : 4.  API Catalogue — Complete Reference or

PHASE 3 SCCM / NSO — Network Configuration Push

What SCCM and NSO Do —

After Step 3 is committed, SCCM constructs the full device configuration and passes it to NSO. NSO is Cisco's Network Services Orchestrator — it knows how to talk to every device in the network. NSO translates the service description into actual router commands and OLT configuration templates, then pushes them to the devices.

This is the only phase that physically changes the network. Everything before this was just data validation and preparation. If NSO successfully configures all devices, the service is live. If any single device fails, NSO automatically rolls back every change it made — the network is left clean with no partial configuration.

| 6 | SCCM → NSO | Internal: Build NSO payload SCCM resolves device assignments, VLAN resources, interface mappings, and QoS policy parameters. It constructs the NSO service template (JSON/YANG model) and triggers NSO execution via the internal SCCM-NSO API. |
| --- | --- | --- |

| 7a | NSO → Router | Push Router Configuration (ASR 907/920) NSO configures the GP or Mandal router via NETCONF: L2VFI (l2 vfi ... manual, vpn id 3301, bridge-domain 3301), service instance on the physical interface (encapsulation dot1q, rewrite ingress tag, bridge-domain), and QoS policy (MPLS EXP marking, CIR policing). Neighbor pseudowire to HUB router is established. |
| --- | --- | --- |

| 7b | NSO → Nokia OLT (via Nokia AMS SOAP | Push OLT/ONT Configuration — Bridge Mode NSO calls Nokia AMS SOAP API with template ORG_SERV_U_D_CVLAN (Untagged) or ORG_SERV_U_D_CVLAN-Tag (Tagged). Parameters: ONT serial number, ethernet slot (ontCardId=1), bandwidth profiles (UP/DN), CVLAN, SVLAN name, subscriber IDs. |
| --- | --- | --- |

| 7c | NSO → Nokia OLT (via Nokia AMS SOAP) | Push OLT/ONT Configuration — Route Mode NSO calls Nokia AMS SOAP API with template TEMPLATE-GI-BB_ROUTE. Parameters: ONT serial number, VEIP slot (ontCardId=14), VEIP QoS profiles, CVLAN, SVLAN. ONT acts as L3 router with DHCP enabled on WAN interface. |
| --- | --- | --- |

| 7d | NSO → Tejas OLT (via Tejas EMS REST/JSON) | Push OLT/ONT Configuration — Bridge Mode Token generation for session authentication, Add ONT by serial number, Make ONT Admin UP, Set Ethernet port (ontCardId = 1) to Bridged Mode, Retrieve Service UUID from EMS, Tejas EMS: Add UNI (Bridge Mode Untagged), Tejas EMS: Add Location & Customer ID. JSON payloads are exchanged with EMS, which translates them into device-native CLI for OLT/ONT configuration. |
| --- | --- | --- |

| 7e | NSO → Tejas OLT (via Tejas EMS REST/JSON) | Push OLT/ONT Configuration — Route Mode NSO calls the Tejas EMS TAPI REST NBI with Route Mode templates. Token generation for session authentication, Add ONT by serial number, Make ONT Admin UP, Configure VEIP slot (ontCardId = 14), Apply VEIP QoS profiles, Assign CVLAN and SVLAN, Enable DHCP on WAN interface (ONT acts as L3 router), Retrieve Service UUID and bind subscriber details. EMS pushes JSON payload downstream, applying routing and service flows to ONT. |
| --- | --- | --- |

OPTICAL DOMAIN DWDM (NSO to DWDM communication)

PHASE 4 Callback to BSS + OSS Inventory Update

What Happens After Network Config is Pushed

Once NSO confirms all devices are configured successfully, SCCM asynchronously sends a notification back to BSS. This notification (callback) tells BSS the service is live, includes the Circuit ID for reference, and carries the success/failure status.

BSS does not poll SCCM for status. It simply waits on its callback endpoint. When the callback arrives, BSS updates the customer's service order to 'Active' and the customer's service is confirmed.

In parallel, OSS can query SCCM at any time to retrieve the inventory of all active services — which devices each service uses, what the circuit IDs are, and the service topology. OSS uses this for network monitoring and fault management.

| 8 | SCCM → BSS | Service Confirmation Activation Callback (SCCM → BSS) https://bgw-test.tfiber.in/procesprov/serviceConformationActivation After SCCM receives the service provisioning details from NSO and associated systems, SCCM automatically calls this URL to provide service activation parameters such as VLAN and IP details. Callback: Service Completion Notification SCCM POSTs to the BSS callback URL: https://172.28.10.9:30098/rest/thirdparty/notification/servicecompletion. Payload includes: service_order_id (original BSS order), unique_id (circuit ID), status_code=2000 (success). BSS acknowledges and marks the service as ACTIVE. |
| --- | --- | --- |

| 9 | OSS → SCCM | Inventory Query (Ongoing / On-Demand) OSS queries SCCM via GET APIs to maintain network inventory: GET /commited_service_orders (all services), GET /service_order_details (per service with A-end, Z-end, device list), GET /service_list (by date range for audits). OSS records service_id, circuit_id, device list, and service topology. |
| --- | --- | --- |

1.4.  API Catalogue — Complete Reference

This section provides the full details of every API call in the GI-BB provisioning lifecycle, in the order they are executed. All SCCM APIs use base URL: https://172.28.2.101/sccm/sccm_api/web/api/v1/

BSS → SCCM APIs (Calls made FROM BSS TO SCCM)

SCCM → BSS API (Callback: SCCM notifies BSS of completion)

OSS → SCCM APIs (Inventory Queries: OSS reads from SCCM)

1.5.  Complete API Sequence — Master Reference Table

The table below shows ALL API calls in the correct execution order for a complete GI-BB service provisioning lifecycle. Steps 1–8 happen per service order. Steps 9–12 are ongoing OSS inventory queries.

| # | From → To | Method | Endpoint | Purpose |
| --- | --- | --- | --- | --- |
| 1 | BSS → SCCM | POST | /login | BSS gets JWT token for session. Required before any other call. |
| 2 | BSS → SCCM | GET | /customers?customer_id= | BSS validates customer exists. If not, call POST /customers to create. |
| 3 | BSS → SCCM | GET | /catalogitem?service_name=... | BSS gets sub_service_id and required parameters for this service type. |
| 4 | BSS → SCCM | POST | /servicedeploy/step1 | Service initiation. Returns transaction_id. Status code: 2001. |
| 5 | BSS → SCCM | POST | /servicedeploy/step2 | Device, interface, QoS parameters sent. Status code: 2002. |
| 6 | BSS → SCCM | POST | /servicedeploy/step3 | Commit. NSO push begins. Returns unique_id (circuit ID). Status: 2000. |
| 7 | SCCM → NSO | Internal | Internal SCCM-NSO | SCCM builds NSO payload and triggers network configuration push. |
| 7a | NSO → Router | NETCONF | ASR 907 / ASR 920 | L2VFI, bridge-domain, service-instance, QoS policy pushed to GP/Mandal router. |
| 7b | NSO → Nokia OLT | SOAP POST | /soap/ApcRemotePort/9.7 | VLAN, ONT port, bandwidth profile, bridge config pushed via Nokia AMS. |
| 7c | NSO → Tejas OLT | REST PUT/POST | /nmsnbi-rest/tapi/data/ | 8-step Tejas EMS sequence: Token→AddONT→AdminUP→ETH mode→GetUUID→AddUNI→CreateSvc→AddLocation |
| 8 | SCCM → BSS | POST (Callback) | /notification/servicecompletion | SCCM notifies BSS: service is live. Includes service_order_id, unique_id, status_code=2000. |
| 9 | OSS → SCCM | GET | /commited_service_orders | OSS retrieves full inventory of all active services with device lists. |
| 10 | OSS → SCCM | GET | /commited_service_orders?service_id= | OSS queries specific service for fault isolation or inventory check. |
| 11 | OSS → SCCM | GET | /service_list?start_date=&end_date= | OSS retrieves services by date for reconciliation and audit. |
| 12 | OSS → SCCM | GET | /service_order_details?service_order_id= | OSS gets A-end, Z-end, full device path for a specific order. |
| 13 | OSS → SCCM | GET | /serviceinstance?start_date=&end_date= | OSS retrieves list of service instance IDs for inventory delta sync. |

1.6.  Status Codes & Error Handling

| Code | Meaning | When It Occurs | Action Required |
| --- | --- | --- | --- |
| 2001 | Step 1 Success | Step 1 POST accepted. Transaction created. | Proceed to Step 2 with the returned transaction_id. |
| 2002 | Step 2 Success | Step 2 POST accepted. Device parameters stored. | Proceed to Step 3 with the same transaction_id. |
| 2000 | Full Success | Step 3 committed OR callback to BSS on service completion. | BSS marks service as ACTIVE. Store circuit unique_id. |
| 200 | Query Success | OSS GET API returned data successfully. | Process the response data for inventory update. |
| 4xx | Client Error | Bad request, auth failure, invalid parameter, customer not found. | Fix the request. Re-authenticate if 401. Check parameters if 400/404. |
| 5xx | Server / NSO Error | NSO push failed, device unreachable, SCCM internal error. | NSO rolls back all changes automatically. BSS notified via callback with non-2000 code. Raise incident. |

1.7.  System Architecture — NSO

NSO (Network Services Orchestrator — Cisco)

NSO provides the network automation layer. It translates service model instances (YANG) into device-specific commands and manages the transactional push to all devices simultaneously. Key features used in T-FIBER:

- Device Manager: Manages device configurations transactionally. Supports NETCONF, CLI, SNMP via Network Element Drivers (NEDs).

- Service Manager: Maps service templates to device configurations using FASTMAP algorithm. One service definition covers create, modify, and delete automatically.

- Configuration Database (CDB): Real-time database always in sync with actual device configurations. No offline inventory lag.

- Rollback: If any single device fails during a push, NSO rolls back ALL changes across ALL devices in the same transaction. No partial configurations left on the network.

- SCCM Integration: NSO runs on 4 virtual servers (1 CFS + 3 RFS) with a combined capacity of 56 vCPU, 115 GB RAM, and 682 GB storage, running on Ubuntu 24.04.4 LTS.

- SCCM → NSO Flow (Internal)

| A | Flow A — Success | NSO pushes config to all devices → Notification sent to SCCM → SCCM runs & generates pass/fail verification report → SCCM sends callback to BSS with status_code=200. |
| --- | --- | --- |
| B | Flow B — Failure | If NSO fails on even 1 device out of N → NSO rolls back all changes from all devices → Error response sent to SCCM → SCCM provides error response to BSS → BSS does NOT mark service as active. |

1.8.Glossary

| Term | Definition |
| --- | --- |
| GI-BB | Gigabit Broadband — T-FIBER's broadband service for government institutions (schools, GP offices, hospitals). |
| BSS | Business Support System — Customer-facing portal. Accepts orders, validates customers, triggers provisioning, receives completion callbacks. |
| OSS | Operations Support System — Network inventory and assurance. Queries SCCM for service and device data. Does NOT provision. |
| SCCM | Service & Configuration Control Manager — Orchestration middleware between BSS and NSO. Validates, builds, and tracks all service transactions. |
| NSO | Network Services Orchestrator (Cisco) — Pushes actual device configurations. Only system that directly touches routers and OLTs. |
| L2VPN | Layer 2 Virtual Private Network — The technology used for GI-BB. Creates a virtual Ethernet connection between the customer site and the internet gateway. |
| GPON | Gigabit Passive Optical Network — The optical fiber access technology delivering internet from the OLT to the customer's ONT device. |
| OLT | Optical Line Terminal — T-FIBER's equipment at the exchange/distribution point. Connects to multiple customer ONTs via fiber. |
| ONT | Optical Network Terminal — The box installed at the customer premises. Converts optical fiber signal to Ethernet for the customer's devices. |
| transaction_id | Temporary ID returned by Step 1 API. Used to link Steps 2 and 3 together. Not the final service ID. |
| unique_id | Final Circuit ID generated by SCCM after Step 3 succeeds (e.g., CKT-GI-BB-SCCM-S000754). Permanent reference for the active service. |
| sub_service_id | SCCM catalog code identifying the specific service variant: 9.5.2.1 = ASR920 Bridge, 9.5.2.2 = ASR907 Bridge, 9.5.2.4 = ASR920 Route, 9.5.2.5 = ASR907 Route. |
| Bridge Mode | ONT passes Ethernet frames transparently. Customer device gets IP from upstream DHCP. DHCP snooping must be Disabled at OLT level. |
| Route Mode | ONT acts as an IP router. ONT gets WAN IP via DHCP from BRAS/BNG. Assigns private IPs to customer LAN. VEIP slot used (not ETH slot). |
| CVLAN / C-Tag | Customer VLAN — inner VLAN assigned to the customer's ONT port. Range: 2537–2699 for GI-BB. |
| SVLAN / S-Tag | Service VLAN — outer VLAN at the OLT uplink. 3301 is the standard GI-BB SVLAN in T-FIBER. |
| MPLS EXP | MPLS Experimental bits — used for QoS traffic marking in the MPLS core network. GI-BB data traffic marked as EXP=2. |
| JWT Token | JSON Web Token — time-limited authentication credential returned by SCCM /login API. Must be included in all subsequent API calls. |
| /28 Subnet | A block of 16 IP addresses (14 usable). Each GI-BB customer gets one /28 from the IP pool (10.10.x.x / 10.11.x.x / 10.12.x.x). |

2.  NSO VLAN Assignment Models

This section defines VLAN allocation models for different service types in NSO (Network Services Orchestrator), ensuring unique S-Tag/C-Tag ranges, scalability per ISP zone, and clarity for provisioning automation.

VLAN Plan Overview

| Types of Services | Service Types | S-Tag | C-Tag | Remarks |
| --- | --- | --- | --- | --- |
| Customer Tag | NA | NA | 1001–2699 |  |
| Enterprise (VAS) | L2VPN | 2700–2999 | NA |  |
| Enterprise (VAS) | L3VPN | NA | 3000–3200 |  |
| HSI (High Speed Internet) | L2VPN Service | 3201–3230 | 1001–1512 | 3 S-TAG for ISP per Zone, 512 C-TAG per OLT |
| OTT | L2VPN Service | 3231–3260 | 1513–2024 | 3 S-TAG for ISP per Zone, 512 C-TAG per OLT |
| VoIP | L2VPN Service | 3261–3290 | 2025–2536 | 3 S-TAG for ISP per Zone, 512 C-TAG per OLT |
| CCTV Surveillance | L3VPN Service (Hub & Spoke) | 3291–3300 |  |  |
| Gov Services (GI) — 98 departments | L2VPN/L3VPN | 3301–3499 | 2537–2699 |  |
| Reserved |  | 3500–4096 |  |  |
| Household | L2VPN | N/A | 3500–3590 | HH PoC Pioneer |
| Household | L2VPN | N/A | 3591–3680 | HH PoC BSNL |
| Household | L2VPN | N/A | 3681–3770 | HH PoC SSC |

Design Principles

- Isolation: Each service type has a dedicated VLAN pool to avoid overlap.

- Scalability: ISP zones provisioned with 3 S-TAGs; each OLT supports 512 C-TAGs.

- Flexibility: Reserved VLANs (3500–4096) for future services.

- Clarity: Household PoC VLANs separated by provider (Pioneer, BSNL, SSC).

Workflow Mapping in NSO

Step 1 — Service Request to NSO

- Identify service type (HSI, OTT, VoIP, etc.).

- Map to VLAN pool based on the table above.

Step 2 — VLAN Assignment Logic

- Allocate next available S-Tag/C-Tag from pool.

- Ensure per-zone ISP allocation rules are enforced.

Step 3 — Provisioning

- Push VLAN assignment to PE/OLT via NSO templates.

- Maintain audit trail in NSO DB for reuse and rollback.

Step 4 — Monitoring

- Track VLAN utilization per service type.

- Trigger alerts when pool utilization >80%.

Example — HSI Service Provisioning

Scenario: Customer request for HSI in Zone A NSO Logic: - Assign S-Tag from 3201–3230 (Zone A ISP pool). - Assign C-Tag from 1001–1512 (OLT pool). Output: VLAN pair (e.g., S-Tag 3205, C-Tag 1010) provisioned on OLT.

4.  IP Pool Assignment Models

For GI-BB
T-Fiber Private IPs

| Field | Value |
| --- | --- |
| IP Block | 10.10.10.0/8 |
| IP Address | 10.10.0.0 |
| Network Address | 10.0.0.0 |
| Usable Host IP Range | 10.0.0.1 — 10.255.255.254 |
| Broadcast Address | 10.255.255.255 |
| Total Number of Hosts | 16,777,216 |
| Number of Usable Hosts | 16,777,214 |
| Subnet Mask | 255.0.0.0 |
| Wildcard Mask | 0.255.255.255 |
| Binary Subnet Mask | 11111111.00000000.00000000.00000000 |
| IP Class | A |
| CIDR Notation | /8 |
| IP Type | Private |

T-Fiber Public IPs

Public IP Blocks

103.162.5.0/24

118.91.190.0/24

202.89.73.0/24

IPv4 pool allocation concept:
When allocating IP addresses manually from a defined pool, the CIDR (Classless Inter-Domain Routing) notation dictates the block size and valid starting points.

- Each subnet must begin at a CIDR-aligned boundary.

- Allocation is based on block size, not on request order.

- This ensures no overlap, no fragmentation, and optimal utilization of the pool.

Base IPv4 Pool

- Pool: 172.16.1.0/24

- Total IPs: 256 (from .0 to .255)

- This pool can be divided into smaller subnets based on CIDR masks.

CIDR Boundary Rules

/30 → Block size = 4 → Valid starts: 0, 4, 8, …
/29 → Block size = 8 → Valid starts: 0, 8, 16, …
/28 → Block size = 16 → Valid starts: 0, 16, 32, …
/27 → Block size = 32 → Valid starts: 0, 32, 64, …
/26 → Block size = 64 → Valid starts: 0, 64, 128, …
/25 → Block size = 128 → Valid starts: 0, 128

These rules prevent misaligned subnets (e.g., you cannot start a /28 at 172.16.1.20 because 20 is not a multiple of 16).

Subnet Allocation Example (from your table)

| Request | CIDR | Subnet Allocated | Address Range |
| --- | --- | --- | --- |
| 1 | /30 | 172.16.1.0/30 | .0 – .3 |
| 2 | /25 | 172.16.1.128/25 | .128 – .255 |
| 3 | /26 | 172.16.1.64/26 | .64 – .127 |
| 4 | /27 | 172.16.1.32/27 | .32 – .63 |
| 5 | /28 | 172.16.1.16/28 | .16 – .31 |
| 6 | /29 | 172.16.1.8/29 | .8 – .15 |
| 7 | /30 | 172.16.1.4/30 | .4 – .7 |

Notice how each subnet starts at the correct boundary and covers exactly its block size.

5.  Traffic Priority for QoS in GPON

Quality of Service (QoS) in the GI-BB GPON network operates at two distinct and separate layers. Both are configured automatically by NSO — no manual intervention is required for any customer provisioning.

| Layer | Where It Applies | What It Does |
| --- | --- | --- |
| Layer 1 | OLT and ONT port | Bandwidth enforcement — upstream policing and downstream shaping per ONT port. This is where the customer's 50 Mbps speed limit is enforced. Done via Nokia AMS SOAP or Tejas EMS REST. |
| Layer 2 | GP/Mandal Router (ASR) | MPLS EXP marking and traffic class prioritisation. GI-BB data traffic is marked EXP=2 in the MPLS core so network routers know which queue to use for this service. Done via NSO NETCONF to ASR 907/920. |

5.1.1  QoS Parameters

| QoS Parameter | Carried In | Purpose |
| --- | --- | --- |
| Bandwidth profile name (e.g., UP_TNET_GI_50M) | Nokia AMS SOAP call | Enforces upstream speed limit (50 Mbps) on the ONT port at the OLT. Profile must pre-exist in OLT. |
| Shaper profile name (e.g., DN_TNET_GI_50M) | Nokia AMS SOAP call | Enforces downstream speed limit (50 Mbps) shaping at the OLT for traffic destined to this ONT. |
| eth-node-edge-point-count (Tejas) | Tejas EMS REST call | Number of ONT LAN ports to enable. Not a speed limit — a port activation count. |
| upstream-bandwidth-profile-uuid (Tejas) | Tejas EMS REST AddUNI | Upload bandwidth profile (e.g., UP_TNET_GI_50M) linked to customer's UNI endpoint. |
| downstream-bandwidth-profile-uuid (Tejas) | Tejas EMS REST AddUNI | Download profile — must be prefixed with 'TejNMS-' for Tejas (e.g., TejNMS-DN_TNET_GI_50M). |
| pbit-value + cir-value + traffic-type | BSS Step 2 → SCCM → NSO → Router | IEEE 802.1p priority and CIR per traffic class (SIGN/CS/DATA/OAM). Used to build router policy-map. |
| shape-value (bps) | BSS Step 2 → SCCM → NSO → Router | Interface-level traffic shaper on the GP/Mandal router. Controls total egress rate on the customer port. |
| MPLS EXP = 2 | NSO → Router (NETCONF) | All GI-BB data traffic entering the MPLS core is marked with EXP bit = 2. This is the global GI-BB QoS marking for the T-Fiber MPLS network. |
| ingress-qos GI_BB | NSO → OLT CLI (one-time) | Ingress QoS profile applied to SVLAN 3301. Maps all dot1p values (0–7) to traffic class 1. Attached to SVLAN during OLT commissioning. |
| outer-pbit-operation: COPY_PBIT | Tejas EMS REST CreateService | Tells Tejas OLT to copy the upstream dot1p priority bits from the customer frame into the outer SVLAN header for QoS treatment in the MPLS core. |

5.2. NSO Pre-configures QoS Infrastructure on OLT

Before customer provisioning, NSO performs a one-time OLT commissioning activity to create the QoS infrastructure required for GI-BB services. These configurations are mandatory and must exist before ONT provisioning can be executed successfully.

Step B1 — Pre-Configured QoS Components

NSO pushes each of these CLI commands to the Nokia OLT via the AMS. These define the upstream speed enforcement rules for every speed tier T-Fiber offers.

! 30 Mbps Upload Profile ← UP_TNET_GI_0M ! 50 Mbps Upload Profile ← UP_TNET_GI_50M ! 100 Mbps Upload Profile ← UP_TNET_GI_100M ! 500 Mbps Upload Profile ← UP_TNET_GI_500M ! 1 Gbps Upload Profile ← UP_TNET_GI_1G

Step B2 — Download Shaper Profiles (one per speed tier)

Downstream shaping profiles — applied at OLT to rate-limit traffic going TO the customer's ONT.

! 30 Mbps Download Shaper ← DN_TNET_GI_30M ! 50 Mbps Download Shaper ← DN_TNET_GI_50M ! 100 Mbps Download Shaper ← DN_TNET_GI_100M ! 500 Mbps Download Shaper ← DN_TNET_GI_500M ! 1 Gbps Download Shaper ← DN_TNET_GI_1G

Step B3 — Ingress QoS Profile (dot1p → traffic class mapping)

Maps all IEEE 802.1p priority values (0 through 7) to traffic class 1. This means all GI-BB traffic — regardless of what dot1p the customer's device sets — is treated as the same traffic class inside the OLT.

configure qos profiles ingress-qos GI_BB dot1-p0-tc 1 dot1-p1-tc 1 dot1-p2-tc 1 dot1-p3-tc 1 dot1-p4-tc 1 dot1-p5-tc 1 dot1-p6-tc 1 dot1-p7-tc 1 -- All 8 dot1p bits (0-7) → traffic class 1 for GI_BB ingress

Step B4 — SVLAN with Ingress QoS Profile Attached

The SVLAN for GI-BB (3301) is created with the ingress QoS profile attached. Every ONT on this SVLAN automatically gets the GI_BB QoS treatment.

! Create SVLAN 3301 with GI_BB ingress QoS profile configure vlan id 3301 mode residential-bridge name GI-Internet_S3301 in-qos-profname name:GI_BB ← GI_BB ingress QoS profile attached here new-broadcast enable ipv6-mcast-ctrl ! Map SVLAN to OLT uplink — PizzaBox OLT configure bridge port nt:xfp:1 vlan-id 3301 ! Map SVLAN to OLT uplink — Chassis OLT configure service vpls 3301 customer 1 v-vpls vlan 3301 create configure service vpls 3301 description GI_BB_Internet configure service vpls 3301 sap lag-1:3301 create configure service vpls 3301 sap lag-1:3301 no shutdown configure service vpls 3301 sap lt:1/1/1:3301 create configure service vpls 3301 sap lt:1/1/1:3301 no shutdown configure service vpls 3301 no shutdown

Step B5 — Bandwidth Profile Table (Speed → Profile Name Reference)

| Mode | Speed | Upload Profile Name | Download Profile Name |
| --- | --- | --- | --- |
| Bridge Untagged | 30 Mbps | UP_TNET_GI_30M | DN_TNET_GI_30M |
| Bridge Untagged | 50 Mbps ★ | UP_TNET_GI_50M | DN_TNET_GI_50M |
| Bridge Untagged | 100 Mbps | UP_TNET_GI_100M | DN_TNET_GI_100M |
| Bridge Untagged | 500 Mbps | UP_TNET_GI_500M | DN_TNET_GI_500M |
| Bridge Untagged | 1 Gbps | UP_TNET_GI_1G | DN_TNET_GI_1G |
| Bridge Tagged | 50 Mbps | UP_TNET_GI_50M-Tag | DN_TNET_GI_50M |
| Bridge Tagged | 100 Mbps | UP_TNET_GI_100M-Tag | DN_TNET_GI_100M |
| Route Mode | 50 Mbps | UP_TNET_GI_50Mb | DN_TNET_GI_50M |
| Route Mode | 100 Mbps | UP_TNET_GI_100Mb | DN_TNET_GI_100M |

NOTE: ★ = used in the customer example throughout this document. For Tejas EMS downstream profiles, prefix 'TejNMS-' must be added: TejNMS-DN_TNET_GI_50M.

5.3.  — QoS Parameters Flow: Customer → BSS → SCCM → NSO

|  | Customer orders 50 Mbps Internet — QoS flows through BSS Steps 1, 2, 3 to SCCM to NSO |
| --- | --- |

Customer: Government School. Plan: 50 Mbps Internet. Mode: Bridge Untagged. OLT: Nokia. Router: ASR920. This example uses the exact values from the BSS API and GIBB E2E documents.

5.3.1  BSS → SCCM  Step 1 — Service Initiation

Step 1 tells SCCM the service type, product, topology, and SVLAN. No QoS parameters in Step 1. SCCM responds with a transaction_id used in Steps 2 and 3.

| POST | https://172.28.2.101/sccm/sccm_api/web/api/v1/servicedeploy/step1 |
| --- | --- |

REQUEST: { "service_order_id": "ORD000355", -- BSS order number "customer_id": "CUST000023", -- Customer in SCCM "sub_service_id": "9.5.2.1", -- Nokia OLT + ASR920 + Bridge Untagged "endpoint_device_count": 1, "topology": 5, -- Hub & Spoke L2VPN topology "location_id": "2", "service_vlan_id": "3301" -- SVLAN for GI-BB } RESPONSE: { "status_code": 2001, -- 2001 = Step 1 success "message": "Service Invoke Create Step1 Success", "data": { "transaction_id": 2274 } -- Used in Step 2 and Step 3 }

| Field | Value | What SCCM Does With It |
| --- | --- | --- |
| sub_service_id | 9.5.2.1 | Identifies: Nokia OLT → ASR920 → Bridge Mode Untagged. SCCM selects template ORG_SERV_U_D_CVLAN for Nokia AMS SOAP call. |
| service_vlan_id | 3301 | GI-BB SVLAN. Used in Nokia AMS networkVlanName and Tejas EMS service creation. |
| topology | 5 | Hub & Spoke L2VPN. SCCM will configure L2VFI + pseudowire on router pointing to HUB ZCR. |

5.3.2  BSS → SCCM  Step 2 — Device and QoS Parameters

Step 2 is where ALL QoS parameters are passed from BSS to SCCM. This includes per-traffic-class pbit and CIR values, the hub-spoke router topology, interface shaper, and customer VLANs. SCCM uses this data to build the complete NSO YANG payload.

| POST | https://172.28.2.101/sccm/sccm_api/web/api/v1/servicedeploy/step2 |
| --- | --- |

REQUEST: { "transaction_id": 2274, -- from Step 1 response "endpoint_devices": [ { "pe_host_name": "NOCTESTGAC001", -- GP Router hostname (ASR920) "service_specific_id": "ORD000355", "qos-config": [ { "pbit-value": "7", -- IEEE 802.1p priority 7 = highest "cir-value": "20", -- 20 Mbps CIR for this class "traffic-type": "SIGN" -- Signalling (SIP, control) }, { "pbit-value": "6", -- Priority 6 "cir-value": "20", -- 20 Mbps "traffic-type": "CS" -- Circuit-switched / voice }, { "pbit-value": "5", -- Priority 5 "cir-value": "20", -- 20 Mbps "traffic-type": "DATA" -- Internet data (HTTP, streaming) }, { "pbit-value": "4", -- Priority 4 "cir-value": "40", -- 40 Mbps "traffic-type": "OAM" -- Operations & Management traffic } ], "parameters": { "hub-hostname": "NOC507RTRN00001", -- ZCR Hub router to connect to "interface-id": "Gig0/0/8", -- Physical port on GP router "shape-value": "200000000", -- 200 Mbps interface shaper (bps) "cir-value": "200000000", -- 200 Mbps CIR on interface "port-count": 1, "customer-vlans": "2542" -- Customer CVLAN } } ] } RESPONSE: { "status_code": 2002, -- 2002 = Step 2 success "message": "Service Invoke Create Step2 Success", "data": { "transaction_id": 2274 } }

Step 2 QoS Field Explanation

| Field | Value | Traffic Type | Meaning and How SCCM Uses It |
| --- | --- | --- | --- |
| pbit-value=7, cir=20 | 7 | SIGN | Highest priority (7). Signalling traffic. SCCM maps this to a router policy-map class with 20 Mbps guaranteed rate. Priority queued in MPLS core. |
| pbit-value=6, cir=20 | 6 | CS | Second highest. Circuit-switched or voice. 20 Mbps guaranteed. Real-time traffic class. |
| pbit-value=5, cir=20 | 5 | DATA | Internet data — web browsing, video streaming, downloads. 20 Mbps guaranteed. Marked MPLS EXP=2 in core. This is the customer's internet service traffic. |
| pbit-value=4, cir=40 | 4 | OAM | Operations and Management. 40 Mbps guaranteed. Management plane traffic (SNMP, TR-069, OAM probes). |
| shape-value | 200000000 | All | 200 Mbps total interface shaper on the GP router's customer-facing port. This is the maximum aggregate rate across all traffic classes. |
| hub-hostname | NOC507RTRN00001 | — | ZCR Hub router. SCCM uses this to configure the L2VFI neighbor (pseudowire endpoint) on the GP router. |

5.3.3  BSS → SCCM  Step 3 — Commit and Execute

Step 3 commits the transaction. SCCM takes all data from Steps 1 and 2, builds the complete NSO YANG service model, and sends it to NSO for execution. NSO then pushes configurations to all devices simultaneously.

| POST | https://172.28.2.101/sccm/sccm_api/web/api/v1/servicedeploy/step3 |
| --- | --- |

REQUEST: { "transaction_id": 2274, "action": 1 -- action=1 means commit and push to network } RESPONSE: { "status_code": 2000, -- 2000 = Full success "message": "Service Invoke Create Step3 Success", "data": { "unique_id": "CKT-GI-BB-SCCM-S000754" -- Circuit ID assigned to this service } }

NOTE: After Step 3 is sent, NSO starts pushing configurations to all devices. BSS does NOT get an immediate synchronous response about device configuration status. BSS waits for the SCCM callback (Step 9 in the sequence) which arrives after NSO confirms all devices are configured.

5.4  — NSO Pushes QoS to OLT (Nokia AMS SOAP / Tejas EMS REST)

| NSO → OLT | NSO references pre-built QoS profiles and links them to the customer's ONT port |
| --- | --- |

NSO does NOT re-create the bandwidth profiles for each customer. The profiles (UP_TNET_GI_50M, DN_TNET_GI_50M) were created once during OLT commissioning (Section 2). NSO only references these profile names in the per-customer provisioning call.

5.4.1  Nokia AMS — SOAP Call with QoS Profiles (Bridge Mode 50 Mbps)

ontEthPortQosProfiles_BandwidthProfileUsQueue1 and ontEthPortQosProfiles_ShaperProfileDsQueue0 are the QoS parameters in the Nokia AMS SOAP configure call.

| SOAP | http://172.28.39.12:8080/soap/services/ApcRemotePort/9.7 |
| --- | --- |

<ns:configure> <objectName>NOCTESTOLTN001:1-1-1-2-10</objectName> <!-- OLT Hostname : PON Port (4 digits) : ONT ID (5th digit) --> <templateName>ORG_SERV_U_D_CVLAN</templateName> <templateVersion>2</templateVersion> <!-- ONT identification --> <argument><n>ontSerialNumber</n><value>TJNW957F5907</value></argument> <argument><n>ontCardId</n><value>1</value></argument> <argument><n>oper_NumberOfDataPorts</n><value>4</value></argument> <argument><n>ethPortId</n><value>1</value></argument> <!-- ▼▼ QoS PARAMETERS — links 50 Mbps profiles to this ONT port ▼▼ --> <argument> <n>ontEthPortQosProfiles_BandwidthProfileUsQueue1</n> <value>UP_TNET_GI_50M</value> <!-- Upload: OLT will police upstream traffic from this ONT to 50 Mbps --> </argument> <argument> <n>ontEthPortQosProfiles_ShaperProfileDsQueue0</n> <value>DN_TNET_GI_50M</value> <!-- Download: OLT will shape downstream traffic to this ONT to 50 Mbps --> </argument> <!-- ▲▲ QoS PARAMETERS END ▲▲ --> <!-- VLAN parameters --> <argument><n>vlanCustomerId</n><value>2542</value></argument> <argument><n>networkVlanName</n><value>GI-Internet_S3301</value></argument> <argument><n>ontSubscriberId2</n><value>TF-S000355-GIBB_2542-50Mbps</value></argument> </ns:configure>

What Nokia AMS Does After Receiving This SOAP Call

| Action | QoS Effect |
| --- | --- |
| Registers ONT TJNW957F5907 on PON port 1-1-1-2 | ONT is now known to the OLT. Ready for traffic. |
| Links UP_TNET_GI_50M to ONT Eth port | OLT enforces 50 Mbps upstream police. If customer sends more than 51200 kbps, OLT drops excess frames. |
| Links DN_TNET_GI_50M shaper to ONT port | OLT shapes downstream to 50 Mbps. Customer cannot receive more than 51200 kbps download. |
| Applies CVLAN 2542 to Eth port 1 | All frames from this ONT are tagged with CVLAN 2542 at the OLT. |
| SVLAN 3301 already has GI_BB ingress QoS attached | All dot1p values from customer device are mapped to traffic class 1 at OLT ingress. SVLAN 3301 carries traffic to the core. |

5.4.2  Nokia AMS — SOAP Call for Route Mode (VEIP — 50 Mbps)

For Route Mode, the QoS parameters are identical in value but the parameter name prefix changes from ontEthPortQosProfiles_ to ontVeipPortQosProfiles_ because the WAN interface is the VEIP port (slot 14), not the physical Ethernet port.

<ns:configure> <templateName>TEMPLATE-GI-BB_ROUTE</templateName> <argument><n>ontCardId</n><value>14</value></argument> <argument><n>oper_NumberOfDataPorts</n><value>1</value></argument> <argument><n>veipPortId</n><value>1</value></argument> <!-- QoS on VEIP WAN interface — same profile names, different prefix --> <argument> <n>ontVeipPortQosProfiles_BandwidthProfileUsQueue1</n> <value>UP_TNET_GI_50M</value> <!-- Upload: 50 Mbps on VEIP WAN port — same profile, VEIP-specific parameter --> </argument> <argument> <n>ontVeipPortQosProfiles_ShaperProfileDsQueue0</n> <value>DN_TNET_GI_50M</value> <!-- Download: 50 Mbps shaper on VEIP WAN port --> </argument> </ns:configure>

5.4.3  Tejas EMS — REST API QoS Parameters (Bridge Mode)

For Tejas ONTs, QoS bandwidth profiles are passed in Step 6 (Add UNI) of the 8-step sequence. The profile names follow the same naming convention but downstream must be prefixed with 'TejNMS-'.

| POST | https://172.28.7.85:446/nmsnbi-rest/tapi/data/context/connectivitycontext/connectivity-service/connectivityserviceEndPoint?activate=true |
| --- | --- |

REQUEST (Add UNI — Step 6): [{ "uuid": "ebd022b9-1bef-4e28-9e26-f1f23c22df98", -- Service UUID from Step 5 "end-point": [{ "name": [ { "value-name": "SerialNumber", "value": "TJNW95D01D3F" }, { "value-name": "ONTEthPortInstance","value": "1" } ], "layer-protocol-name": "ETH", "nrp-carrier-eth-connectivity-end-point-resource": { "ce-vlan-id-list-and-untag": { "vlan-id": [{ "vlan-id": 2542 }], -- Customer CVLAN "untagged-and-prio-tagged-included": true }, "cos-identifier-list": [{ "cos-name": "0" }] }, -- ▼▼ QoS PARAMETERS FOR TEJAS OLT ▼▼ "upstream-bandwidth-profile": { "upstream-bandwidth-profile-uuid": "UP_TNET_GI_50M" -- Upload: 50 Mbps profile applied to this customer's UNI }, "downstream-bandwidth-profile": { "downstream-bandwidth-profile-uuid": "TejNMS-DN_TNET_GI_50M" -- Download: MUST prefix 'TejNMS-' for Tejas downstream profiles }, -- ▲▲ QoS PARAMETERS END ▲▲ "additional-information": [ { "value-name": "outer-pbit-operation", "value": "COPY_PBIT" }, -- Copies upstream dot1p priority bits into SVLAN header -- This preserves traffic class info in the MPLS core { "value-name": "ingress-operation", "value": "Swap" }, { "value-name": "ingress-vlan1", "value": "3301" } -- Swaps CVLAN 2542 to SVLAN 3301 at OLT uplink ] }] }] RESPONSE: { "statusCode": 200, "msg": "ConnectivityService EndPoint added and activated Successfully" }

NOTE: COPY_PBIT — this is a critical QoS parameter for Tejas. It tells the Tejas OLT to copy the IEEE 802.1p priority bits from the customer's frame into the outer SVLAN header. Without this, the MPLS core routers would not know the traffic priority and could not enforce QoS in the network core.

5.5  — NSO Pushes QoS Policy to Router (NETCONF / CLI)

| NSO → Router | NSO pushes MPLS EXP marking and traffic class policy-map to GP/Mandal router (ASR 907/920) |
| --- | --- |

NSO pushes the router configuration via NETCONF to the GP Router (ASR920) at the customer's location. The router is the entry point into the T-Fiber MPLS core. This is where traffic is marked with MPLS EXP bits so that core routers know how to prioritise it.

5.5.1  Full Router Configuration Pushed by NSO

! ============================================================ ! L2VFI Configuration — Creates the Hub & Spoke L2VPN service ! ============================================================ l2 vfi GIBB_ORD000355 manual vpn id 3301 -- VPN identifier for GI-BB service bridge-domain 3301 mtu 9100 -- Jumbo frame support neighbor 172.17.236.1 encapsulation mpls -- Pseudowire to HUB ZCR router (NOC507RTRN00001) ! ============================================================ ! Service Instance — Maps customer CVLAN to bridge domain ! ============================================================ interface GigabitEthernet0/0/8 service instance 3301 ethernet encapsulation dot1q 2542 -- Customer CVLAN 2542 rewrite ingress tag push dot1q 3301 symmetric -- Tags upstream frame with SVLAN 3301 bridge-domain 3301 ! ============================================================ ! QoS Policy — MPLS EXP marking + policing per traffic class ! ============================================================ policy-map GIBB-IN class class-default set qos-group 2 set mpls experimental imposition 2 -- ALL GI-BB data marked EXP=2 police cir 200000000 bc <BC_VALUE> conform-action transmit exceed-action drop -- Traffic above 200 Mbps CIR dropped interface GigabitEthernet0/0/8 service-policy input GIBB-IN -- Apply QoS inbound on customer port

5.5.2  Traffic Class to MPLS EXP Mapping — GI-BB

— Logical Inventory: 'MPLS EXP (QoS) = 2 — Data queue marking for GI-BB traffic in MPLS network.'

| Traffic Type | dot1p (pbit) | CIR (Mbps) | MPLS EXP | Router Queue | Meaning |
| --- | --- | --- | --- | --- | --- |
| SIGN — Signalling | 7 (highest) | 20 Mbps | 7 | Priority / Strict | SIP, H.323, network control. Highest priority. Guaranteed delivery. |
| CS — Circuit Switched | 6 | 20 Mbps | 6 | Expedited | Voice calls. Second highest. Real-time traffic, delay-sensitive. |
| DATA — Internet | 5 | 20 Mbps | 2 ★ | Data / Best Effort | HTTP, HTTPS, streaming, file downloads. Customer internet traffic. Marked EXP=2 in core. |
| OAM — Management | 4 | 40 Mbps | 4 | Management | SNMP, TR-069, OAM probes, management plane. 40 Mbps guaranteed. |

NOTE: ★ MPLS EXP=2 is the fixed marking for all GI-BB DATA traffic in the T-Fiber MPLS core. This is set by the router policy-map GIBB-IN applied by NSO. All data traffic — regardless of what the customer's device marks — exits the GP router with EXP=2.

5.5.3  Hub ZCR Router Configuration (NSO also pushes this)

NSO also configures the Hub router (ZCR — ASR 9000) to accept the pseudowire from this new customer's spoke router. The hub router terminates all spoke pseudowires for GI-BB.

! Hub Router — ZCR (ASR 9000) — NOC507RTRN00001 L2VPN bridge group GIBB bridge-domain 3301 mtu 9100 interface Bundle-Ether15.3301 ! vfi GIBB_HUB neighbor <SPOKE1_LOOPBACK> pw-id 3301 pw-class PW1 neighbor <SPOKE2_LOOPBACK> pw-id 3301 pw-class PW1 -- New spoke (this customer's GP router) added here neighbor <NEW_GP_ROUTER_LOOPBACK> pw-id 3301 pw-class PW1

5.6  — SCCM Callback to BSS and Live QoS State

| AFTER | NSO confirms all devices configured — SCCM sends callback to BSS — service is ACTIVE |
| --- | --- |

5.6.1  SCCM Callback to BSS

After NSO successfully pushes configuration to all devices (router and OLT), SCCM sends a callback notification to BSS. BSS marks the customer ACTIVE and triggers billing.

| POST | https://172.28.10.9:30098/rest/thirdparty/notification/servicecompletion |
| --- | --- |

REQUEST (SCCM → BSS): { "service_order_id": "ORD000355", "unique_id": "CKT-GI-BB-SCCM-S000754", -- Circuit ID "status_code": 2000, -- 2000 = Full success "deviceid": "1241,1242", -- Device IDs configured "playbook_status": "NA" -- Direct NSO push, no Ansible } BSS Response: Service Completion Notification Received Successfully

5.6.2  QoS State After Successful Provisioning

| Where | QoS Active State | Enforcement Mechanism |
| --- | --- | --- |
| Nokia OLT — Upstream (from ONT) | 50 Mbps hard limit. Frames above 51200 kbps dropped at OLT. | UP_TNET_GI_50M bandwidth profile policing on ONT Eth port. |
| Nokia OLT — Downstream (to ONT) | 50 Mbps shaped. Customer cannot receive more than 51200 kbps. | DN_TNET_GI_50M shaper profile on ONT Eth port. |
| Tejas OLT — Upstream | 50 Mbps hard limit per UNI (customer port). | UP_TNET_GI_50M profile in AddUNI call. |
| Tejas OLT — Downstream | 50 Mbps shaped. | TejNMS-DN_TNET_GI_50M profile in AddUNI call. |
| GP Router — Ingress | All frames entering from customer port marked MPLS EXP=2. Policed at 200 Mbps CIR. | policy-map GIBB-IN applied inbound on Gig0/0/8. |
| MPLS Core — All Core Routers | GI-BB data traffic forwarded in EXP=2 queue. Lower priority than EXP>2 traffic. | MPLS EXP=2 marking set by GP router policy-map. |
| Hub ZCR Router — Egress | Traffic from all spokes aggregated. Forwarded toward BNG/internet. | L2VPN bridge-domain 3301 terminates all pseudowires. |

5.6.3  Complete API Sequence Summary — QoS Perspective

| # | From | To | API | QoS Data Carried |
| --- | --- | --- | --- | --- |
| 1 | BSS | SCCM | Step 1 | sub_service_id=9.5.2.1 → tells SCCM which OLT type and mode → selects correct AMS template. |
| 2 | BSS | SCCM | Step 2 | qos-config[pbit+cir+traffic-type], shape-value, hub-hostname, customer-vlans. ALL QoS parameters for router. |
| 3 | BSS | SCCM | Step 3 | action=1 — triggers NSO execution. No QoS data, just commit signal. |
| 4 | SCCM | NSO | Internal | NSO YANG model built by SCCM containing all service parameters including bandwidth profile names and router policy parameters. |
| 5 | NSO | Nokia AMS | SOAP POST | ontEthPortQosProfiles_BandwidthProfileUsQueue1=UP_TNET_GI_50M, ShaperProfileDsQueue0=DN_TNET_GI_50M — links 50 Mbps QoS to ONT port. |
| 5 | NSO | Tejas EMS | REST POST | upstream-bandwidth-profile-uuid=UP_TNET_GI_50M, downstream-bandwidth-profile-uuid=TejNMS-DN_TNET_GI_50M, outer-pbit-operation=COPY_PBIT. |
| 6 | NSO | ASR 907/920 | NETCONF | policy-map GIBB-IN: set mpls experimental imposition 2, police cir 200000000, pbit class entries (SIGN/CS/DATA/OAM). |
| 7 | NSO | ZCR Hub | NETCONF | Adds new spoke pseudowire neighbor to Hub's L2VPN bridge-domain 3301. |
| 8 | SCCM | BSS | Callback | status_code=2000, unique_id=CKT-GI-BB-SCCM-S000754. BSS marks customer ACTIVE. QoS is live. |

2.  ONT LAN Port Activation During Service Provisioning

2.1 Overview

During ONT installation, all Ethernet LAN ports remain in a disabled (SHUT) state by default. As part of the automated provisioning workflow, NSO activates the required ONT ports through Nokia AMS SOAP APIs or Tejas EMS REST APIs. No manual intervention is required on the ONT.

The number of ports activated depends on:

- Service Mode (Bridge or Route)

- ONT Vendor and Model

- Service Template selected during provisioning

This activation ensures that the customer-facing interfaces become operational immediately after successful service provisioning.

2.2  Port Activation Logic

The primary parameter controlling port activation is:

oper_NumberOfDataPorts

This parameter specifies the number of ONT Ethernet interfaces to be enabled.

Service Mode Mapping

| Service Mode | Template | Slot Type | Ports Enabled |
| --- | --- | --- | --- |
| Bridge Mode — Untagged | ORG_SERV_U_D_CVLAN | Ethernet | 4 LAN Ports |
| Bridge Mode — Tagged | ORG_SERV_U_D_CVLAN-Tag | Ethernet | 4 LAN Ports |
| Route Mode | TEMPLATE-GI-BB_ROUTE | VEIP | 1 VEIP Port |

Bridge Mode

- ONT acts as a Layer-2 bridge.

- Ethernet LAN ports are activated.

- Customer receives service through physical ONT LAN ports.

Route Mode

- ONT acts as a Layer-3 router.

- VEIP (Virtual Ethernet Interface Point) is activated.

- WAN IP is assigned on the VEIP interface.

- Customer LAN ports continue to be available behind the routing function.

2.3  ONT Model Port Allocation

The number of ports enabled is determined based on the installed ONT model.

Examples

| OEM | ONT Model | Type | Slot Type | Slot No. | No. of Ports |
| --- | --- | --- | --- | --- | --- |
| Nokia | G-240G-E | Non-WiFi | ethernet | 1 | 4 |
| Nokia | G-240G-E | Non-WiFi | veip | 14 | 1 |
| Nokia | G-2425G-A | WiFi ONT | ethernet | 1 | 4 |
| Nokia | G-2425G-A | WiFi ONT | veip | 14 | 1 |
| Tejas | TJ2100N-11G | Non-WiFi | ethernet | 1 | 4 |
| Tejas | TJ2100N-11G | Non-WiFi | veip | 4 | 1 |
| Tejas | TJ2100N-11GT | Non-WiFi | ethernet | 1 | 4 |
| Pioneer | DTP9762 | WiFi ONT | ethernet | 1 | 2 |
| Dasan | H660GM-A | WiFi ONT | ethernet | 1 | 4 |
| Altice | SD6BKGC | WiFi ONT | ethernet | 1 | 2 |
| NetLink | HG323DAC | WiFi ONT | ethernet | 4 | 4 |
| Tejas | TJ2100N-G004011EAC1 | WiFi ONT | ethernet | 1 | 4 |

NSO derives the required port count from the ONT model inventory and passes this information to the EMS during provisioning.

2.4  ONT Port Activation Workflow

Nokia OLT

For Nokia GPON deployments:

- NSO invokes Nokia AMS SOAP APIs.

- Service templates are applied to the ONT.

- Parameter oper_NumberOfDataPorts determines the number of ports to activate.

- AMS programs the OLT configuration.

- OLT enables the corresponding ONT Ethernet or VEIP interfaces.

NSO sends this SOAP call to Nokia AMS. The oper_NumberOfDataPorts parameter inside the call is what enables the ONT LAN ports:

Tejas OLT

For Tejas GPON deployments:

- NSO invokes Tejas EMS REST APIs.

- ONT is pre-provisioned with the required Ethernet port count.

- Ports are activated using:

  - node-edge-point-count

  - administrative-state = UNLOCKED

- EMS programs the OLT configuration.

- ONT interfaces become operational.

2.5  Key Configuration Parameters

| Parameter | Platform | Purpose |
| --- | --- | --- |
| oper_NumberOfDataPorts | Nokia AMS | Number of ONT LAN ports to activate |
| ontCardId | Nokia AMS | Ethernet or VEIP slot selection |
| ethPortId | Nokia AMS | Physical Ethernet interface identifier |
| veipPortId | Nokia AMS | VEIP interface identifier |
| node-edge-point-count | Tejas EMS | Number of Ethernet ports provisioned |
| administrative-state=UNLOCKED | Tejas EMS | Activates the ONT interface |
| Bandwidth Profile | Nokia AMS | Upload QoS profile |
| Shaper Profile | Nokia AMS | Download QoS profile |

2.6 Port State Lifecycle

| Service Event | Port State | Description |
| --- | --- | --- |
| ONT Installation | SHUT / DISABLED | Default state after installation |
| Service Provisioning | ENABLED / UNSHUT | Ports activated through NSO provisioning |
| Service Suspension | SUSPENDED | Service temporarily disabled |
| Service Resumption | ENABLED / UNSHUT | Service restored |
| Service Disconnection | UNCONFIGURED | Service configuration removed and ports disabled |

2.7  Summary

ONT port activation is fully automated through NSO-driven provisioning workflows. Based on service type, ONT model, and provisioning template, NSO communicates with Nokia AMS or Tejas EMS to enable the required Ethernet or VEIP interfaces. This ensures consistent service activation, eliminates manual ONT configuration, and provides a standardized provisioning process across all supported GPON platforms.

7. NSO Retry, Conflict Detection, and Provisioning Mechanism

7.1. Overview

Cisco NSO uses an optimistic concurrency model to maximize transaction throughput and enable multiple service provisioning requests to execute in parallel. Instead of locking all data during processing, NSO allows transactions to proceed concurrently and validates data consistency during commit.

When a transaction detects that data previously read has been modified by another transaction, NSO identifies a transaction conflict. Depending on the conflict phase, NSO either performs an automatic retry or rejects the transaction and requires a manual retry.

This mechanism ensures:

- Safe concurrent service provisioning.

- Increased system scalability and throughput.

- Consistent service data in CDB.

- Transactional rollback and recovery capability.

7.2. Transaction Conflict Detection

A transaction conflict occurs when:

- A transaction reads a data element.

- Another transaction modifies the same data before the first transaction commits.

- The first transaction attempts to commit changes.

NSO performs a conflict check immediately before writing data to the Configuration Database (CDB).

Typical Conflict Scenarios

| Transaction Type | Conflict Possible |
| --- | --- |
| Read Only | No |
| Multiple Reads | No |
| Write Only | Rare (last write wins) |

When a conflict occurs, NSO records the event in the developer logs and returns a transaction conflict error.

Typical error:

ERR_TRANSACTION_CONFLICT

Conflict detected between concurrent transactions

7.3. NSO Transaction Commit Workflow

The commit process consists of the following stages:

- Service Mapping (FASTMAP)

- Validation

- Transform Processing

- Global Lock Acquisition

- Conflict Check

- Write to CDB

- Device Configuration Deployment

- Subscriber/Kicker Execution

- Lock Release and Transaction Completion

Before the global lock is acquired, NSO tracks all read and write operations performed during service processing. The conflict checker validates that the accessed data has not changed since it was first read.

7.4. Automatic Retry Mechanism

NSO provides built-in automatic retry functionality for transaction phases fully controlled by NSO.

Automatic Retry Supported

| Phase | Automatic Retry |
| --- | --- |
| Pre-Transform | Yes |
| Transform (FASTMAP) | Yes |
| Validation | Yes |
| Rollback Creation | Yes |

Automatic Retry Not Supported

| Phase | Automatic Retry |
| --- | --- |
| Work Phase (External Logic) | No |

Examples of work-phase operations include:

- External Python scripts

- NETCONF clients

- Custom external integrations

- Third-party provisioning applications

When conflicts occur during these operations, NSO cannot safely rerun the external logic and therefore rejects the transaction.

7.5. Conflict Optimization and Scheduling

NSO automatically tracks recurring service conflicts and optimizes future transaction scheduling.

Benefits include:

- Reduced transaction retries.

- Improved provisioning performance.

- Better concurrency handling.

- Intelligent service execution ordering.

Conflict information can be viewed using:

admin@ncs# unhide debug

admin@ncs# show services scheduling conflict

NSO dynamically learns service conflicts and temporarily prevents conflicting services from executing simultaneously.

7.6. Manual Retry Handling

When a conflict occurs during the work phase, NSO returns a transaction conflict error and the transaction must be restarted.

Manual retry is typically required for:

- Action implementations

- External provisioning scripts

- Subscriber applications

- Custom integration services

For custom applications, Cisco NSO provides built-in retry utilities such as:

Python:

m.run_with_retry()

@retry_on_conflict()

Java:

Maapi.ncsRunWithRetry()

These utilities automatically re-execute the transaction when a conflict is detected.

7.7. Provisioning Impact

During service provisioning, NSO ensures transactional consistency across all participating network devices.

Successful Provisioning:

Order → SCCM → NSO ↓ Configuration Generated ↓ Conflict Check Passed ↓ CDB Commit ↓ Device Provisioning ↓ Success Callback

Provisioning Failure:

Order → SCCM → NSO ↓ Conflict Detected ↓ Retry (if supported) ↓ Failure ↓ Rollback ↓ Error Callback to SCCM

This all-or-nothing transaction model ensures partial configurations are not left on network devices.

8. Additional PON Service Provisioning for Existing GI-BB Customers.

Initialization (setCommonVariables function)

The variable is populated when specific conditions are met:

- Service is GI-BB

- Topology is NOT Hub (meaning it's a Spoke)

- The ref_id starts with 9.5 or 9.4 (specific service reference IDs)

It queries the database for a previous Spoke topology service order for the same endpoint device that has status='3' (active).

Usage Pattern Throughout the Code

The variable is checked in multiple functions to determine if this is an existing customer adding a new order:

- getVPNId() - If committed order exists, reuses VPN ID from previous order instead of generating new one

- getRDValue() - Reuses Route Distinguisher from previous order

- getVRFName() - Reuses VRF name from previous committed order

- getDeviceMode() - Uses device mode from previous order

- getCircuitid() - Reuses circuit ID from previous order

- getnokiafreeontid() - Uses CVLAN/SVLAN values from previous committed order

Core Logic

When !empty($this->committed_spoke_serviceOrder) is true, the code calls:

getPreviousGIBBCommitedOrderVariableValue($parameterName)

This function retrieves the parameter value (vpn-id, rd, VRF-name, etc.) from the previous committed order and reuses it for the new order, instead of generating or allocating new values.

Purpose

This ensures that when an existing GI-BB customer adds a new Spoke service, they maintain consistency by reusing key network parameters from their previous service order rather than creating duplicates or conflicting values.

6.  T-DIA / ILL Service Delivery and Provisioning Workflow

T-DIA ILL is a dedicated internet access service offering for Enterprise and Government Institutions, delivered over T-FIBER's IP/MPLS backbone with GPON and fiber last-mile connectivity.

# 6.1. Network Architecture Overview

The T-FIBER network is a greenfield IP/MPLS based architecture deployed across Telangana under the Bharatnet initiative. The core is built on Cisco routers with Nokia and Tejas GPON as the access layer. End-to-end reachability is achieved using BGP Label Unicast (BGP-LU).

## 6.1.2 Network Hierarchy

| Layer | Node Type | Platform | Role |
| --- | --- | --- | --- |
| Core | Zonal Core Router (ZCR) | Cisco ASR9K | MPLS Core, Route Reflector, ILL HUB, ISP Peering |
| Aggregation | Mandal Router (MAR) | Cisco ASR9K | Mandal Ring aggregation, MPLS P-router |
| Access | GP Router | Cisco ASR920/907 | L3VPN PE Spoke, Customer GW |
| OLT | GPON OLT | Nokia / Tejas | Passive Optical Line Termination |
| Customer | ONT / CPE | Nokia ONT / Customer Router | Last-mile termination at customer premises |

## 6.1.3 T-DIA ILL Topology (HUB & Spoke L3VPN)

The ILL service uses a HUB & Spoke L3VPN topology where:

- Each Enterprise/Government customer gets one L3VPN Spoke on the GP/Mandal router

- A single HUB exists at the Route Reflector ZCR (or one HUB per Zonal Router)

- The ISP upstream link terminates on the ZCR

- Traffic flows: Customer CPE → ONT → OLT → GP Router (Spoke PE) → Mandal Ring → ZCR (HUB) → ISP → Internet

## 6.1.4 Route Targets and Route Distinguishers

| RT/RD | Value | Node | Description |
| --- | --- | --- | --- |
| RT Export (Spoke) | 141688:33 | GP/Mandal Router | Customer routes exported from Spoke PE |
| RT Import (Spoke) | 141688:34 | GP/Mandal Router | Spoke imports default route from HUB |
| RT Export (HUB) | 141688:34 | ZCR | HUB exports default/ISP routes to all Spokes |
| RT Import (HUB) | 141688:33 | ZCR | HUB imports all Spoke customer prefixes |
| RD (Spoke) | Router-ID:XX | GP/Mandal Router | Unique RD per customer VRF instance |
| RD (HUB PoC) | 172.17.236.1:6 | ZCR (ASR9K) | HUB VRF RD used in lab verification |

## 6.1.5 VLAN Plan

| Service Type | S-Tag Range | C-Tag | Remarks |
| --- | --- | --- | --- |
| Enterprise L3VPN (T-DIA ILL) | 3000 – 3200 | N/A | GPON last mile; local to router |
| GI L3VPN (T-DIA ILL) | 3301 – 3499 | N/A | Government Institutions |
| Nokia OLT C-tag (ONT side) | 2000 (pvid) | 2700–2800 | ONT port PVID; swapped to S-tag at NNI |
| Infrastructure VLAN | 2 – 1000 | N/A | Management and infrastructure |
| Customer Tag | N/A | 1001–2699 | Customer VLAN space |
| Reserved | 3500 – 4096 | N/A | Future use |

Note: VLANs are significant only locally to the router (L3VPN) and can be reused across different GP/Mandal/Zonal routers if they are free on that specific router.

# 6.2. End-to-End Provisioning Flow

The T-DIA ILL service provisioning is fully automated via the SCCM + NSO stack. The flow covers seven stages from customer order intake to service verification.

## Phase 1: Order Intake & Feasibility (BSS/OSS)

| Step | System | Action |
| --- | --- | --- |
| 1 | Customer / BSS | Enterprise or Government customer raises an ILL service request. Bandwidth tier, last-mile type, CPE type, and public IP requirement are specified. |
| 2 | BSS | BSS receives and validates the order. BSS holds the billing system and communicates to SCCM-NSO via North Bound REST APIs. |
| 3 | OSS | OSS performs feasibility check using customer Lat/Long coordinates against device inventory. OSS maintains device inventory and service inventory for all network elements. |
| 4 | BSS → SCCM | Validated order handed to SCCM via North Bound API. OSS also connects to SCCM with device/inventory data. |

## Phase 2: SCCM Orchestration

SCCM (Service Configuration and Compliance Manager) is the middleware between OSS/BSS and NSO. It enforces service logic, input validation, topology discovery, and southbound API to NSO.

### 2a. Input Variable Assembly

SCCM combines User Defined (UD) parameters (provided by the operator) with System Defined (SD) parameters that SCCM derives automatically:

| Variable | UD/SD | Value / Description |
| --- | --- | --- |
| VRF-name | SD | TDIA-ILL |
| rt-export / rt-import | SD | 141688:33 / 141688:34 |
| internet-vlan (S-tag) | SD | 3000–3200 (Enterprise) ¦ 3301–3499 (GI) |
| BDI interface number | SD | Auto-assigned (e.g. BDI3302) |
| ipv4-wan-address | SD | /30 or /29+ from TFiber public pool |
| routing-protocol | SD/UD | connected ¦ static ¦ eBGP |
| qos-name | SD | TDIA-ILL-IN / TDIA-ILL-OUT |
| mtu | SD | 9100 (IOS-XE) ¦ 9216 (IOS-XR) |
| ont-serial-number | UD | Provided by operator during provisioning |
| ds-bandwidth-profile | UD | Nokia: UP_100M ¦ Tejas: 102400 kbps |
| cust-as-no / remote-as-no | UD | Required for eBGP PE-CE scenario only |
| bfd-required | UD | Optional – bidirectional forwarding detection |

### 2b. Topology Discovery & Service Package Selection

SCCM identifies the full end-to-end topology based on the endpoint device, then selects the appropriate NSO service package:

| NSO Service Package | Scenario |
| --- | --- |
| TDIA-DIRECT-SINGLE-ASR920 | Fiber last mile directly on ASR920 (single port) |
| TDIA-DIRECT-SINGLE-ASR907 | Fiber last mile directly on ASR907 (single port) |
| TDIA-DIRECT-SINGLE-ASR9K | Fiber last mile directly on ASR9K (single port) |
| TDIA-NOKIA-SINGLE-ASR920 | Nokia GPON OLT + ASR920 GP Router |
| TDIA-NOKIA-SINGLE-ASR907 | Nokia GPON OLT + ASR907 GP Router |
| TDIA-NOKIA-SINGLE-ASR9K | Nokia GPON OLT + ASR9K MAG/ZCR |
| TDIA-TEJAS-SINGLE-ASR920 | Tejas GPON OLT + ASR920 GP Router |
| TDIA-TEJAS-SINGLE-ASR907 | Tejas GPON OLT + ASR907 GP Router |
| TDIA-TEJAS-SINGLE-ASR9K | Tejas GPON OLT + ASR9K MAG/ZCR |
| TDIA-DIRECT-ASR920-MULTIPORT | Dual fiber LAG on ASR920 (redundant last mile) |
| TDIA-DIRECT-ASR907-MULTIPORT | Dual fiber LAG on ASR907 (redundant last mile) |
| TDIA-DIRECT-ASR9K-MULTIPORT | Dual fiber LAG on ASR9K (redundant last mile) |

### 2c. SCCM → NSO API & Provisioning Flow

- SCCM constructs the NSO service payload and sends via RESTCONF (ports 8080/8888) or CLI (port 4569)

- NSO checks CDB synchronisation with devices – if out of sync, performs sync-from first

- NSO computes device-specific configs and pushes atomically (all-or-nothing transaction)

- Flow A (Success): NSO confirms push → SCCM triggers Playbook post-check → verification PDF generated → BSS notified

- Flow B (Failure): If any device fails, NSO rolls back all changes across all devices → SCCM reports failure to user

Important – Out-of-Band Configuration Warning Any configuration applied directly to devices via CLI (outside NSO) is NOT visible to NSO's service model until explicitly reconciled. If NSO's CDB and device configuration differ, dry-run outputs during subsequent MACD operations will not be as expected.

# 6.3. NSO Solution Design

## 6.3.1 NSO Architecture

Cisco NSO (Network Service Orchestrator) has two main integrated layers:

- Service Manager: Manages high-level service definitions. Uses YANG service model defining T-DIA ILL attributes. Service-to-device mapping is implemented via Python/Java templates.

- Device Manager: Manages device configurations transactionally. Uses Network Element Drivers (NEDs) modelled in YANG for each device type. NEDs support CLI, NETCONF, and REST-based device interfaces.

## 6.3.2 NSO Connectivity Details

| System | VIP | Active / Standby | Ports |
| --- | --- | --- | --- |
| NSO (Primary) | 172.28.2.150 | 172.28.2.109 / 172.28.2.118 | 4569 (CLI), 4570 (HA), 8080 (RESTCONF), 8888 (SSL), 2022-23 |
| NSO (Secondary) | 172.28.2.133 | 172.28.2.110 / 172.28.2.119 | Same as above |
| NSO (Tertiary) | 172.28.105.120 | 172.28.2.111 / 172.28.2.120 | Same as above |
| Nokia AMS PKG-A | 172.28.39.11 / .12 | – | 8443 (HTTPS) |
| Nokia AMS PKG-B | 172.28.43.71 / .73 | – | 8443 (HTTPS) |
| Nokia AMS PKG-C | 172.28.43.101 / .103 | – | 8443 (HTTPS) |
| Tejas EMS PKG-A | 172.28.7.85 | – | 446 (HTTPS) |
| Tejas EMS PKG-B | 172.28.12.215 | – | 446 (HTTPS) |
| Tejas EMS PKG-C | 172.28.13.215 | – | 446 (HTTPS) |

| System | Hostname | IP Address | Role | OS Version |
| --- | --- | --- | --- | --- |
| NSO CFS Leader | cfsleader | 172.28.2.17 | CFS Leader | Ubuntu 24.04.4 LTS |
| NSO RFS Leader-1 | rfsleader1 | 172.28.2.18 | RFS Leader | Ubuntu 24.04.4 LTS |
| NSO RFS Leader-2 | rfsleader2 | 172.28.2.19 | RFS Leader | Ubuntu 24.04.4 LTS |
| NSO RFS Leader-3 | rfsleader3 | 172.28.2.20 | RFS Leader | Ubuntu 24.04.4 LTS |
| Legacy NSO CFS | CFS | 172.28.2.150 | CFS Node | Ubuntu 20.04.2 LTS |
| Legacy NSO RFS Primary | TFDCCISNSRFS1 | 172.28.2.110 / 172.28.2.133 | RFS Primary | Ubuntu 20.04.4 LTS |
| Legacy NSO RFS Standby | RFS1-STBY | 172.28.2.119 | RFS Standby | Ubuntu 20.04.2 LTS |
| Nokia AMS PKG-A | AMS Cluster-A | 172.28.39.11 / 172.28.39.12 | Nokia EMS | HTTPS |
| Nokia AMS PKG-B | AMS Cluster-B | 172.28.43.71 / 172.28.43.73 | Nokia EMS | HTTPS |
| Nokia AMS PKG-C | AMS Cluster-C | 172.28.43.101 / 172.28.43.103 | Nokia EMS | HTTPS |
| Tejas EMS PKG-A | Tejas EMS-A | 172.28.7.85 | Tejas OLT EMS | HTTPS |
| Tejas EMS PKG-B | Tejas EMS-B | 172.28.12.215 | Tejas OLT EMS | HTTPS |
| Tejas EMS PKG-C | Tejas EMS-C | 172.28.13.215 | Tejas OLT EMS | HTTPS |

| Environment | Nodes | CPU | RAM | Storage |
| --- | --- | --- | --- | --- |
| Current NSO Cluster | 4 VMs (1 CFS + 3 RFS) | 56 vCPU | 115 GB | 682 GB |
| Legacy NSO Cluster | 3 VMs (1 CFS + 2 RFS) | 24 vCPU | 112 GB | 195 GB |

Legacy NSO Infrastructure – Virtual Machine Specifications

| VM | vCPU | RAM | Storage |
| --- | --- | --- | --- |
| CFS (172.28.2.150) | 8 | 48 GB | 49 GB |
| TFDCCISNSRFS1 (RFS Primary) | 8 | 32 GB | 97 GB |
| RFS1-STBY (RFS Standby) | 8 | 32 GB | 49 GB |

Current NSO Infrastructure – Virtual Machine Specifications

| VM | vCPU | RAM | Storage |
| --- | --- | --- | --- |
| cfsleader | 8 | 16 GB | 97 GB |
| rfsleader1 | 16 | 33 GB | 195 GB |
| rfsleader2 | 16 | 33 GB | 195 GB |
| rfsleader3 | 16 | 33 GB | 195 GB |

## 6.3.3 NSO Configuration Pushed – Cisco Router (IOS-XR HUB / ZCR)

### Configured Components

- VRF: TDIA-ILL

- Route Targets:

  - Import: 141688:33

  - Export: 141688:34

- WAN/Uplink Interface connected to ISP

- eBGP session established with upstream ISP

- Route policies applied for inbound and outbound route filtering

- QoS policy applied on WAN interface

- Public TFiber prefixes advertised to ISP

Key Functions

- Provides Internet gateway for TDIA/ILL services.

- Exchanges routes with upstream ISP through eBGP.

- Applies QoS and route filtering policies.

## 6.3.4 NSO Configuration Pushed – Cisco Router (IOS-XE Spoke / GP Router)

Configured Components

- VRF: TDIA-ILL

- Service VLAN: 3302

- Bridge Domain: 3302

- BDI Interface created for Layer-3 termination

- Customer public IP assigned on BDI interface

- BGP redistribution of connected and static routes

- QoS policy applied

Key Functions

- Terminates customer service VLAN.

- Provides Layer-3 connectivity between GPON and MPLS core.

- Advertises customer routes to the network.

# 6.4. GPON Last-Mile Configuration

## 6.4.1 Nokia OLT Configuration

Configured Components

- Ingress and egress QoS profiles

- Bandwidth profiles for customer speed plan

- Service VLAN 3302 creation

- Bridge configuration between customer UNI and uplink

- ONT UNI port activation

- Uplink mapping towards aggregation network

Key Functions

- Delivers last-mile GPON connectivity.

- Applies customer bandwidth and QoS policies.

- Maps customer traffic to service VLAN.

## 6.4.2 Chassis OLT (VPLS Configuration)

Configured Components

- VPLS Service: 3302

- Uplink SAP configuration

- Access SAP configuration

- Service activation (No Shutdown)

Key Functions

- Extends customer Layer-2 service through VPLS.

- Connects GPON access network with MPLS transport network.

## 6.4.2 Tejas OLT Configuration

### Configured Components

- Service Creation through Tejas EMS

- Service Type: ELAN

- Service VLAN (SVLAN): 3302

- Customer VLAN (CVLAN): 2700-2800

- GPON Service Profile Assignment

- VLAN Translation (CVLAN to SVLAN)

- Bandwidth Profile Configuration (100 Mbps)

Key Functions

- Creates Layer-2 service on the GPON network.

- Maps customer traffic to the service VLAN.

- Applies bandwidth and QoS policies.

- Connects customer access network to the MPLS transport network.

# 6.5. PE-CE Routing Scenarios

NSO supports multiple routing models based on customer requirements and last-mile connectivity.

| Scenario | Deployment Model |
| --- | --- |
| Scenario 1 | GP/Mandal Router acts as Customer Gateway |
| Scenario 2 | Static Route between PE and Customer Router |
| Scenario 3 | WiFi ONT in Router Mode (NAT Gateway) |
| Scenario 4 | External WiFi Router as Gateway |
| Scenario 5 | Customer-Owned Fiber CPE with QoS Enforcement |
| Scenario 6 | Customer eBGP Peering with TFiber |

Key Functions

- Supports both Static and BGP routing.

- Supports customer-owned and TFiber-provided gateways.

- Provides flexibility for different enterprise deployment requirements.

# 6.6. IP Address Management

## 6.6.1 Public IP Allocation

NSO/SCCM manages all IP address allocation for T-DIA ILL services. TFiber's public IP block 103.162.4.0/23 is aggregated at the ZCR HUB and announced to the upstream ISP via eBGP.

| Allocation Type | Subnet Size | Description |
| --- | --- | --- |
| WAN Link (P2P) | /30 | 2 usable IPs: PE BDI + Customer CPE/ONT WAN |
| Customer LAN (small) | /29 | 6 usable IPs; GW on PE BDI, rest for customer devices |
| Customer LAN (larger) | /28 or larger | Enterprise customers needing more public IPs |
| TFiber Public Pool | 103.162.4.0/23 | Aggregated and announced to ISP from ZCR HUB |
| PoC Test Subnet | 103.162.5.252/30 | Used for ILL PoC verification in test lab |

Key Functions

- SCCM/NSO manages IP allocation automatically.

- Public IP pools are advertised through the Internet gateway.

- Supports both WAN and customer LAN addressing requirements.

## 6.6.2 Internal IP Ranges (SCCM/NSO Infrastructure)

| Pool | Purpose |
| --- | --- |
| 172.22.0.0/16, 172.23.0.0/16 | External service IPs |
| 172.24.0.0/16, 172.25.0.0/16 | Internal service IPs |
| 172.28.0.0/16 | NSO, SCCM, OLT management IPs (in-band) |

# 6.7. QoS Design

## 6.7.1 QoS Overview

All T-DIA/ILL traffic is classified into the Data Traffic Class and is maintained consistently across the access, aggregation, and MPLS core networks.

## QoS Policy Components

| Component | Purpose |
| --- | --- |
| Traffic Classification | Classifies T-DIA/ILL Data Traffic |
| QoS Marking | Assigns QoS Group and MPLS EXP Values |
| Bandwidth Enforcement | Ensures Customer CIR Compliance |
| Traffic Prioritization | Maintains End-to-End Service Quality |

## Bandwidth Enforcement

| Last-Mile Type | Enforcement Point | Method |
| --- | --- | --- |
| GPON (Nokia/Tejas) | OLT | Bandwidth Profile (CIR/PIR) |
| Fiber (Single/Dual LAG) | GP/Mandal Router | Policing and Traffic Shaping |

Key Functions

- Guarantees subscribed customer bandwidth.

- Prevents bandwidth overutilization.

- Maintains QoS consistency across the network

# 6.8. Service Verification Procedures

After service provisioning is completed, post-checks are performed to validate end-to-end T-DIA/ILL service availability.

| Verification Step | Validation Objective |
| --- | --- |
| HUB Route Verification | Confirm default route, public IP advertisement, and MPLS reachability. |
| Spoke Route Verification | Confirm VRF routing, default gateway, and service subnet reachability. |
| ARP Verification | Verify Layer-2 and Layer-3 neighbor resolution. |
| P2P Connectivity Test | Validate connectivity between PE and customer endpoint. |
| Internet Reachability Test | Verify access to public internet through the ISP gateway. |
| Bandwidth Validation | Confirm subscribed bandwidth and QoS policy enforcement. |
| Public Reachability Validation | Verify customer public IP advertisement and global internet accessibility. |

Key Validation Checks

- Routing information successfully propagated across the MPLS network.

- Customer service subnet reachable from the PE router.

- Public internet access available via the upstream ISP.

- Configured bandwidth profile enforced correctly.

- Customer public IP addresses reachable from external networks.

- End-to-end service activation successfully completed.

# 6.9. SCCM & NSO System Requirements

| Component | Description | VMs | vCPU | RAM (GB) | Storage (GB) | Version |
| --- | --- | --- | --- | --- | --- | --- |
| SCCM | Worker (PHP/Apache) | 2 | 16 | 64 | 300 | 8.0.91 / 2.4.52 |
| SCCM | RabbitMQ | 3 | 16 | 64 | 300 | 3.9.12 |
| SCCM | Web Server (PHP/Apache) | 2 | 16 | 64 | 300 | SCCM v4.1.1 |
| SCCM | DB (MySQL) | 2 | 16 | 64 | 1500 | 8.0.27 |
| SCCM | Discovery | 1 | 16 | 32 | 300 | v1.0.0 |
| SCCM | Playbook | 1 | 16 | 32 | 300 | v1.1.1 |
| SCCM | Load Balancer (HAProxy) | 2 | 16 | 32 | 300 | 2.2.9 |
| NSO | Current NSO Cluster (1 CFS + 3 RFS) | 4 | 56 | 115 | 682 | NSO 6.4 |
| NSO | Legacy NSO Cluster (1 CFS + 1 RFS Primary + 1 RFS Standby) | 3 | 24 | 112 | 195 | NSO 6.4 |

Host OS (Hypervisor): VMware  |  Guest OS (VMs): Ubuntu 24.04.4 LTS

## Current NSO Infrastructure

| VM | vCPU | RAM | Storage |
| --- | --- | --- | --- |
| cfsleader | 8 | 16 GB | 97 GB |
| rfsleader1 | 16 | 33 GB | 195 GB |
| rfsleader2 | 16 | 33 GB | 195 GB |
| rfsleader3 | 16 | 33 GB | 195 GB |
| Total | 56 vCPU | 115 GB | 682 GB |

## Legacy NSO Infrastructure

| VM | vCPU | RAM | Storage |
| --- | --- | --- | --- |
| CFS (172.28.2.150) | 8 | 48 GB | 49 GB |
| TFDCCISNSRFS1 (RFS Primary) | 8 | 32 GB | 97 GB |
| RFS1-STBY (RFS Standby) | 8 | 32 GB | 49 GB |
| Total | 24 vCPU | 112 GB | 195 GB |

## Infrastructure Summary

| Environment | Nodes | CPU | RAM | Storage |
| --- | --- | --- | --- | --- |
| Current NSO Cluster | 4 VMs (1 CFS + 3 RFS) | 56 vCPU | 115 GB | 682 GB |
| Legacy NSO Cluster | 3 VMs (1 CFS + 2 RFS) | 24 vCPU | 112 GB | 195 GB |