LLD Document

Cisco Network Services Orchestrator (NSO)

| Prepared by | Reviewed By | Approved by |
| --- | --- | --- |

Table of Contents

1. TCell P2P (P2MP covered — TCell P2P product to be included)	3

2. ONT LAN Ports unshut while configuration done based on number of services & requirement	32

3. NSO VLAN Assignment Models	36

4. IP Pool Assignment Models	38

5. Traffic priority for QoS in GPON not covered	40

6. GI ILL (configuration concept available but not covered as product Automation list)	54

7. NSO Transaction Retry and Provisioning Mechanism	69

8. Additionally Second configuration of PON port not working to be resolved 
while already existing customer is in use, We cant go live without this option. 
This was working when Cisco has done the Demo now its not working.	75

1.  TCell P2P — L2 VPN Point to Point (P2P)

For point-to-point cellular connectivity, the following scenarios are defined:

Scenario 1 — DCWAN/ZCR (Headend or Site A) and MAG/GP (Spoke or Site B)
Scenario 2 — MAG/GP (Headend or Site A) and MAG/GP (Spoke or Site B)

Under Scenario 2, three possible sub-cases exist:

- MAG as Site A and GP as Site B

- MAG as Site A and another MAG as Site B

- GP as Site A and another GP as Site B

NOTE: Considering BGP-LU, end-to-end L2VPN can be created between other ring MAG & GP nodes.

Both scenarios are supported with two transport options:

- Service Configuration without BGP-LU Transport

- Service Configuration with BGP-LU Transport

Configuration Details

Scenario 1 — DCWAN/ZCR (Site A) and MAG/GP (Site B)

DCWAN/ZCR Configuration — Ingress Interface

interface Bundle-Ether<As per bundle number>.<As per VLAN-ID> l2transport

service-policy input <As per policy-map_IN>

service-policy output <As per policy-map_OUT>

description <As per naming format>

encapsulation dot1q <As per customer VLAN>

!

DCWAN/ZCR Configuration — L2VPN

L2VPN

!

bridge group <As per bridge group name>

bridge-domain <As per Bridge domain ID>

mtu 9100

interface Bundle-Ether <As per bundle number>.<As per VLAN-ID>

!

vfi <As per VFI name>

neighbour <As per Neighbor IP> <As per pseudowire ID> ! Add Mandal/GP Router MAG/GP Loopback 0

pw-class <As per name of the pseudowire class>

!

MAG/GP Configuration — Egress Interface (UNI)

interface Gix/x/x ! UNI Interface

service-policy input <As per policy-map_IN>

service-policy output <As per policy-map_OUT>

service instance <As per service instance ID> ethernet

encapsulation dot1q <As per customer VLAN IDs>

rewrite ingress tag push dot1q <As per Bridge domain ID> symmetric

bridge-domain <As per Bridge domain ID>

!

MAG/GP Configuration — L2VPN

l2 vfi <As per VFI name> manual

vpn id <As per VPN-ID>

mtu 9100

bridge-domain <As per Bridge domain ID>

neighbor <As per Neighbor IP> encapsulation mpls ! Add Headend Router DC-WAN/ZCR Loopback 0

!

NOTE: The UNI interface may differ — it can be on a port-channel, in which case the port-channel UNI deployment configurations apply with Active/Standby LACP.

Scenario 2 — MAG/GP (Site A) and MAG/GP (Site B)

MAG Configuration (Site A) — Ingress Interface

interface Gix/x/x ! UNI Interface

service-policy input <As per policy-map_IN>

service-policy output <As per policy-map_OUT>

service instance <As per service instance ID> ethernet

encapsulation dot1q <As per VLAN-ID list>

rewrite ingress tag push dot1q <As per Bridge domain ID> symmetric

bridge-domain <As per Bridge domain ID>

!

MAG Configuration — L2VPN

l2 vfi <As per VFI name> manual

vpn id <As per VPN-ID>

mtu 9100

bridge-domain <As per Bridge domain ID>

neighbor <As per Neighbor IP> encapsulation mpls ! Add Headend Router DC-WAN/ZCR Loopback 0

!

GP Configuration (Site B) — Ingress Interface

interface Gix/x/x ! UNI Interface

service-policy input <As per policy-map_IN>

service-policy output <As per policy-map_OUT>

service instance <As per service instance ID> ethernet

encapsulation dot1q <As per VLAN-ID list>

rewrite ingress tag push dot1q <As per Bridge domain ID> symmetric

bridge-domain <As per Bridge domain ID>

!

GP Configuration — L2VPN

l2 vfi <As per VFI name> manual

vpn id <As per VPN-ID>

mtu 9100

bridge-domain <As per Bridge domain ID>

neighbor <As per Neighbor IP> encapsulation mpls ! Add Headend Router DC-WAN/ZCR Loopback 0

!

NOTE: The UNI interface may differ — it can be on a port-channel. Refer to port-channel UNI deployments with SDM(Switch Database Management) commands and Active/Standby LACP configurations.

Additional P2P Scenarios — Routers Not Yet Integrated with MPLS (Managed via TSWAN)

Where Mandal/GP routers are not yet integrated into the MPLS network, the following additional P2P scenarios apply. Router management is via TSWAN network until MPLS integration is complete.

Additional Scenario 1 — Both Sites on Same Mandal Router

- Cell Tower Site-1 connected to Mandal router on fiber last mile (single or dual), Port-Channel A configured with 1 or 2 links.

- Cell Tower Site-2 also connected to the same Mandal router on fiber last mile, Port-Channel B configured with 1 or 2 links.

- Port-Channel A and Port-Channel B are configured under the same bridge domain to establish direct P2P connectivity between Site-1 and Site-2.

- CIR and QoS configured as per requirement and subscription.

Mandal Router Configuration

interface Gi<x/x/x>

description <As per site description>

no ip address

negotiation auto

service-policy input <As per policy-map_IN>

service-policy output <As per policy-map_OUT>

service instance <As per service instance ID> ethernet

description <As per site description>

encapsulation dot1q <As per VLAN-ID list>

bridge-domain <As per Bridge domain ID>

interface Gi<x/x/x>

description <As per site description>

no ip address

negotiation auto

service-policy input <As per policy-map_IN>

service-policy output <As per policy-map_OUT>

service instance <As per service instance ID> ethernet

description <As per site description>

encapsulation dot1q <As per VLAN-ID list>

bridge-domain <As per Bridge domain ID>

Additional Scenario 2 — Site-1 on Mandal Router, Site-2 on GP Router (Not in MPLS)

- Mandal and GP routers are not integrated with MPLS network, but connectivity between them is ready.

- Cell Tower Site-1 connected to Mandal router on fiber last mile, Port-Channel A with 1 or 2 links.

- Cell Tower Site-2 connected to GP router on fiber last mile, Port-Channel A with 1 or 2 links.

- QinQ configured on router ports facing Cell Tower.

- L2VPN configured between Mandal and GP routers to establish P2P connectivity.

- CIR and QoS configured on the LM port-channel per requirement.

Mandal Router Configuration

interface Gi<x/x/x>

description <As per site description>

no ip address

negotiation auto

service-policy input <As per policy-map_IN>

service-policy output <As per policy-map_OUT>

service instance <As per service instance ID> ethernet

description <As per site description>

encapsulation dot1q <As per VLAN-ID list>

rewrite ingress tag push dot1q <As per Bridge domain ID> symmetric

bridge-domain <As per Bridge domain ID>

! L2VPN:

l2 vfi <As per VFI name> manual

vpn id <As per VPN-ID>

mtu 9100

bridge-domain <As per Bridge domain ID>

neighbor <As per Neighbor IP> encapsulation mpls ! L2VPN to GP router

GP Router Configuration

interface Gi<x/x/x>

description <As per site description>

no ip address

negotiation auto

service-policy input <As per policy-map_IN>

service-policy output <As per policy-map_OUT>

service instance <As per service instance ID> ethernet

description <As per site description>

encapsulation dot1q <As per VLAN-ID list>

rewrite ingress tag push dot1q <As per Bridge domain ID> symmetric

bridge-domain <As per Bridge domain ID>

! L2VPN:

l2 vfi <As per VFI name> manual

vpn id <As per VPN-ID>

mtu 9100

bridge-domain <As per Bridge domain ID>

neighbor <As per Neighbor IP> encapsulation mpls ! L2VPN to Mandal router

Troubleshooting Commands — P2P (Spoke/MAG/GP Routers)

| Command | Purpose |
| --- | --- |
| show run ¦ s l2 vfi | Check and verify the L2 VFI configuration on MAG & GP router |
| show mpls l2transport vc vcid <Number> | Check and verify the L2VPN status on MAG & GP |
| show l2vpn vfi name <name> detail | Check and verify the L2VPN status on MAG & GP |
| show bridge-domain <Number> | Check and validate the MAC addresses |
| show policy-map interface <interface-id> {input/output} | Validate the QoS policy status on MAG & GP |

QoS Policy — TCell L2 VPN Point to Point (P2P)

Traffic Class Definition

| Traffic Class | CoS Match | CIR % | Egress Priority | Purpose |
| --- | --- | --- | --- | --- |
| Signalling (SIGN) | CoS 7 | 5% | Priority Level 1 — Strict | Control / Signalling |
| Circuit Switched (CS) | CoS 6 | 10% | Priority Level 2 — Strict | Voice / CS Bearer |
| OAM | CoS 5 | 5% | Bandwidth (best-effort) | Operations & Mgmt |
| Data | CoS 4 | 80% | Bandwidth (best-effort) | User Data / Internet |

NOTE: Signalling and CS use Strict Priority — always served first regardless of queue depth. OAM and Data use bandwidth queuing with queue-limit. Aggregate CIR on MAG/GP is 200 Mbps (parent police).

SDC Router — QoS Policy

Classifies CoS bits into internal qos-group values. No CIR policing at SDC — classification and remarking only.

| Ingress — Child Policy Map | Egress — Policy Map |
| --- | --- |
| class-map match-all AIRTEL_CELL_SIGN_IN match cos 7 ! class-map match-all AIRTEL_CELL_CS_IN match cos 6 ! class-map match-all AIRTEL_CELL_OAM_IN match cos 5 ! class-map match-all AIRTEL_CELL_DATA_IN match cos 4 ! policy-map AIRTELCELL_CHILD-IN class AIRTEL_CELL_SIGN_IN set qos-group 7 ! class AIRTEL_CELL_CS_IN set qos-group 6 ! class AIRTEL_CELL_OAM_IN set qos-group 2 ! class AIRTEL_CELL_DATA_IN set qos-group 1 ! Policy-Map AIRTELCELL_PARENT-IN class class-default service-policy AIRTELCELL_CHILD-IN ! | policy-map AIRTELCELL-OUT class AIRTEL_CELL_SIGN_OUT ! qos-group 7 set cos 5 ! class AIRTEL_CELL_CS_OUT ! qos-group 6 set cos 6 ! class AIRTEL_CELL_OAM_OUT ! qos-group 2 set cos 5 ! class AIRTEL_CELL_DATA_OUT ! qos-group 1 set cos 4 ! |

DCWAN / ZCR Router (HUB) — QoS Policy

Classifies CoS, marks MPLS EXP bits for core transport, and applies strict priority queuing at egress toward SDC.

| Ingress — Child Policy Map (with MPLS EXP) | Egress — Priority Queuing |
| --- | --- |
| policy-map AIRTELCELL_CHILD-IN class AIRTEL_CELL_SIGN_IN set qos-group 5 set mpls experimental imposition 5 ! class AIRTEL_CELL_CS_IN set qos-group 6 set mpls experimental imposition 6 ! class AIRTEL_CELL_OAM_IN set qos-group 2 set mpls experimental imposition 2 ! class AIRTEL_CELL_DATA_IN set qos-group 1 set mpls experimental imposition 1 ! class class-default set qos-group 0 ! Policy-Map AIRTELCELL_PARENT-IN class class-default service-policy AIRTELCELL_CHILD-IN ! | policy-map AIRTELCELL-OUT class AIRTEL_CELL_SIGN_OUT set cos 7 priority level 1 ! class AIRTEL_CELL_CS_OUT set cos 6 priority level 2 ! class AIRTEL_CELL_OAM_OUT set cos 5 ! class AIRTEL_CELL_DATA_OUT set cos 4 ! ! Apply to interface: interface Bundle-Ether15.2700 l2transport service-policy input AIRTELCELL_PARENT-IN service-policy output AIRTELCELL-OUT ! |

MAG — Mandal Router (Spoke, ASR907) — QoS Policy

CIR policing per class at ingress. Parent policy enforces 200 Mbps aggregate. Egress applies hierarchical shaping (200 Mbps) with priority queuing.

| Ingress — CIR Policing + MPLS EXP | Egress — Priority + BW + 200 Mbps Shaper |
| --- | --- |
| policy-map AIRTELCELL_CHILD-IN class AIRTEL_CELL_SIGN_IN police cir percent 5 set qos-group 5 set mpls experimental imposition 5 ! class AIRTEL_CELL_CS_IN police cir percent 10 set qos-group 6 set mpls experimental imposition 6 ! class AIRTEL_CELL_OAM_IN police cir percent 5 set qos-group 2 set mpls experimental imposition 2 ! class AIRTEL_CELL_DATA_IN police cir percent 80 set qos-group 1 set mpls experimental imposition 1 ! class class-default set qos-group 0 set mpls experimental imposition 0 ! Policy-Map AIRTELCELL_PARENT-IN class class-default police cir 200000000 bc 4161500 conform-action transmit exceed-action drop service-policy AIRTELCELL_CHILD-IN ! | policy-map AIRTELCELL-OUT class AIRTEL_CELL_SIGN_OUT priority level 1 percent 5 set cos 7 ! class AIRTEL_CELL_CS_OUT priority level 2 percent 10 set cos 6 ! class AIRTEL_CELL_OAM_OUT bandwidth percent 5 set cos 4 ! class AIRTEL_CELL_DATA_OUT bandwidth percent 80 set cos 3 queue-limit 4000000 bytes ! class class-default set cos 0 ! Policy-Map AIRTELCELL_PARENT-OUT class class-default shape average 200000000 4000000 service-policy AIRTELCELL-OUT ! |

ASR907 — Egress QoS on Port-Channel (SDM Template — One-Time)

! Step 1 — Enable SDM template (box will reload after this) sdm prefer enable_portchannel_qos_multiple_active reload location all ! Step 2 — Post-reload: enable platform CLI BEFORE creating port-channel platform qos-port-channel_multiple_active <port-channel#> ! Step 3 — Apply policy on port-channel interface Port-channel <Number> service-policy input AIRTELCELL_PARENT-IN service-policy output AIRTELCELL_PARENT-OUT

GP — Gram Panchayat Router (Spoke, ASR920) — QoS Policy

Ingress is identical to MAG. Egress differs — ASR920 does NOT support egress shaping on port-channel. Apply egress on member links; ingress on port-channel service instance.

| Egress — Priority + BW + 200 Mbps Shaper | ASR920 — Apply Points (Port-Channel Limitation) |
| --- | --- |
| policy-map AIRTELCELL-OUT class AIRTEL_CELL_SIGN_OUT ! qos-group 5 priority level 1 percent 5 set cos 7 ! class AIRTEL_CELL_CS_OUT ! qos-group 6 priority level 2 percent 10 set cos 6 ! class AIRTEL_CELL_OAM_OUT ! qos-group 2 bandwidth percent 5 set cos 4 ! class AIRTEL_CELL_DATA_OUT ! qos-group 1 bandwidth percent 80 queue-limit 2000000 bytes set cos 3 ! class class-default set cos 0 ! Policy-Map AIRTELCELL_PARENT-OUT class class-default shape average 200000000 2000000 service-policy AIRTELCELL-OUT ! | ! Egress — on MEMBER LINKS only (NOT port-channel) interface <member-link-1> service-policy input AIRTELCELL_PARENT-IN service-policy output AIRTELCELL_PARENT-OUT ! interface <member-link-2> service-policy input AIRTELCELL_PARENT-IN service-policy output AIRTELCELL_PARENT-OUT ! ! Ingress — on port-channel SERVICE INSTANCE interface <port-channel> service instance <XX> ethernet service-policy input AIRTELCELL_PARENT-IN service-policy output AIRTELCELL_PARENT-OUT bridge-domain 2700 ! ! LACP Active/Standby interface <port-channel> lacp fast-switchover lacp max-bundle 1 ! |

QoS Summary — All Routers

| Router | Ingress Policing | MPLS EXP Mark | Egress Queuing | Aggregate Shape |
| --- | --- | --- | --- | --- |
| SDC | No — CoS classify only | No | CoS remarking only | None |
| DCWAN/ZCR (HUB) | No — CoS classify only | EXP 1/2/5/6 | Priority L1/L2 + BW | None |
| MAG — ASR907 | Yes — CIR% per class | EXP 1/2/5/6 | Priority L1/L2 + BW (burst 4M) | 200 Mbps |
| GP — ASR920 | Yes — CIR% per class | EXP 1/2/5/6 | Priority L1/L2 + BW (burst 2M) | 200 Mbps |

ASR907 vs ASR920 — Key QoS Difference

| Feature | ASR907 (MAG) | ASR920 (GP) |
| --- | --- | --- |
| Egress QoS on Port-Channel | Supported — SDM template + platform CLI | NOT supported — apply on member links |
| SDM Template / Box Reload | Required — box reloads | Not required |
| Ingress QoS Apply Point | Port-channel interface | Port-channel service instance |
| Egress Burst Size | shape average 200000000 4000000 | shape average 200000000 2000000 |

IMPORTANT: For ASR907: The SDM template (sdm prefer enable_portchannel_qos_multiple_active) is a one-time operation that requires a box reload. The platform qos-port-channel_multiple_active command must be applied after reload and BEFORE creating the port-channel.

SERVICE PROVISIONING

End-to-End Service Provisioning — BSS | OSS | SCCM | NSO | Network

This document provides a complete overview of the automated service provisioning system, describes the end-to-end provisioning flow, explains the role of each system involved, and highlights the exact API calls that enable seamless integration.
Service connects government institutions — schools, Gram Panchayats, health centres — to high-speed broadband internet using T-FIBER's optical fiber network. The entire provisioning process, from accepting a customer order to activating the service on the network, is automated through a chain of four interconnected software systems.

Four Systems in the Chain

| System | Full Name | Plain-English Role |
| --- | --- | --- |
| BSS | Business Support System | The customer-facing portal. A government institution places a broadband order here. BSS validates the customer, creates the order, and triggers the provisioning chain. It also receives a final notification when the service is live. |
| OSS | Operations Support System | The network inventory system. OSS keeps track of every device, service, and circuit in the T-FIBER network. It queries SCCM to maintain an up-to-date picture of all active services — used for fault management, audits, and capacity planning. |
| SCCM | Service & Config Control Manager | The orchestration middleware. SCCM sits between BSS and the network. It receives orders from BSS, figures out which devices need configuration, builds the service, sends it to NSO, and notifies BSS when done. It also exposes inventory APIs to OSS. |
| NSO | Network Services Orchestrator (Cisco) | The network configuration engine. NSO receives service templates from SCCM and pushes the actual router and OLT configurations. It is the only system that directly touches network devices. If a push fails on any device, NSO rolls back all changes automatically. |

NOTE: T-FIBER's network is built on IP/MPLS core (Cisco ASR routers) with GPON access (Nokia and Tejas OLTs). GI-BB is delivered as an L2VPN Hub & Spoke service over this infrastructure.

BSS Flowchart diagram

1.2.  Device Inventory

1.2.1  Physical Network Devices

- The following hardware devices are part of the T-FIBER Service provisioning environment

| Device Type | Hostname | Management IP | Platform | Role in GI-BB |
| --- | --- | --- | --- | --- |
| GP Router | NOCTESTGAC001 | 172.17.128.1 | Cisco ASR 920 | Spoke — GP Level PE |
| Mandal Router | NOCTESTMAR001 | 172.17.224.1 | Cisco ASR 907 | Spoke — Mandal PE |
| Nokia OLT | NOCTESTOLTN001 | 172.17.128.30 | Nokia 7360 (PizzaBox) | GPON OLT — GP Zone |
| Nokia OLT | NOCTESTOLTN003 | 172.17.224.27 | Nokia 7362 (Chassis) | GPON OLT — Mandal Zone |
| HUB Router | NOC507RTRN00001 | 172.17.236.1 | Cisco ASR 9000 (ZCR) | L2VPN Hub Head-end |
| Tejas ONT | Serial: TJNW95D01B1D | — | TJ2100N Series | Customer Premises |
| Nokia ONT | Serial: ALCLFBDC7DCD | — | G-240G-E / G-2425G-A | Customer Premises |
| SCCM/NSO Server | -- | 172.28.2.101 | VM (PHP/Apache) | Orchestration Engine |
| BSS Server | -- | 172.28.10.9 | Application Server | Customer Order Portal |
| Nokia EMS | Nokia AMS (APC) | 172.28.39.12 | Nokia 5520 AMS v9.7.08 | Nokia OLT EMS |
| Tejas EMS | Tejas NMS | 172.28.7.85 | Tejas EMS v8_1_0 | Tejas OLT EMS |

PHYSICAL INVENTORY VISIBILITY IS THROUGH Below API CALL

curl --location 'https://172.28.2.101/sccm/sccm_api/web/api/v1/commited_service_orders' \ --header 'Authorization: Bearer eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpc3MiOiJodHRwOlwvXC8xNzIuMjguMi4xMDEiLCJhdWQiOiJodHRwOlwvXC8xNzIuMjguMi4xMDEiLCJpYXQiOjE3NzY3NDk5MDUsIm5iZiI6MTc3Njc0OTkwNSwiZXhwIjoxNzc2NzUzNTA1LCJqdGkiOiI5ZlNuakZUbGNZRzRsSHRyT0h5R2N6ZWF0T3g3RTlDQyJ9.-JbRpHVgrBfQaz5S-tqQyte1eu7W03i0nTbmLPr8agE' \ --header 'Cookie: _csrf=00429983e4ab06b1fe534ff951e1fe39c90520cf2f0a8b15369fed7cd237db6da%3A2%3A%7Bi%3A0%3Bs%3A5%3A%22_csrf%22%3Bi%3A1%3Bs%3A32%3A%22w3dOJ2Njj7MVS5pZ7H70bUIB4tM8UOb8%22%3B%7D; _identity-backend=6db655b91d135354cbd13985f288e082520277b20195512ba1e43f50b9734985a%3A2%3A%7Bi%3A0%3Bs%3A17%3A%22_identity-backend%22%3Bi%3A1%3Bs%3A47%3A%22%5B52%2C%229fSnjFTlcYG4lHtrOHyGczeatOx7E9CC%22%2C2592000%5D%22%3B%7D' Response: { "status_code": 200, "sub_status_code": 2000, "system": "", "message": "Success", "data": [ { "id": "3875", "service_id": "L3VPN-NOKIA-ASR920", "circuit_id": "CKT-L3VPN-SCCM-L3VPN-NOKIA-ASR920", "devices_list": { "ONT": "ONT1", "OLT": "NOCTESTOLT0003:1-1-4-1-1", "EMS": "nokia-ams-pkga", "MAG1": "NOCTESTMAG001", "GAC0": "NOCTESTGAC001", "ont-serial-number": "ALCLFBE50893", "connected-router": "NOCTESTGAC001", "connected-router-ip": "172.17.128.1" }, "service_start_point": "ONT1", "service_end_point": "", "interfaceID": "Port-channel12", "service_type": "L3VPN / GPON " }, { "id": "4716", "service_id": "S101000049", "circuit_id": "S004092-01-04-000518-TF0001", "devices_list": { "ONT": "ONT6/ONT", "OLT": "SECURASOOLT001:1-1-1-1-6/ONT", "EMS": "nokia-ams-pkga", "ont-serial-number": "ALCLFBDC7DCD", "connected-router": "SECURASOGPR001", "connected-router-ip": "172.16.0.184" }, "service_start_point": "ONT6/ONT", "service_end_point": "", "gateway-ip": "10.14.143.97", "customer-ip": "172.24.36.42", "interfaceID": "Port-channel20", "service_type": "GI-BB / GPON " }, { "id": "5714", "service_id": "S111042026-11050201-008540", "circuit_id": "AC00006300-05-04-000518-TF0001", "devices_list": { "ONT": "ONT3/ONT", "OLT": "SECURASOOLT001:1-1-1-1-3/ONT", "EMS": "nokia-ams-pkga", "ont-serial-number": "TJNW95A36B9F", "connected-router": "SECURASOGPR001", "connected-router-ip": "172.16.0.184" }, "service_start_point": "ONT3/ONT", "service_end_point": "", "interfaceID": "Port-channel20", "service_type": "TDIA / GPON " } ], "app_data": [] }

1.2.2  Logical Inventory

Logical resources assigned to GI-BB services:

| Logical Element | Value / Range | Description |
| --- | --- | --- |
| S-Tag (SVLAN) | 3291 – 3300 | GI-BB outer VLAN on L3VPN Hub & Spoke (ISP VLAN range) |
| C-Tag (CVLAN) | 2537 – 2699 | Customer VLAN assigned per ONT port (from VLAN Master Plan) |
| OLT Network VLAN | 3301 | L2 forwarding VLAN inside Nokia OLT (l2fwder-vlan). Same across all GI-BB services. |
| Bridge Domain | 3301 | L2 bridge domain on GP/Mandal router where pseudowires and interfaces meet |
| VPN ID | 3301 | Shared VPN identifier across HUB and all Spoke routers in the same L2VPN |
| MPLS EXP (QoS) | 2 | MPLS Experimental bits marking for GI-BB data traffic in the MPLS core |
| GI-BB IP Pool A | 10.10.0.0/16 → /28 subnets | Subscriber IP pool — Zone A. Each customer gets one /28 (14 usable IPs). Pool A — unassigned to router. |
| GI-BB IP Pool B | 10.11.0.0/16 → /28 subnets | Subscriber IP pool — Zone B. Mapped to GP routers (172.16.x.x management IPs). PKG: B/C assigned. |
| GI-BB IP Pool C | 10.12.0.0/16 → /28 subnets | Subscriber IP pool — Zone C. Mapped to GP routers (172.16.x.x). PKG: A assigned. |
| Subnet Mask | 255.255.255.240 (/28) | Each GI-BB customer gets a /28 block. First IP = Network, Second = Gateway, Last = Broadcast. |
| Internal IP Range | 172.24.0.0/16 – 172.25.0.0/16 | Internal T-FIBER service addressing (management / L3VPN internal use) |
| External IP Range | 172.22.0.0/16 – 172.23.0.0/16 | External service addressing |
| Route Target (Internal) | RT 1–4999 | RT 1–100: Internal services. RT 101–4999: Government/internal customers (G-to-G, G-to-B) |
| Route Target (External) | RT 5000–9999 | External VPN customers (C-to-C, G-to-C) |
| SCCM Base URL | https://172.28.2.101/sccm/sccm_api/web/api/v1/ | All SCCM REST API calls use this base URL |
| BSS Callback URL | https://172.28.10.9:30098/rest/thirdparty/notification/servicecompletion | SCCM calls this URL to notify BSS of service completion |
| Nokia AMS SOAP URL | http://172.28.39.12:8080/soap/services/ApcRemotePort/9.7 | NSO uses this to push OLT/ONT configurations via SOAP |
| Tejas EMS REST URL | https://172.28.7.85:446/nmsnbi-rest/tapi/data/ | NSO uses this for Tejas OLT provisioning via REST |

LOGICAL INVENTORY VISIBILITY IS THROUGH Below API CALL

curl --location 'https://172.28.2.101/sccm/sccm_api/web/api/v1/service_provisional_details?service_order_id=L3VPN-SER-0002' \ --header 'Authorization: Bearer eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpc3MiOiJodHRwOlwvXC8xNzIuMjguMi4xMDEiLCJhdWQiOiJodHRwOlwvXC8xNzIuMjguMi4xMDEiLCJpYXQiOjE3NzY3NDk5MDUsIm5iZiI6MTc3Njc0OTkwNSwiZXhwIjoxNzc2NzUzNTA1LCJqdGkiOiI5ZlNuakZUbGNZRzRsSHRyT0h5R2N6ZWF0T3g3RTlDQyJ9.-JbRpHVgrBfQaz5S-tqQyte1eu7W03i0nTbmLPr8agE' \ --header 'Cookie: _csrf=e6299892152d72a248f3244ea5d32a8497530a61ceb9f97b16dc73ca1f38cc86a%3A2%3A%7Bi%3A0%3Bs%3A5%3A%22_csrf%22%3Bi%3A1%3Bs%3A32%3A%22aNcWtp5dMpoBMTNa8BvrHafOaDETsT5T%22%3B%7D; _identity-backend=6db655b91d135354cbd13985f288e082520277b20195512ba1e43f50b9734985a%3A2%3A%7Bi%3A0%3Bs%3A17%3A%22_identity-backend%22%3Bi%3A1%3Bs%3A47%3A%22%5B52%2C%229fSnjFTlcYG4lHtrOHyGczeatOx7E9CC%22%2C2592000%5D%22%3B%7D' Response: { "status_code": 200, "sub_status_code": 2000, "system": "", "message": "Success", "data": [ { "id": "284", "service_order_id": "L3VPN-SER-0002", "customer_id": "20", "service_model_id": "346", "endpoints": "1", "port_number": "no", "qos": "0", "bandwidth": null, "profile": null, "flexi_profile": "", "user_defined_data": "{\"vlan-id\":\"123\",\"bfd-required\":\"yes\"}", "system_defined_data": null, "termination_devices": null, "device_list": "[\"cisco_xe\"]", "connected_device": null, "final_nso_payload": "{\"mpls-l3vpn\":{\"service-id\":\"L3VPN-SER-0002\",\"link\":[{\"circuit-id\":\"11:11\",\"vpn-id\":1123,\"site-id\":\"1111\",\"topology-name\":\"topology123\",\"routing-protocol\":\"ospf\",\"pe-type\":\"single\",\"sub-service-id\":\"1.2.1.3\",\"interface\":[{\"interface-id\":\"Gig0\/3\/10\",\"connected-device\":\"cisco_xe\",\"transaction-id\":284,\"device-mode\":\"primary\",\"ip-type\":\"both\",\"customer-vlan\":[{\"vlan-id\":\"123\"}],\"service-instance-no\":[{\"inst-no\":\"12\"}]},{\"interface-id\":\"BDI234\",\"connected-device\":\"cisco_xe\",\"transaction-id\":284,\"ip-type\":\"both\",\"bfd-required\":\"yes\",\"vrf-name\":\"t-fiber12\",\"ipv4-wan-address\":\"1.1.1.1 255.255.255.255\",\"ipv6-wan-address\":\"FE80:0000:0000:0000:0202:B3FF:FE1E:8329\/128\"},{\"interface-id\":\"Gig0\/3\/10\",\"connected-device\":\"cisco_xe\",\"transaction-id\":284,\"device-mode\":\"primary\",\"ip-type\":\"both\",\"customer-vlan\":[{\"vlan-id\":\"123\"}],\"service-instance-no\":[{\"inst-no\":\"12\"}]},{\"interface-id\":\"BDI234\",\"connected-device\":\"cisco_xe\",\"transaction-id\":284,\"ip-type\":\"both\",\"bfd-required\":\"yes\",\"vrf-name\":\"t-fiber12\",\"ipv4-wan-address\":\"1.1.1.1 255.255.255.255\",\"ipv6-wan-address\":\"FE80:0000:0000:0000:0202:B3FF:FE1E:8329\/128\"}],\"prefix-set-list\":[{\"device\":\"cisco_xe\",\"name\":\"t-pre\",\"transaction-id\":284,\"ip-type\":\"ipv4\",\"cust-pref-name-ipv4\":\"pre-ipv4\",\"cust-pref-name-ipv6\":\"pre-ipv6\",\"seq-no\":\"10\",\"ipv4-pool\":\"1.1.1.1\/32\",\"ipv6-pool\":\"FE80:0000:0000:0000:0202:B3FF:FE1E:8329\/128\"},{\"device\":\"cisco_xe\",\"name\":\"t-pre\",\"transaction-id\":5327,\"ip-type\":\"ipv6\",\"cust-pref-name-ipv4\":\"pre-ipv4\",\"cust-pref-name-ipv6\":\"pre-ipv6\",\"seq-no\":\"10\",\"ipv4-pool\":\"1.1.1.1\/32\",\"ipv6-pool\":\"FE80:0000:0000:0000:0202:B3FF:FE1E:8329\/128\"}],\"vrf-definitions\":[{\"device\":\"cisco_xe\",\"vrf-name\":\"t-fiber12\",\"transaction-id\":284,\"address-family\":\"both\",\"rd\":\"1:1\",\"rt-import\":[{\"rt\":\"65535:101\"}],\"rt-export\":[{\"rt\":\"65535:101\"}]},{\"device\":\"cisco_xe\",\"vrf-name\":\"t-fiber12\",\"transaction-id\":284,\"address-family\":\"both\",\"rd\":\"1:1\",\"rt-import\":[{\"rt\":\"65535:101\"}],\"rt-export\":[{\"rt\":\"65535:101\"}]}],\"route-map-list\":[{\"device\":\"cisco_xe\",\"name\":\"route-map-name-lp-ipv4\",\"transaction-id\":\"456\",\"sequence\":\"10\",\"prefix-list-ipv4\":\"pre\",\"local-preference\":\"yes\"},{\"device\":\"cisco_xe\",\"name\":\"route-map-name-lp-ipv6\",\"transaction-id\":\"456\",\"sequence\":\"10\",\"prefix-list-ipv6\":\"pe\",\"local-preference\":\"yes\"},{\"device\":\"cisco_xe\",\"transaction-id\":\"456\",\"name\":\"route-p\",\"sequence\":\"10\",\"prefix-list-ipv4\":\"pre\"},{\"device\":\"cisco_xe\",\"transaction-id\":\"456\",\"name\":\"route-pw\",\"sequence\":\"10\",\"prefix-list-ipv6\":\"pe\"}],\"ospf\":[{\"device\":\"cisco_xe\",\"transaction-id\":\"456\",\"vrf-name\":\"t-fiber12\",\"bgp-id\":\"141688\",\"ospf-process-id\":\"11\",\"ip-type\":\"both\",\"route-map-ipv4\":\"route-map-name-lp-ipv4\",\"route-map-ipv6\":\"route-map-name-lp-ipv6\",\"network\":\"1.1.1.1\",\"ospf-subnet\":\"0.0.0.255\",\"ospf-area-id\":\"0\"},{\"device\":\"cisco_xe\",\"transaction-id\":\"456\",\"vrf-name\":\"t-fiber12\",\"bgp-id\":\"141688\",\"ospf-process-id\":\"11\",\"ip-type\":\"both\",\"route-map-ipv4\":\"rout-fiber\",\"route-map-ipv6\":\"router-to\",\"network\":\"1.1.1.1\",\"ospf-subnet\":\"0.0.0.255\",\"ospf-area-id\":\"0\"}]}]}}", "scheduled_status": "NOW", "scheduled_date": null, "created_by": "1", "created_on": "2022-06-27 17:03:57", "updated_by": null, "updated_on": "2022-06-28 04:44:29", "deleted_by": null, "deleted_on": null, "is_active": "1", "service_instance_status": "NOT_STARTED", "type_of_change": "NEW", "inclusion_hostname": null, "rt_export_import": null, "topology": "Full Mesh", "order_type": "create", "parent_id": null, "parent_transaction_id": null, "leg_number": null, "hub_hostname": null, "uniqueId": "CUST0000021.2.1.31237S00002009", "status": "3", "action": "1", "remarks": "", "dryrun_output": "{\"dryrun-result\": {\n\"native\": {\n\"device\": [\n{\"name\":\"rfs1\"\n,\"data\":\"<rpc xmlns=\\\"urn:ietf:params:xml:ns:netconf:base:1.0\\\"\\n message-id=\\\"3\\\">\\n <edit-config xmlns:nc=\\\"urn:ietf:params:xml:ns:netconf:base:1.0\\\">\\n <target>\\n <running/>\\n </target>\\n <test-option>test-then-set</test-option>\\n <error-option>rollback-on-error</error-option>\\n <with-inactive xmlns=\\\"http://tail-f.com/ns/netconf/inactive/1.0\\\"/>\\n <config>\\n <l3vpn xmlns=\\\"http://com.cisco/rjil/l3vpn\\\">\\n <service-id>L3VPN-SER-0002</service-id>\\n <link>\\n <circuit-id>11:11</circuit-id>\\n <route-map-list>\\n <device>cisco_xe</device>\\n <name>route-map-name-lp-ipv4</name>\\n <transaction-id>456</transaction-id>\\n <local-preference>yes</local-preference>\\n <sequence>10</sequence>\\n <prefix-list-ipv4>pre</prefix-list-ipv4>\\n </route-map-list>\\n <route-map-list>\\n <device>cisco_xe</device>\\n <name>route-map-name-lp-ipv6</name>\\n <transaction-id>456</transaction-id>\\n <prefix-list-ipv6>pe</prefix-list-ipv6>\\n <local-preference>yes</local-preference>\\n <sequence>10</sequence>\\n </route-map-list>\\n <route-map-list>\\n <device>cisco_xe</device>\\n <name>route-p</name>\\n <transaction-id>456</transaction-id>\\n <sequence>10</sequence>\\n <prefix-list-ipv4>pre</prefix-list-ipv4>\\n </route-map-list>\\n <route-map-list>\\n <device>cisco_xe</device>\\n <name>route-pw</name>\\n <transaction-id>456</transaction-id>\\n <prefix-list-ipv6>pe</prefix-list-ipv6>\\n <sequence>10</sequence>\\n </route-map-list>\\n <vrf-definitions>\\n <device>cisco_xe</device>\\n <vrf-name>t-fiber12</vrf-name>\\n <transaction-id>284</transaction-id>\\n <address-family>both</address-family>\\n <rt-import>\\n <rt>65535:101</rt>\\n </rt-import>\\n <rt-export>\\n <rt>65535:101</rt>\\n </rt-export>\\n <rd>1:1</rd>\\n </vrf-definitions>\\n <prefix-set-list>\\n <device>cisco_xe</device>\\n <name>t-pre</name>\\n <transaction-id>284</transaction-id>\\n <cust-pref-name-ipv6>pre-ipv6</cust-pref-name-ipv6>\\n <ipv4-pool>1.1.1.1/32</ipv4-pool>\\n <ip-type>ipv4</ip-type>\\n <seq-no>10</seq-no>\\n <ipv6-pool>FE80:0000:0000:0000:0202:B3FF:FE1E:8329/128</ipv6-pool>\\n <cust-pref-name-ipv4>pre-ipv4</cust-pref-name-ipv4>\\n </prefix-set-list>\\n <prefix-set-list>\\n <device>cisco_xe</device>\\n <name>t-pre</name>\\n <transaction-id>5327</transaction-id>\\n <cust-pref-name-ipv6>pre-ipv6</cust-pref-name-ipv6>\\n <ipv4-pool>1.1.1.1/32</ipv4-pool>\\n <ip-type>ipv6</ip-type>\\n <seq-no>10</seq-no>\\n <ipv6-pool>FE80:0000:0000:0000:0202:B3FF:FE1E:8329/128</ipv6-pool>\\n <cust-pref-name-ipv4>pre-ipv4</cust-pref-name-ipv4>\\n </prefix-set-list>\\n <interface>\\n <interface-id>BDI234</interface-id>\\n <connected-device>cisco_xe</connected-device>\\n <transaction-id>284</transaction-id>\\n <ipv6-wan-address>FE80:0000:0000:0000:0202:B3FF:FE1E:8329/128</ipv6-wan-address>\\n <bfd-required>yes</bfd-required>\\n <ip-type>both</ip-type>\\n <dual-homed>true</dual-homed>\\n <ipv4-wan-address>1.1.1.1 255.255.255.255</ipv4-wan-address>\\n <vrf-name>t-fiber12</vrf-name>\\n </interface>\\n <interface>\\n <interface-id>Gig0/3/10</interface-id>\\n <connected-device>cisco_xe</connected-device>\\n <transaction-id>284</transaction-id>\\n <ip-type>both</ip-type>\\n <service-instance-no>\\n <inst-no>12</inst-no>\\n </service-instance-no>\\n <dual-homed>true</dual-homed>\\n <customer-vlan>\\n <vlan-id>123</vlan-id>\\n </customer-vlan>\\n <device-mode>primary</device-mode>\\n </interface>\\n <routing-protocol>ospf</routing-protocol>\\n <sub-service-id>1.2.1.3</sub-service-id>\\n <pe-type>single</pe-type>\\n <ospf>\\n <device>cisco_xe</device>\\n <vrf-name>t-fiber12</vrf-name>\\n <transaction-id>456</transaction-id>\\n <route-map-ipv4>rout-fiber</route-map-ipv4>\\n <bgp-id>141688</bgp-id>\\n <route-map-ipv6>router-to</route-map-ipv6>\\n <ip-type>both</ip-type>\\n <ospf-process-id>11</ospf-process-id>\\n <network>1.1.1.1</network>\\n <ospf-subnet>0.0.0.255</ospf-subnet>\\n <ospf-area-id>0</ospf-area-id>\\n </ospf>\\n </link>\\n </l3vpn>\\n </config>\\n </edit-config>\\n</rpc>\\n\"}\n,{\"name\":\"cisco_xe\"\n,\"data\":\"vrf definition t-fiber12\\n rd 1:1\\n route-target export 65535:101\\n route-target import 65535:101\\n address-family ipv4\\n exit-address-family\\n !\\n address-family ipv6\\n exit-address-family\\n !\\n!\\nfhrp version vrrp v3\\nip prefix-list pre-ipv4 seq 10 permit 1.1.1.1/32\\nipv6 prefix-list pre-ipv6 seq 10 permit fe80::202:b3ff:fe1e:8329/128\\ninterface GigabitEthernet0/3/10\\n no ip address\\n no shutdown\\n no switchport\\n service instance 12 ethernet\\n encapsulation dot1q 123\\n rewrite ingress tag pop 1 symmetric\\n bridge-domain 123\\n exit\\n mtu 9216\\n load-interval 30\\nexit\\ninterface BDI234\\n bfd interval 200 min_rx 200 multiplier 3\\n vrf forwarding t-fiber12\\n ip address 1.1.1.1 255.255.255.255\\n no ip unreachables\\n no ip proxy-arp\\n no ip redirects\\n ip mtu 9216\\n ip mask-reply\\n ipv6 address fe80::202:b3ff:fe1e:8329/128\\n ipv6 enable\\n ipv6 mtu 9216\\n load-interval 30\\n no shutdown\\nexit\\nroute-map route-map-name-lp-ipv4 permit 10\\n match ip address prefix-list pre\\n set local-preference 200\\n!\\nroute-map route-map-name-lp-ipv6 permit 10\\n match ipv6 address prefix-list pe\\n set local-preference 200\\n!\\nroute-map route-p permit 10\\n match ip address prefix-list pre\\n!\\nroute-map route-pw permit 10\\n match ipv6 address prefix-list pe\\n!\\nrouter ospf 11 vrf t-fiber12\\n log-adjacency-changes\\n redistribute bgp 141688 metric-type 1 subnets\\n network 1.1.1.1 0.0.0.255 area 0\\nexit\\nrouter bgp 141688\\n address-family ipv4 unicast vrf t-fiber12\\n redistribute ospf 11 route-map rout-fiber match internal external 1\\n exit-address-family\\n !\\n address-family ipv6 unicast vrf t-fiber12\\n redistribute ospf 11 route-map router-to match internal external 1\\n exit-address-family\\n !\\n!\\n\"}\n]\n}\n}\n}\n", "commit_output": "", "delete_output": null, "delete_dryrun_output": null, "endpoint_device_id": "1237", "port": "", "requested_port": null, "verification_status": "0", "validation_container": null, "variables": "{\"1237\":{\"l3vpn-common-component_2_primary-CISCO\":{\"92\":{\"value\":\"ospf\",\"module_id\":\"2\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"}},\"interface_36_primary-CISCO\":{\"49\":{\"value\":\"primary\",\"module_id\":\"36\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"45\":{\"value\":\"cisco_xe\",\"module_id\":\"36\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"50\":{\"value\":\"12\",\"module_id\":\"36\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"48\":{\"value\":\"Gig0\/3\/10\",\"module_id\":\"36\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"87\":{\"value\":\"both\",\"module_id\":\"36\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"31\":{\"value\":\"123\",\"module_id\":\"36\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"}},\"interface_30_primary-CISCO\":{\"88\":{\"value\":\"yes\",\"module_id\":\"30\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"45\":{\"value\":\"cisco_xe\",\"module_id\":\"30\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"87\":{\"value\":\"both\",\"module_id\":\"30\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"76\":{\"value\":\"1.1.1.1 255.255.255.255\",\"module_id\":\"30\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"77\":{\"value\":\"FE80:0000:0000:0000:0202:B3FF:FE1E:8329\/128\",\"module_id\":\"30\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"3\":{\"value\":\"t-fiber12\",\"module_id\":\"30\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"}},\"prefix-set-list_45_primary-CISCO\":{\"90\":{\"value\":\"pre-ipv4\",\"module_id\":\"45\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"91\":{\"value\":\"pre-ipv6\",\"module_id\":\"45\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"45\":{\"value\":\"cisco_xe\",\"module_id\":\"45\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"84\":{\"value\":\"1.1.1.1\/32\",\"module_id\":\"45\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"85\":{\"value\":\"FE80:0000:0000:0000:0202:B3FF:FE1E:8329\/128\",\"module_id\":\"45\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"83\":{\"value\":\"10\",\"module_id\":\"45\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"}},\"vrf-definitions_28_primary-CISCO\":{\"45\":{\"value\":\"cisco_xe\",\"module_id\":\"28\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"6\":{\"value\":\"1:1\",\"module_id\":\"28\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"65\":{\"value\":\"65535:101\",\"module_id\":\"28\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"64\":{\"value\":\"65535:101\",\"module_id\":\"28\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"3\":{\"value\":\"t-fiber12\",\"module_id\":\"28\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"}},\"route-map-list_46_primary-CISCO\":{\"45\":{\"value\":\"cisco_xe\",\"module_id\":\"46\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"83\":{\"value\":\"10\",\"module_id\":\"46\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"8\":{\"value\":\"456\",\"module_id\":\"46\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"}},\"ospf_49_primary-CISCO\":{\"86\":{\"value\":\"141688\",\"module_id\":\"49\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"45\":{\"value\":\"cisco_xe\",\"module_id\":\"49\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"87\":{\"value\":\"both\",\"module_id\":\"49\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"93\":{\"value\":\"route-map-name-lp-ipv4\",\"module_id\":\"49\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"94\":{\"value\":\"route-map-name-lp-ipv6\",\"module_id\":\"49\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"8\":{\"value\":\"456\",\"module_id\":\"49\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"3\":{\"value\":\"t-fiber12\",\"module_id\":\"49\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"}},\"prefix-set-list_32_primary-CISCO\":{\"90\":{\"value\":\"pre-ipv4\",\"module_id\":\"32\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"91\":{\"value\":\"pre-ipv6\",\"module_id\":\"32\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"45\":{\"value\":\"cisco_xe\",\"module_id\":\"32\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"84\":{\"value\":\"1.1.1.1\/32\",\"module_id\":\"32\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"85\":{\"value\":\"FE80:0000:0000:0000:0202:B3FF:FE1E:8329\/128\",\"module_id\":\"32\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"83\":{\"value\":\"10\",\"module_id\":\"32\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"}},\"route-map-list_34_primary-CISCO\":{\"45\":{\"value\":\"cisco_xe\",\"module_id\":\"34\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"83\":{\"value\":\"10\",\"module_id\":\"34\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"8\":{\"value\":\"456\",\"module_id\":\"34\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"}},\"ospf_35_primary-CISCO\":{\"86\":{\"value\":\"141688\",\"module_id\":\"35\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"45\":{\"value\":\"cisco_xe\",\"module_id\":\"35\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"87\":{\"value\":\"both\",\"module_id\":\"35\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"69\":{\"value\":\"rout-fiber\",\"module_id\":\"35\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"70\":{\"value\":\"router-to\",\"module_id\":\"35\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"8\":{\"value\":\"456\",\"module_id\":\"35\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"},\"3\":{\"value\":\"t-fiber12\",\"module_id\":\"35\",\"device_mode\":\"primary\",\"device_type\":\"CISCO\"}}}}", "playbook_status": null, "adl_type": "false", "allowed_device_list": "", "service_specific_id": "S00002009", "adl_rtexport": null, "adl_vrf_import_block": null, "dry_ref_id": null, "depend_on_id": null, "macd_json": null, "service_trnx_instance_id": null, "isp_name": "0", "location_id": "0", "ref_id": null, "gpon_only": null, "precheck_status": null, "precheck_url": null, "postcheck_status": null, "postcheck_url": null, "delete_precheck_status": null, "delete_precheck_url": null, "delete_postcheck_status": null, "delete_postcheck_url": null, "force_commit": "0", "reqDtlID": null } ], "app_data": [] }

1.2.3  Sub-Service ID Reference (Service type Variants (Gi-bb))

Each GI-BB service variant has a unique sub_service_id used in BSS → SCCM API calls:

| Sub Service ID | Router | Mode | VLAN Type | Description |
| --- | --- | --- | --- | --- |
| 9.5.2.1 | ASR 920 | Bridge | Untagged / Tagged | Nokia OLT → ASR920. Bridge mode. CPE sends untagged or tagged frames. |
| 9.5.2.2 | ASR 907 | Bridge | Untagged / Tagged | Nokia OLT → ASR907. Bridge mode. OLT adds PVID or single-tag. |
| 9.5.2.4 | ASR 920 | Route | Tagged | Nokia OLT → ASR920. Route mode. ONT acts as IP router. DHCP enabled on WAN. |
| 9.5.2.5 | ASR 907 | Route | Tagged | Nokia OLT → ASR907. Route mode. ONT acts as IP router. DHCP enabled on WAN. |

NOTE: Each entry has a Sub Service Id (like 9.5.2.6) And Its Naming convention is as given below (like GI-BB-NOKIA-SINGLE-ASR9k-ROUTE).

- First digit (9) → Refers to the main service family (here, GI-BB = “Gigabit Backbone” service).

- Second digit (5) → Indicates the vendor/device type (e.g., Nokia).

- Third digit (2) → Defines the deployment model (e.g., “Single” device setup vs. dual/cluster).

- Fourth digit (6) → Specifies the device model and function (e.g., ASR9k with routing).

1.2.4  GI-BB Test Scenarios

The following 12 test scenarios were executed from BSS-WFM-SCCM(NSO).

| Sr | OLT ¦ ONT | Router | Mode | VLAN Tag | Sub Svc ID | Service Order |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Nokia ¦ Tejas | ASR 907 | Bridge | Untagged | 9.5.2.2 | S101000587 |
| 2 | Nokia ¦ Nokia | ASR 907 | Bridge | Untagged | 9.5.2.2 | S101000582 |
| 3 | Nokia ¦ Tejas | ASR 907 | Route | Tagged | 9.5.2.5 | S101000592 |
| 4 | Nokia ¦ Nokia | ASR 907 | Route | Tagged | 9.5.2.5 | S101000594 |
| 5 | Nokia ¦ Tejas | ASR 920 | Bridge | Untagged | 9.5.2.1 | S101000547 |
| 6 | Nokia ¦ Nokia | ASR 920 | Bridge | Untagged | 9.5.2.1 | S101000607 |
| 7 | Nokia ¦ Tejas | ASR 920 | Route | Tagged | 9.5.2.4 | S101000604 |
| 8 | Nokia ¦ Nokia | ASR 920 | Route | Tagged | 9.5.2.4 | S101000601 |
| 9 | Nokia ¦ Nokia | ASR 920 | Bridge | Tagged | 9.5.2.1 | S101000543 |
| 10 | Nokia ¦ Tejas | ASR 920 | Bridge | Tagged | 9.5.2.1 | S101000635 |
| 11 | Nokia ¦ Tejas | ASR 907 | Bridge | Tagged | 9.5.2.2 | S101000638 |
| 12 | Nokia ¦ Nokia | ASR 907 | Bridge | Tagged | 9.5.2.2 | S101000639 |

1.3.  End-to-End Service Provisioning Flow

The diagram below shows the complete lifecycle of a service order —

| STEP 1–2 BSS Customer Order | STEP 3–5 SCCM Orchestration | STEP 6–7 NSO + NETWORK Config Push | STEP 8–9 CALLBACK + OSS Completion |
| --- | --- | --- | --- |

PHASE 1 BSS — Customer Order & Authentication

What BSS Does —

BSS is the starting point. A government institution (school, Gram Panchayat, clinic, etc.) places a broadband order through the BSS portal or CRM system. BSS is responsible for making sure the customer is valid, the service is available, and then handing the order to SCCM with all the technical parameters SCCM needs to build the connection.

BSS does NOT configure any network device. Its job is to collect the order, validate it, pass it correctly to SCCM, and then wait for SCCM to tell it the service is active.

| 1 | BSS | Customer places service order A government institution submits a GI-BB broadband request specifying: location, bandwidth plan, CPE type (Nokia/Tejas ONT), and service type (Bridge/Route mode). BSS generates a unique Service Order ID (e.g., S101000587). |
| --- | --- | --- |

| 2 | BSS → SCCM | Login — get access token POST /login with username and password. SCCM returns a JWT Bearer Token. BSS must include this token in the Authorization header of every subsequent API call to SCCM in this session. |
| --- | --- | --- |

| 2a | BSS → SCCM | Validate customer record GET /customers?customer_id= — BSS checks if the customer exists in SCCM. If the customer is new, BSS first creates them: POST /customers with customer_name, customer_id, short_name. |
| --- | --- | --- |

| 2b | BSS → SCCM | Fetch sub-service catalog GET /catalogitem — BSS retrieves the sub_service_id and list of required parameters (User Defined and System Defined) for the specific service type. This determines what fields must be sent in Step 3. |
| --- | --- | --- |

PHASE 2 BSS → SCCM — Three-Step Service Provisioning

How the 3-Step Provisioning Works

Once BSS has authenticated and fetched the catalog, it calls SCCM in three sequential steps. 
Step 1 is the cover sheet (service identity), Step 2 is the technical specifications (which device, which port, QoS requirements), and Step 3 is pressing the Submit button to execute everything.

Each step must succeed before the next begins. If Step 2 fails, Step 3 must not be called. SCCM validates each step before accepting the next.

| 3 | BSS → SCCM | Step 1 — Initiate Service Order POST /servicedeploy/step1. BSS sends the service order ID, customer ID, sub_service_id, device count, topology, location, service VLAN, and ISP name. SCCM creates a transaction record and returns a transaction_id (e.g., 2274). This ID links all three steps together. |
| --- | --- | --- |

| 4 | BSS → SCCM | Step 2 — Device & QoS Parameters POST /servicedeploy/step2. BSS sends the transaction_id plus device-level details: PE hostname (which router), service-specific ID, QoS config (pbit values and CIR % per traffic class: SIGN/CS/DATA/OAM), hub router hostname, interface ID, shaping value (200 Mbps), customer VLANs. |
| --- | --- | --- |

| 5 | BSS → SCCM | Step 3 — Commit & Execute POST /servicedeploy/step3. BSS sends the transaction_id with action=1 (Create). SCCM builds the full NSO payload, runs a dry-run validation, then commits the service. SCCM returns a unique_id (Circuit ID, e.g., CKT-GI-BB-SCCM-S000754). Network provisioning begins in the background. |
| --- | --- | --- |

Note: Step 1,2,3 For Api’s information Refer: Below Phase 4- Point : 4.  API Catalogue — Complete Reference or

PHASE 3 SCCM / NSO — Network Configuration Push

What SCCM and NSO Do —

After Step 3 is committed, SCCM constructs the full device configuration and passes it to NSO. NSO is Cisco's Network Services Orchestrator — it knows how to talk to every device in the network. NSO translates the service description into actual router commands and OLT configuration templates, then pushes them to the devices.

This is the only phase that physically changes the network. Everything before this was just data validation and preparation. If NSO successfully configures all devices, the service is live. If any single device fails, NSO automatically rolls back every change it made — the network is left clean with no partial configuration.

| 6 | SCCM → NSO | Internal: Build NSO payload SCCM resolves device assignments, VLAN resources, interface mappings, and QoS policy parameters. It constructs the NSO service template (JSON/YANG model) and triggers NSO execution via the internal SCCM-NSO API. |
| --- | --- | --- |

| 7a | NSO → Router | Push Router Configuration (ASR 907/920) NSO configures the GP or Mandal router via NETCONF: L2VFI (l2 vfi ... manual, vpn id 3301, bridge-domain 3301), service instance on the physical interface (encapsulation dot1q, rewrite ingress tag, bridge-domain), and QoS policy (MPLS EXP marking, CIR policing). Neighbor pseudowire to HUB router is established. |
| --- | --- | --- |

| 7b | NSO → Nokia OLT (via Nokia AMS SOAP | Push OLT/ONT Configuration — Bridge Mode NSO calls Nokia AMS SOAP API with template ORG_SERV_U_D_CVLAN (Untagged) or ORG_SERV_U_D_CVLAN-Tag (Tagged). Parameters: ONT serial number, ethernet slot (ontCardId=1), bandwidth profiles (UP/DN), CVLAN, SVLAN name, subscriber IDs. |
| --- | --- | --- |

| 7c | NSO → Nokia OLT (via Nokia AMS SOAP) | Push OLT/ONT Configuration — Route Mode NSO calls Nokia AMS SOAP API with template TEMPLATE-GI-BB_ROUTE. Parameters: ONT serial number, VEIP slot (ontCardId=14), VEIP QoS profiles, CVLAN, SVLAN. ONT acts as L3 router with DHCP enabled on WAN interface. |
| --- | --- | --- |

| 7d | NSO → Tejas OLT (via Tejas EMS REST/JSON) | Push OLT/ONT Configuration — Bridge Mode Token generation for session authentication, Add ONT by serial number, Make ONT Admin UP, Set Ethernet port (ontCardId = 1) to Bridged Mode, Retrieve Service UUID from EMS, Tejas EMS: Add UNI (Bridge Mode Untagged), Tejas EMS: Add Location & Customer ID. JSON payloads are exchanged with EMS, which translates them into device-native CLI for OLT/ONT configuration. |
| --- | --- | --- |

| 7e | NSO → Tejas OLT (via Tejas EMS REST/JSON) | Push OLT/ONT Configuration — Route Mode NSO calls the Tejas EMS TAPI REST NBI with Route Mode templates. Token generation for session authentication, Add ONT by serial number, Make ONT Admin UP, Configure VEIP slot (ontCardId = 14), Apply VEIP QoS profiles, Assign CVLAN and SVLAN, Enable DHCP on WAN interface (ONT acts as L3 router), Retrieve Service UUID and bind subscriber details. EMS pushes JSON payload downstream, applying routing and service flows to ONT. |
| --- | --- | --- |

OPTICAL DOMAIN DWDM (NSO to DWDM communication)

PHASE 4 Callback to BSS + OSS Inventory Update

What Happens After Network Config is Pushed

Once NSO confirms all devices are configured successfully, SCCM asynchronously sends a notification back to BSS. This notification (callback) tells BSS the service is live, includes the Circuit ID for reference, and carries the success/failure status.

BSS does not poll SCCM for status. It simply waits on its callback endpoint. When the callback arrives, BSS updates the customer's service order to 'Active' and the customer's service is confirmed.

In parallel, OSS can query SCCM at any time to retrieve the inventory of all active services — which devices each service uses, what the circuit IDs are, and the service topology. OSS uses this for network monitoring and fault management.

| 8 | SCCM → BSS | Callback: Service Completion Notification SCCM POSTs to the BSS callback URL: https://172.28.10.9:30098/rest/thirdparty/notification/servicecompletion. Payload includes: service_order_id (original BSS order), unique_id (circuit ID), status_code=2000 (success). BSS acknowledges and marks the service as ACTIVE. |
| --- | --- | --- |

| 9 | OSS → SCCM | Inventory Query (Ongoing / On-Demand) OSS queries SCCM via GET APIs to maintain network inventory: GET /commited_service_orders (all services), GET /service_order_details (per service with A-end, Z-end, device list), GET /service_list (by date range for audits). OSS records service_id, circuit_id, device list, and service topology. |
| --- | --- | --- |

1.4.  API Catalogue — Complete Reference

This section provides the full details of every API call in the GI-BB provisioning lifecycle, in the order they are executed. All SCCM APIs use base URL: https://172.28.2.101/sccm/sccm_api/web/api/v1/

BSS → SCCM APIs (Calls made FROM BSS TO SCCM)

SCCM → BSS API (Callback: SCCM notifies BSS of completion)

OSS → SCCM APIs (Inventory Queries: OSS reads from SCCM)

1.5.  Complete API Sequence — Master Reference Table

The table below shows ALL API calls in the correct execution order for a complete GI-BB service provisioning lifecycle. Steps 1–8 happen per service order. Steps 9–12 are ongoing OSS inventory queries.

| # | From → To | Method | Endpoint | Purpose |
| --- | --- | --- | --- | --- |
| 1 | BSS → SCCM | POST | /login | BSS gets JWT token for session. Required before any other call. |
| 2 | BSS → SCCM | GET | /customers?customer_id= | BSS validates customer exists. If not, call POST /customers to create. |
| 3 | BSS → SCCM | GET | /catalogitem?service_name=... | BSS gets sub_service_id and required parameters for this service type. |
| 4 | BSS → SCCM | POST | /servicedeploy/step1 | Service initiation. Returns transaction_id. Status code: 2001. |
| 5 | BSS → SCCM | POST | /servicedeploy/step2 | Device, interface, QoS parameters sent. Status code: 2002. |
| 6 | BSS → SCCM | POST | /servicedeploy/step3 | Commit. NSO push begins. Returns unique_id (circuit ID). Status: 2000. |
| 7 | SCCM → NSO | Internal | Internal SCCM-NSO | SCCM builds NSO payload and triggers network configuration push. |
| 7a | NSO → Router | NETCONF | ASR 907 / ASR 920 | L2VFI, bridge-domain, service-instance, QoS policy pushed to GP/Mandal router. |
| 7b | NSO → Nokia OLT | SOAP POST | /soap/ApcRemotePort/9.7 | VLAN, ONT port, bandwidth profile, bridge config pushed via Nokia AMS. |
| 7c | NSO → Tejas OLT | REST PUT/POST | /nmsnbi-rest/tapi/data/ | 8-step Tejas EMS sequence: Token→AddONT→AdminUP→ETH mode→GetUUID→AddUNI→CreateSvc→AddLocation |
| 8 | SCCM → BSS | POST (Callback) | /notification/servicecompletion | SCCM notifies BSS: service is live. Includes service_order_id, unique_id, status_code=2000. |
| 9 | OSS → SCCM | GET | /commited_service_orders | OSS retrieves full inventory of all active services with device lists. |
| 10 | OSS → SCCM | GET | /commited_service_orders?service_id= | OSS queries specific service for fault isolation or inventory check. |
| 11 | OSS → SCCM | GET | /service_list?start_date=&end_date= | OSS retrieves services by date for reconciliation and audit. |
| 12 | OSS → SCCM | GET | /service_order_details?service_order_id= | OSS gets A-end, Z-end, full device path for a specific order. |
| 13 | OSS → SCCM | GET | /serviceinstance?start_date=&end_date= | OSS retrieves list of service instance IDs for inventory delta sync. |

1.6.  Status Codes & Error Handling

| Code | Meaning | When It Occurs | Action Required |
| --- | --- | --- | --- |
| 2001 | Step 1 Success | Step 1 POST accepted. Transaction created. | Proceed to Step 2 with the returned transaction_id. |
| 2002 | Step 2 Success | Step 2 POST accepted. Device parameters stored. | Proceed to Step 3 with the same transaction_id. |
| 2000 | Full Success | Step 3 committed OR callback to BSS on service completion. | BSS marks service as ACTIVE. Store circuit unique_id. |
| 200 | Query Success | OSS GET API returned data successfully. | Process the response data for inventory update. |
| 4xx | Client Error | Bad request, auth failure, invalid parameter, customer not found. | Fix the request. Re-authenticate if 401. Check parameters if 400/404. |
| 5xx | Server / NSO Error | NSO push failed, device unreachable, SCCM internal error. | NSO rolls back all changes automatically. BSS notified via callback with non-2000 code. Raise incident. |

1.7.  System Architecture — NSO

NSO (Network Services Orchestrator — Cisco)

NSO provides the network automation layer. It translates service model instances (YANG) into device-specific commands and manages the transactional push to all devices simultaneously. Key features used in T-FIBER:

- Device Manager: Manages device configurations transactionally. Supports NETCONF, CLI, SNMP via Network Element Drivers (NEDs).

- Service Manager: Maps service templates to device configurations using FASTMAP algorithm. One service definition covers create, modify, and delete automatically.

- Configuration Database (CDB): Real-time database always in sync with actual device configurations. No offline inventory lag.

- Rollback: If any single device fails during a push, NSO rolls back ALL changes across ALL devices in the same transaction. No partial configurations left on the network.

- SCCM Integration: NSO runs on 2 physical servers × 36 vCPU × 256 GB RAM × 3600 GB storage (NSO v5.7, RHEL 8.1).

SCCM → NSO Flow (Internal)

| A | Flow A — Success | NSO pushes config to all devices → Notification sent to SCCM → SCCM runs & generates pass/fail verification report → SCCM sends callback to BSS with status_code=2000. |
| --- | --- | --- |
| B | Flow B — Failure | If NSO fails on even 1 device out of N → NSO rolls back all changes from all devices → Error response sent to SCCM → SCCM provides error response to BSS → BSS does NOT mark service as active. |

1.8.Glossary

| Term | Definition |
| --- | --- |
| GI-BB | Gigabit Broadband — T-FIBER's broadband service for government institutions (schools, GP offices, hospitals). |
| BSS | Business Support System — Customer-facing portal. Accepts orders, validates customers, triggers provisioning, receives completion callbacks. |
| OSS | Operations Support System — Network inventory and assurance. Queries SCCM for service and device data. Does NOT provision. |
| SCCM | Service & Configuration Control Manager — Orchestration middleware between BSS and NSO. Validates, builds, and tracks all service transactions. |
| NSO | Network Services Orchestrator (Cisco) — Pushes actual device configurations. Only system that directly touches routers and OLTs. |
| L2VPN | Layer 2 Virtual Private Network — The technology used for GI-BB. Creates a virtual Ethernet connection between the customer site and the internet gateway. |
| GPON | Gigabit Passive Optical Network — The optical fiber access technology delivering internet from the OLT to the customer's ONT device. |
| OLT | Optical Line Terminal — T-FIBER's equipment at the exchange/distribution point. Connects to multiple customer ONTs via fiber. |
| ONT | Optical Network Terminal — The box installed at the customer premises. Converts optical fiber signal to Ethernet for the customer's devices. |
| transaction_id | Temporary ID returned by Step 1 API. Used to link Steps 2 and 3 together. Not the final service ID. |
| unique_id | Final Circuit ID generated by SCCM after Step 3 succeeds (e.g., CKT-GI-BB-SCCM-S000754). Permanent reference for the active service. |
| sub_service_id | SCCM catalog code identifying the specific service variant: 9.5.2.1 = ASR920 Bridge, 9.5.2.2 = ASR907 Bridge, 9.5.2.4 = ASR920 Route, 9.5.2.5 = ASR907 Route. |
| Bridge Mode | ONT passes Ethernet frames transparently. Customer device gets IP from upstream DHCP. DHCP snooping must be Disabled at OLT level. |
| Route Mode | ONT acts as an IP router. ONT gets WAN IP via DHCP from BRAS/BNG. Assigns private IPs to customer LAN. VEIP slot used (not ETH slot). |
| CVLAN / C-Tag | Customer VLAN — inner VLAN assigned to the customer's ONT port. Range: 2537–2699 for GI-BB. |
| SVLAN / S-Tag | Service VLAN — outer VLAN at the OLT uplink. 3301 is the standard GI-BB SVLAN in T-FIBER. |
| MPLS EXP | MPLS Experimental bits — used for QoS traffic marking in the MPLS core network. GI-BB data traffic marked as EXP=2. |
| JWT Token | JSON Web Token — time-limited authentication credential returned by SCCM /login API. Must be included in all subsequent API calls. |
| /28 Subnet | A block of 16 IP addresses (14 usable). Each GI-BB customer gets one /28 from the IP pool (10.10.x.x / 10.11.x.x / 10.12.x.x). |

2.  ONT LAN Ports — Enable (Unshut) During Service Configuration

ONT LAN Port Activation — How Ports Are Unshut During Provisioning
This is the most important technical detail of the ONT provisioning step. When the field engineer installs the ONT, all LAN ports are in a SHUT (disabled) state by default. They are enabled AUTOMATICALLY as part of the NSO provisioning SOAP/REST call — not manually. The number of ports enabled is driven by the service mode and the ONT model.

2.1 The Parameter That Controls It

"oper_NumberOfDataPorts — Number of Ethernet Ports in ONT. This will vary based on ONT Model,

NSO includes this parameter in the Nokia AMS SOAP call and in the Tejas EMS REST call. The AMS/EMS then configures the OLT to enable exactly that many LAN ports on the ONT. Before this SOAP/REST call is executed, all ports are disabled. After it is executed, ports are live and ready for customer devices.

2.2  Port Count by Service Mode

| Service Mode | Template Used | ontCardId (Slot) | oper_NumberOfDataPorts | Effect on ONT |
| --- | --- | --- | --- | --- |
| Bridge Mode — Untagged | ORG_SERV_U_D_CVLAN | 1 (Ethernet slot) | 4 | All 4 LAN ports ENABLED (unshut) |
| Bridge Mode — Tagged | ORG_SERV_U_D_CVLAN-Tag | 1 (Ethernet slot) | 4 | All 4 LAN ports ENABLED (unshut) |
| Route Mode | TEMPLATE-GI-BB_ROUTE | 14 (VEIP slot) | 1 | 1 VEIP port ENABLED (WAN port for routing). Ethernet ports not activated via this parameter. |

NOTE: In Route Mode, ontCardId=14 means the VEIP (Virtual Ethernet Interface Point) slot is used — not the physical Ethernet card. The ONT acts as a router. WAN IP is obtained via DHCP on the VEIP interface. Physical LAN ports are still available to customer devices but the uplink uses the VEIP.

2.3  ONT Model Port Reference

The oper_NumberOfDataPorts value is looked up from this table based on the specific ONT model installed by the field engineer:

| OEM | ONT Model | Type | Slot Type | Slot No. | No. of Ports |
| --- | --- | --- | --- | --- | --- |
| Nokia | G-240G-E | Non-WiFi | ethernet | 1 | 4 |
| Nokia | G-240G-E | Non-WiFi | veip | 14 | 1 |
| Nokia | G-2425G-A | WiFi ONT | ethernet | 1 | 4 |
| Nokia | G-2425G-A | WiFi ONT | veip | 14 | 1 |
| Tejas | TJ2100N-11G | Non-WiFi | ethernet | 1 | 4 |
| Tejas | TJ2100N-11G | Non-WiFi | veip | 4 | 1 |
| Tejas | TJ2100N-11GT | Non-WiFi | ethernet | 1 | 4 |
| Pioneer | DTP9762 | WiFi ONT | ethernet | 1 | 2 |
| Dasan | H660GM-A | WiFi ONT | ethernet | 1 | 4 |
| Altice | SD6BKGC | WiFi ONT | ethernet | 1 | 2 |
| NetLink | HG323DAC | WiFi ONT | ethernet | 4 | 4 |
| Tejas | TJ2100N-G004011EAC1 | WiFi ONT | ethernet | 1 | 4 |

2.4  Nokia AMS SOAP Call — Bridge Mode (How Ports Are Enabled)

NSO sends this SOAP call to Nokia AMS. The oper_NumberOfDataPorts parameter inside the call is what enables the ONT LAN ports:

Bridge Mode — Untagged  |  Template: ORG_SERV_U_D_CVLAN

URL: http://172.28.39.12:8080/soap/services/ApcRemotePort/9.7 Method: SOAP POST <ns:configure> <objectName>NOCTESTOLTN001:1-1-1-2-10</objectName> <!-- OLT:PON Port:ONT ID --> <templateName>ORG_SERV_U_D_CVLAN</templateName> <!-- Bridge Mode Untagged template --> <templateVersion>2</templateVersion> <argument><name>ontSerialNumber</name> <value>TJNW957F5907</value></argument> <!-- ONT Serial (from field engineer) --> <argument><name>ontCardId</name> <value>1</value></argument> <!-- Ethernet card slot --> <argument><name>oper_NumberOfDataPorts</name> <value>4</value></argument> <!-- *** 4 LAN PORTS ENABLED (UNSHUT) *** --> <argument><name>ethPortId</name> <value>1</value></argument> <!-- First Ethernet port for service --> <argument><name>ontEthPortQosProfiles_BandwidthProfileUsQueue1</name> <value>UP_TNET_GI_30M</value></argument> <!-- 30 Mbps upload profile --> <argument><name>ontEthPortQosProfiles_ShaperProfileDsQueue0</name> <value>DN_TNET_GI_30M</value></argument> <!-- 30 Mbps download profile --> <argument><name>vlanCustomerId</name> <value>2537</value></argument> <!-- CVLAN assigned to this customer --> <argument><name>networkVlanName</name> <value>GI-Internet_S3301</value></argument> <!-- SVLAN 3301 --> <argument><name>ontSubscriberId2</name> <value>TF-S002002482-GIBB_2537-30Mbps</value></argument> <!-- ServiceID_CVLAN_BW --> </ns:configure>

Route Mode  |  Template: TEMPLATE-GI-BB_ROUTE  |  oper_NumberOfDataPorts = 1

URL: http://172.28.39.12:8080/soap/services/ApcRemotePort/9.7 Method: SOAP POST <ns:configure> <objectName>NOCTESTOLTN001:1-1-1-2-1</objectName> <templateName>TEMPLATE-GI-BB_ROUTE</templateName> <!-- Route Mode template --> <templateVersion>1</templateVersion> <argument><name>ontCardId</name> <value>14</value></argument> <!-- VEIP slot 14 (not Ethernet slot 1) --> <argument><name>oper_NumberOfDataPorts</name> <value>1</value></argument> <!-- *** 1 VEIP PORT ENABLED *** --> <argument><name>veipPortId</name> <value>1</value></argument> <!-- Default VEIP port --> <argument><name>ontVeipPortQosProfiles_BandwidthProfileUsQueue1</name> <value>UP_TNET_GI_30M</value></argument> <argument><name>vlanCustomerId</name> <value>2542</value></argument> </ns:configure>

2.5  Tejas EMS REST Call — How Ports Are Enabled

For Tejas ONTs, NSO uses the Tejas EMS REST API. The port count is set in the Add ONT step (Step 2 of 8) using eth-circuitpack-preprovision-data node-edge-point-count:

Step 2 — Add ONT with Port Count  (POST to Tejas EMS)

URL: https://172.28.7.85:446/nmsnbi-rest/tapi/data/context/physicalcontext/device/equipment/preprovision Method: POST { "preprovision-data": { "serial-number": "TJNW95D01D3F", // ONT Serial (from field engineer) "id": "10", // Unique ONT ID on this PON port "eth-circuitpack-preprovision-data": { "node-edge-point-count": 4 // *** 4 LAN PORTS ENABLED (UNSHUT) *** }, "pon-node-edge-point": "TEJAS-T-FIBER¦172.17.128.34¦1¦1¦11", // OLT IP + PON port "upstream-mapping-type": "VID", "pbit-control": "DISABLED" } }

Step 4 — Enable ETH Port with UNLOCKED state (Bridge Mode)

URL: https://172.28.7.85:446/nmsnbi-rest/tapi/data/context/physical-context/device/ remoteunit/node-edge-point=TEJAS-T-FIBER¦172.17.128.34¦1-1-11-10¦1¦1 Method: PUT { "node-edge-point": { "additional-information": [ { "value-name": "mode", "value": "BRIDGED" } // Bridge Mode ], "administrative-state": "UNLOCKED" // *** PORT UNLOCKED (UNSHUT) *** } }

2.6  Parameter Reference — All Fields That Drive Port Activation

| Parameter | Used In | What It Controls |
| --- | --- | --- |
| oper_NumberOfDataPorts | Nokia AMS SOAP | Number of ONT LAN (Ethernet) ports to ENABLE. Value comes from ONT model's port count in Section 1.4 table. Bridge Mode = 4, Route Mode = 1. |
| ontCardId | Nokia AMS SOAP | Which card slot to use. Slot 1 = physical Ethernet ports (Bridge Mode). Slot 14 = VEIP slot (Route Mode). Varies by ONT model — see 1.4 table. |
| eth-circuitpack node-edge-point-count | Tejas EMS REST | Number of Ethernet ports to provision on Tejas ONT. Same logic as Nokia's oper_NumberOfDataPorts. |
| administrative-state: UNLOCKED | Tejas EMS REST | The Tejas REST API equivalent of port UNSHUT. Setting UNLOCKED activates the port so customer devices can connect. |
| ethPortId | Nokia AMS SOAP | Specifies which specific Ethernet port number gets the service configuration (e.g., Port 1). Multiple services could use different port IDs. |
| veipPortId | Nokia AMS SOAP | VEIP port ID for Route Mode. Default = 1. Used when ontCardId=14. |
| BandwidthProfileUsQueue1 | Nokia AMS SOAP | Upload bandwidth profile applied to the enabled port. E.g., UP_TNET_GI_30M = 30 Mbps upstream. Determines the speed the customer gets. |
| ShaperProfileDsQueue0 | Nokia AMS SOAP | Download shaper profile. E.g., DN_TNET_GI_30M = 30 Mbps downstream. Applied to enabled port. |

2.7  Service Lifecycle — Port States

| Service Event | Port State | How It Happens |
| --- | --- | --- |
| Before NSO provisioning | SHUT / DISABLED | Default state of ONT after physical installation by field engineer. No internet access. |
| NSO Configure (Step 12c/12d) | UNSHUT / ENABLED | NSO sends SOAP/REST call with oper_NumberOfDataPorts or administrative-state=UNLOCKED. AMS/EMS configures OLT → ONT ports come live. Customer can connect. |
| Service Suspension | SUSPENDED | NSO calls Nokia AMS with <ns:suspend> operation using template ORG_SERV_U_D. Ports are suspended — customer cannot access internet but ONT config is retained. |
| Service Resumption | UNSHUT / ENABLED | NSO calls Nokia AMS with <ns:resume> operation. Ports restored to active state. Internet access resumes. |
| Service Disconnection | UNCONFIGURED | NSO calls Nokia AMS with <ns:unconfigure> operation using ORG_SERV_U_D_CVLAN template. ONT service configuration removed completely. Ports shut permanently. |

3.  NSO VLAN Assignment Models

This section defines VLAN allocation models for different service types in NSO (Network Services Orchestrator), ensuring unique S-Tag/C-Tag ranges, scalability per ISP zone, and clarity for provisioning automation.

VLAN Plan Overview

| Types of Services | Service Types | S-Tag | C-Tag | Remarks |
| --- | --- | --- | --- | --- |
| Customer Tag | NA | NA | 1001–2699 |  |
| Enterprise (VAS) | L2VPN | 2700–2999 | NA |  |
| Enterprise (VAS) | L3VPN | NA | 3000–3200 |  |
| HSI (High Speed Internet) | L2VPN Service | 3201–3230 | 1001–1512 | 3 S-TAG for ISP per Zone, 512 C-TAG per OLT |
| OTT | L2VPN Service | 3231–3260 | 1513–2024 | 3 S-TAG for ISP per Zone, 512 C-TAG per OLT |
| VoIP | L2VPN Service | 3261–3290 | 2025–2536 | 3 S-TAG for ISP per Zone, 512 C-TAG per OLT |
| CCTV Surveillance | L3VPN Service (Hub & Spoke) | 3291–3300 |  |  |
| Gov Services (GI) — 98 departments | L2VPN/L3VPN | 3301–3499 | 2537–2699 |  |
| Reserved |  | 3500–4096 |  |  |
| Household | L2VPN | N/A | 3500–3590 | HH PoC Pioneer |
| Household | L2VPN | N/A | 3591–3680 | HH PoC BSNL |
| Household | L2VPN | N/A | 3681–3770 | HH PoC SSC |

Design Principles

- Isolation: Each service type has a dedicated VLAN pool to avoid overlap.

- Scalability: ISP zones provisioned with 3 S-TAGs; each OLT supports 512 C-TAGs.

- Flexibility: Reserved VLANs (3500–4096) for future services.

- Clarity: Household PoC VLANs separated by provider (Pioneer, BSNL, SSC).

Workflow Mapping in NSO

Step 1 — Service Request to NSO

- Identify service type (HSI, OTT, VoIP, etc.).

- Map to VLAN pool based on the table above.

Step 2 — VLAN Assignment Logic

- Allocate next available S-Tag/C-Tag from pool.

- Ensure per-zone ISP allocation rules are enforced.

Step 3 — Provisioning

- Push VLAN assignment to PE/OLT via NSO templates.

- Maintain audit trail in NSO DB for reuse and rollback.

Step 4 — Monitoring

- Track VLAN utilization per service type.

- Trigger alerts when pool utilization >80%.

Example — HSI Service Provisioning

Scenario: Customer request for HSI in Zone A NSO Logic: - Assign S-Tag from 3201–3230 (Zone A ISP pool). - Assign C-Tag from 1001–1512 (OLT pool). Output: VLAN pair (e.g., S-Tag 3205, C-Tag 1010) provisioned on OLT.

4.  IP Pool Assignment Models

For GI-BB

T-Fiber Private IPs

| Field | Value |
| --- | --- |
| IP Block | 10.10.10.0/8 |
| IP Address | 10.10.0.0 |
| Network Address | 10.0.0.0 |
| Usable Host IP Range | 10.0.0.1 — 10.255.255.254 |
| Broadcast Address | 10.255.255.255 |
| Total Number of Hosts | 16,777,216 |
| Number of Usable Hosts | 16,777,214 |
| Subnet Mask | 255.0.0.0 |
| Wildcard Mask | 0.255.255.255 |
| Binary Subnet Mask | 11111111.00000000.00000000.00000000 |
| IP Class | A |
| CIDR Notation | /8 |
| IP Type | Private |

T-Fiber Public IPs

Public IP Blocks

103.162.5.0/24

118.91.190.0/24

202.89.73.0/24

IPv4 pool allocation concept:
When allocating IP addresses manually from a defined pool, the CIDR (Classless Inter-Domain Routing) notation dictates the block size and valid starting points.

- Each subnet must begin at a CIDR-aligned boundary.

- Allocation is based on block size, not on request order.

- This ensures no overlap, no fragmentation, and optimal utilization of the pool.

Base IPv4 Pool

- Pool: 172.16.1.0/24

- Total IPs: 256 (from .0 to .255)

- This pool can be divided into smaller subnets based on CIDR masks.

CIDR Boundary Rules

/30 → Block size = 4 → Valid starts: 0, 4, 8, …
/29 → Block size = 8 → Valid starts: 0, 8, 16, …
/28 → Block size = 16 → Valid starts: 0, 16, 32, …
/27 → Block size = 32 → Valid starts: 0, 32, 64, …
/26 → Block size = 64 → Valid starts: 0, 64, 128, …
/25 → Block size = 128 → Valid starts: 0, 128

These rules prevent misaligned subnets (e.g., you cannot start a /28 at 172.16.1.20 because 20 is not a multiple of 16).

Subnet Allocation Example (from your table)

| Request | CIDR | Subnet Allocated | Address Range |
| --- | --- | --- | --- |
| 1 | /30 | 172.16.1.0/30 | .0 – .3 |
| 2 | /25 | 172.16.1.128/25 | .128 – .255 |
| 3 | /26 | 172.16.1.64/26 | .64 – .127 |
| 4 | /27 | 172.16.1.32/27 | .32 – .63 |
| 5 | /28 | 172.16.1.16/28 | .16 – .31 |
| 6 | /29 | 172.16.1.8/29 | .8 – .15 |
| 7 | /30 | 172.16.1.4/30 | .4 – .7 |

Notice how each subnet starts at the correct boundary and covers exactly its block size.

5.  Traffic Priority for QoS in GPON

Quality of Service (QoS) in the GI-BB GPON network operates at two distinct and separate layers. Both are configured automatically by NSO — no manual intervention is required for any customer provisioning.

| Layer | Where It Applies | What It Does |
| --- | --- | --- |
| Layer 1 | OLT and ONT port | Bandwidth enforcement — upstream policing and downstream shaping per ONT port. This is where the customer's 50 Mbps speed limit is enforced. Done via Nokia AMS SOAP or Tejas EMS REST. |
| Layer 2 | GP/Mandal Router (ASR) | MPLS EXP marking and traffic class prioritisation. GI-BB data traffic is marked EXP=2 in the MPLS core so network routers know which queue to use for this service. Done via NSO NETCONF to ASR 907/920. |

5.1.1  QoS Parameters

| QoS Parameter | Carried In | Purpose |
| --- | --- | --- |
| Bandwidth profile name (e.g., UP_TNET_GI_50M) | Nokia AMS SOAP call | Enforces upstream speed limit (50 Mbps) on the ONT port at the OLT. Profile must pre-exist in OLT. |
| Shaper profile name (e.g., DN_TNET_GI_50M) | Nokia AMS SOAP call | Enforces downstream speed limit (50 Mbps) shaping at the OLT for traffic destined to this ONT. |
| eth-node-edge-point-count (Tejas) | Tejas EMS REST call | Number of ONT LAN ports to enable. Not a speed limit — a port activation count. |
| upstream-bandwidth-profile-uuid (Tejas) | Tejas EMS REST AddUNI | Upload bandwidth profile (e.g., UP_TNET_GI_50M) linked to customer's UNI endpoint. |
| downstream-bandwidth-profile-uuid (Tejas) | Tejas EMS REST AddUNI | Download profile — must be prefixed with 'TejNMS-' for Tejas (e.g., TejNMS-DN_TNET_GI_50M). |
| pbit-value + cir-value + traffic-type | BSS Step 2 → SCCM → NSO → Router | IEEE 802.1p priority and CIR per traffic class (SIGN/CS/DATA/OAM). Used to build router policy-map. |
| shape-value (bps) | BSS Step 2 → SCCM → NSO → Router | Interface-level traffic shaper on the GP/Mandal router. Controls total egress rate on the customer port. |
| MPLS EXP = 2 | NSO → Router (NETCONF) | All GI-BB data traffic entering the MPLS core is marked with EXP bit = 2. This is the global GI-BB QoS marking for the T-Fiber MPLS network. |
| ingress-qos GI_BB | NSO → OLT CLI (one-time) | Ingress QoS profile applied to SVLAN 3301. Maps all dot1p values (0–7) to traffic class 1. Attached to SVLAN during OLT commissioning. |
| outer-pbit-operation: COPY_PBIT | Tejas EMS REST CreateService | Tells Tejas OLT to copy the upstream dot1p priority bits from the customer frame into the outer SVLAN header for QoS treatment in the MPLS core. |

5.2. NSO Pre-configures QoS Infrastructure on OLT

These configurations are pushed by NSO to the Nokia OLT ONE TIME during OLT commissioning — before any customer service is provisioned. They must exist in the OLT, otherwise per-customer SOAP calls will fail with an exception. NSO pushes these via Nokia OLT CLI through the AMS.

'Before proceeding with ONT service provisioning using NBI below basic infrastructures needs to configure in the OLT by pushing CLI template into OLT by NSO, otherwise NBI configure operations will result into exception.'

|  | NSO pushes OLT infrastructure — one time per OLT at commissioning |
| --- | --- |

Step B1 — Upload Bandwidth Profiles (one per speed tier)

NSO pushes each of these CLI commands to the Nokia OLT via the AMS. These define the upstream speed enforcement rules for every speed tier T-Fiber offers.

! 30 Mbps Upload Profile configure qos profiles bandwidth UP_TNET_GI_30M committed-info-rate 0 assured-info-rate 0 excessive-info-rate 30720 ! 50 Mbps Upload Profile ← used for our customer example configure qos profiles bandwidth UP_TNET_GI_50M committed-info-rate 0 assured-info-rate 0 excessive-info-rate 51200 -- excessive-info-rate 51200 = 51200 kbps = 50 Mbps ! 100 Mbps Upload Profile configure qos profiles bandwidth UP_TNET_GI_100M committed-info-rate 0 assured-info-rate 0 excessive-info-rate 102400 ! 500 Mbps Upload Profile configure qos profiles bandwidth UP_TNET_GI_500M committed-info-rate 0 assured-info-rate 0 excessive-info-rate 512000 ! 1 Gbps Upload Profile configure qos profiles bandwidth UP_TNET_GI_1G committed-info-rate 0 assured-info-rate 0 excessive-info-rate 1024000

Step B2 — Download Shaper Profiles (one per speed tier)

Downstream shaping profiles — applied at OLT to rate-limit traffic going TO the customer's ONT.

! 30 Mbps Download Shaper configure qos profiles shaper DN_TNET_GI_30M committed-info-rate 0 excess-info-rate 30720 type singletokenbucketgpon committed-burst-size 0 ! 50 Mbps Download Shaper ← used for our customer example configure qos profiles shaper DN_TNET_GI_50M committed-info-rate 0 excess-info-rate 51200 type singletokenbucketgpon committed-burst-size 0 ! 100 Mbps Download Shaper configure qos profiles shaper DN_TNET_GI_100M committed-info-rate 0 excess-info-rate 102400 type singletokenbucketgpon committed-burst-size 0 ! 500 Mbps Download Shaper configure qos profiles shaper DN_TNET_GI_500M committed-info-rate 0 excess-info-rate 512000 type singletokenbucketgpon committed-burst-size 0 ! 1 Gbps Download Shaper configure qos profiles shaper DN_TNET_GI_1G committed-info-rate 0 excess-info-rate 1024000 type singletokenbucketgpon committed-burst-size 0

Step B3 — Ingress QoS Profile (dot1p → traffic class mapping)

Maps all IEEE 802.1p priority values (0 through 7) to traffic class 1. This means all GI-BB traffic — regardless of what dot1p the customer's device sets — is treated as the same traffic class inside the OLT.

configure qos profiles ingress-qos GI_BB dot1-p0-tc 1 dot1-p1-tc 1 dot1-p2-tc 1 dot1-p3-tc 1 dot1-p4-tc 1 dot1-p5-tc 1 dot1-p6-tc 1 dot1-p7-tc 1 -- All 8 dot1p bits (0-7) → traffic class 1 for GI_BB ingress

Step B4 — SVLAN with Ingress QoS Profile Attached

The SVLAN for GI-BB (3301) is created with the ingress QoS profile attached. Every ONT on this SVLAN automatically gets the GI_BB QoS treatment.

! Create SVLAN 3301 with GI_BB ingress QoS profile configure vlan id 3301 mode residential-bridge name GI-Internet_S3301 in-qos-profname name:GI_BB ← GI_BB ingress QoS profile attached here new-broadcast enable ipv6-mcast-ctrl ! Map SVLAN to OLT uplink — PizzaBox OLT configure bridge port nt:xfp:1 vlan-id 3301 ! Map SVLAN to OLT uplink — Chassis OLT configure service vpls 3301 customer 1 v-vpls vlan 3301 create configure service vpls 3301 description GI_BB_Internet configure service vpls 3301 sap lag-1:3301 create configure service vpls 3301 sap lag-1:3301 no shutdown configure service vpls 3301 sap lt:1/1/1:3301 create configure service vpls 3301 sap lt:1/1/1:3301 no shutdown configure service vpls 3301 no shutdown

Step B5 — Bandwidth Profile Table (Speed → Profile Name Reference)

| Mode | Speed | Upload Profile Name | Download Profile Name |
| --- | --- | --- | --- |
| Bridge Untagged | 30 Mbps | UP_TNET_GI_30M | DN_TNET_GI_30M |
| Bridge Untagged | 50 Mbps ★ | UP_TNET_GI_50M | DN_TNET_GI_50M |
| Bridge Untagged | 100 Mbps | UP_TNET_GI_100M | DN_TNET_GI_100M |
| Bridge Untagged | 500 Mbps | UP_TNET_GI_500M | DN_TNET_GI_500M |
| Bridge Untagged | 1 Gbps | UP_TNET_GI_1G | DN_TNET_GI_1G |
| Bridge Tagged | 50 Mbps | UP_TNET_GI_50M-Tag | DN_TNET_GI_50M |
| Bridge Tagged | 100 Mbps | UP_TNET_GI_100M-Tag | DN_TNET_GI_100M |
| Route Mode | 50 Mbps | UP_TNET_GI_50Mb | DN_TNET_GI_50M |
| Route Mode | 100 Mbps | UP_TNET_GI_100Mb | DN_TNET_GI_100M |

NOTE: ★ = used in the customer example throughout this document. For Tejas EMS downstream profiles, prefix 'TejNMS-' must be added: TejNMS-DN_TNET_GI_50M.

5.3.  — QoS Parameters Flow: Customer → BSS → SCCM → NSO

|  | Customer orders 50 Mbps Internet — QoS flows through BSS Steps 1, 2, 3 to SCCM to NSO |
| --- | --- |

Customer: Government School. Plan: 50 Mbps Internet. Mode: Bridge Untagged. OLT: Nokia. Router: ASR920. This example uses the exact values from the BSS API and GIBB E2E documents.

5.3.1  BSS → SCCM  Step 1 — Service Initiation

Step 1 tells SCCM the service type, product, topology, and SVLAN. No QoS parameters in Step 1. SCCM responds with a transaction_id used in Steps 2 and 3.

| POST | https://172.28.2.101/sccm/sccm_api/web/api/v1/servicedeploy/step1 |
| --- | --- |

REQUEST: { "service_order_id": "ORD000355", -- BSS order number "customer_id": "CUST000023", -- Customer in SCCM "sub_service_id": "9.5.2.1", -- Nokia OLT + ASR920 + Bridge Untagged "endpoint_device_count": 1, "topology": 5, -- Hub & Spoke L2VPN topology "location_id": "2", "service_vlan_id": "3301" -- SVLAN for GI-BB } RESPONSE: { "status_code": 2001, -- 2001 = Step 1 success "message": "Service Invoke Create Step1 Success", "data": { "transaction_id": 2274 } -- Used in Step 2 and Step 3 }

| Field | Value | What SCCM Does With It |
| --- | --- | --- |
| sub_service_id | 9.5.2.1 | Identifies: Nokia OLT → ASR920 → Bridge Mode Untagged. SCCM selects template ORG_SERV_U_D_CVLAN for Nokia AMS SOAP call. |
| service_vlan_id | 3301 | GI-BB SVLAN. Used in Nokia AMS networkVlanName and Tejas EMS service creation. |
| topology | 5 | Hub & Spoke L2VPN. SCCM will configure L2VFI + pseudowire on router pointing to HUB ZCR. |

5.3.2  BSS → SCCM  Step 2 — Device and QoS Parameters

Step 2 is where ALL QoS parameters are passed from BSS to SCCM. This includes per-traffic-class pbit and CIR values, the hub-spoke router topology, interface shaper, and customer VLANs. SCCM uses this data to build the complete NSO YANG payload.

| POST | https://172.28.2.101/sccm/sccm_api/web/api/v1/servicedeploy/step2 |
| --- | --- |

REQUEST: { "transaction_id": 2274, -- from Step 1 response "endpoint_devices": [ { "pe_host_name": "NOCTESTGAC001", -- GP Router hostname (ASR920) "service_specific_id": "ORD000355", "qos-config": [ { "pbit-value": "7", -- IEEE 802.1p priority 7 = highest "cir-value": "20", -- 20 Mbps CIR for this class "traffic-type": "SIGN" -- Signalling (SIP, control) }, { "pbit-value": "6", -- Priority 6 "cir-value": "20", -- 20 Mbps "traffic-type": "CS" -- Circuit-switched / voice }, { "pbit-value": "5", -- Priority 5 "cir-value": "20", -- 20 Mbps "traffic-type": "DATA" -- Internet data (HTTP, streaming) }, { "pbit-value": "4", -- Priority 4 "cir-value": "40", -- 40 Mbps "traffic-type": "OAM" -- Operations & Management traffic } ], "parameters": { "hub-hostname": "NOC507RTRN00001", -- ZCR Hub router to connect to "interface-id": "Gig0/0/8", -- Physical port on GP router "shape-value": "200000000", -- 200 Mbps interface shaper (bps) "cir-value": "200000000", -- 200 Mbps CIR on interface "port-count": 1, "customer-vlans": "2542" -- Customer CVLAN } } ] } RESPONSE: { "status_code": 2002, -- 2002 = Step 2 success "message": "Service Invoke Create Step2 Success", "data": { "transaction_id": 2274 } }

Step 2 QoS Field Explanation

| Field | Value | Traffic Type | Meaning and How SCCM Uses It |
| --- | --- | --- | --- |
| pbit-value=7, cir=20 | 7 | SIGN | Highest priority (7). Signalling traffic. SCCM maps this to a router policy-map class with 20 Mbps guaranteed rate. Priority queued in MPLS core. |
| pbit-value=6, cir=20 | 6 | CS | Second highest. Circuit-switched or voice. 20 Mbps guaranteed. Real-time traffic class. |
| pbit-value=5, cir=20 | 5 | DATA | Internet data — web browsing, video streaming, downloads. 20 Mbps guaranteed. Marked MPLS EXP=2 in core. This is the customer's internet service traffic. |
| pbit-value=4, cir=40 | 4 | OAM | Operations and Management. 40 Mbps guaranteed. Management plane traffic (SNMP, TR-069, OAM probes). |
| shape-value | 200000000 | All | 200 Mbps total interface shaper on the GP router's customer-facing port. This is the maximum aggregate rate across all traffic classes. |
| hub-hostname | NOC507RTRN00001 | — | ZCR Hub router. SCCM uses this to configure the L2VFI neighbor (pseudowire endpoint) on the GP router. |

5.3.3  BSS → SCCM  Step 3 — Commit and Execute

Step 3 commits the transaction. SCCM takes all data from Steps 1 and 2, builds the complete NSO YANG service model, and sends it to NSO for execution. NSO then pushes configurations to all devices simultaneously.

| POST | https://172.28.2.101/sccm/sccm_api/web/api/v1/servicedeploy/step3 |
| --- | --- |

REQUEST: { "transaction_id": 2274, "action": 1 -- action=1 means commit and push to network } RESPONSE: { "status_code": 2000, -- 2000 = Full success "message": "Service Invoke Create Step3 Success", "data": { "unique_id": "CKT-GI-BB-SCCM-S000754" -- Circuit ID assigned to this service } }

NOTE: After Step 3 is sent, NSO starts pushing configurations to all devices. BSS does NOT get an immediate synchronous response about device configuration status. BSS waits for the SCCM callback (Step 9 in the sequence) which arrives after NSO confirms all devices are configured.

5.4  — NSO Pushes QoS to OLT (Nokia AMS SOAP / Tejas EMS REST)

| NSO → OLT | NSO references pre-built QoS profiles and links them to the customer's ONT port |
| --- | --- |

NSO does NOT re-create the bandwidth profiles for each customer. The profiles (UP_TNET_GI_50M, DN_TNET_GI_50M) were created once during OLT commissioning (Section 2). NSO only references these profile names in the per-customer provisioning call.

5.4.1  Nokia AMS — SOAP Call with QoS Profiles (Bridge Mode 50 Mbps)

ontEthPortQosProfiles_BandwidthProfileUsQueue1 and ontEthPortQosProfiles_ShaperProfileDsQueue0 are the QoS parameters in the Nokia AMS SOAP configure call.

| SOAP | http://172.28.39.12:8080/soap/services/ApcRemotePort/9.7 |
| --- | --- |

<ns:configure> <objectName>NOCTESTOLTN001:1-1-1-2-10</objectName> <!-- OLT Hostname : PON Port (4 digits) : ONT ID (5th digit) --> <templateName>ORG_SERV_U_D_CVLAN</templateName> <templateVersion>2</templateVersion> <!-- ONT identification --> <argument><n>ontSerialNumber</n><value>TJNW957F5907</value></argument> <argument><n>ontCardId</n><value>1</value></argument> <argument><n>oper_NumberOfDataPorts</n><value>4</value></argument> <argument><n>ethPortId</n><value>1</value></argument> <!-- ▼▼ QoS PARAMETERS — links 50 Mbps profiles to this ONT port ▼▼ --> <argument> <n>ontEthPortQosProfiles_BandwidthProfileUsQueue1</n> <value>UP_TNET_GI_50M</value> <!-- Upload: OLT will police upstream traffic from this ONT to 50 Mbps --> </argument> <argument> <n>ontEthPortQosProfiles_ShaperProfileDsQueue0</n> <value>DN_TNET_GI_50M</value> <!-- Download: OLT will shape downstream traffic to this ONT to 50 Mbps --> </argument> <!-- ▲▲ QoS PARAMETERS END ▲▲ --> <!-- VLAN parameters --> <argument><n>vlanCustomerId</n><value>2542</value></argument> <argument><n>networkVlanName</n><value>GI-Internet_S3301</value></argument> <argument><n>ontSubscriberId2</n><value>TF-S000355-GIBB_2542-50Mbps</value></argument> </ns:configure>

What Nokia AMS Does After Receiving This SOAP Call

| Action | QoS Effect |
| --- | --- |
| Registers ONT TJNW957F5907 on PON port 1-1-1-2 | ONT is now known to the OLT. Ready for traffic. |
| Links UP_TNET_GI_50M to ONT Eth port | OLT enforces 50 Mbps upstream police. If customer sends more than 51200 kbps, OLT drops excess frames. |
| Links DN_TNET_GI_50M shaper to ONT port | OLT shapes downstream to 50 Mbps. Customer cannot receive more than 51200 kbps download. |
| Applies CVLAN 2542 to Eth port 1 | All frames from this ONT are tagged with CVLAN 2542 at the OLT. |
| SVLAN 3301 already has GI_BB ingress QoS attached | All dot1p values from customer device are mapped to traffic class 1 at OLT ingress. SVLAN 3301 carries traffic to the core. |

5.4.2  Nokia AMS — SOAP Call for Route Mode (VEIP — 50 Mbps)

For Route Mode, the QoS parameters are identical in value but the parameter name prefix changes from ontEthPortQosProfiles_ to ontVeipPortQosProfiles_ because the WAN interface is the VEIP port (slot 14), not the physical Ethernet port.

<ns:configure> <templateName>TEMPLATE-GI-BB_ROUTE</templateName> <argument><n>ontCardId</n><value>14</value></argument> <argument><n>oper_NumberOfDataPorts</n><value>1</value></argument> <argument><n>veipPortId</n><value>1</value></argument> <!-- QoS on VEIP WAN interface — same profile names, different prefix --> <argument> <n>ontVeipPortQosProfiles_BandwidthProfileUsQueue1</n> <value>UP_TNET_GI_50M</value> <!-- Upload: 50 Mbps on VEIP WAN port — same profile, VEIP-specific parameter --> </argument> <argument> <n>ontVeipPortQosProfiles_ShaperProfileDsQueue0</n> <value>DN_TNET_GI_50M</value> <!-- Download: 50 Mbps shaper on VEIP WAN port --> </argument> </ns:configure>

5.4.3  Tejas EMS — REST API QoS Parameters (Bridge Mode)

For Tejas ONTs, QoS bandwidth profiles are passed in Step 6 (Add UNI) of the 8-step sequence. The profile names follow the same naming convention but downstream must be prefixed with 'TejNMS-'.

| POST | https://172.28.7.85:446/nmsnbi-rest/tapi/data/context/connectivitycontext/connectivity-service/connectivityserviceEndPoint?activate=true |
| --- | --- |

REQUEST (Add UNI — Step 6): [{ "uuid": "ebd022b9-1bef-4e28-9e26-f1f23c22df98", -- Service UUID from Step 5 "end-point": [{ "name": [ { "value-name": "SerialNumber", "value": "TJNW95D01D3F" }, { "value-name": "ONTEthPortInstance","value": "1" } ], "layer-protocol-name": "ETH", "nrp-carrier-eth-connectivity-end-point-resource": { "ce-vlan-id-list-and-untag": { "vlan-id": [{ "vlan-id": 2542 }], -- Customer CVLAN "untagged-and-prio-tagged-included": true }, "cos-identifier-list": [{ "cos-name": "0" }] }, -- ▼▼ QoS PARAMETERS FOR TEJAS OLT ▼▼ "upstream-bandwidth-profile": { "upstream-bandwidth-profile-uuid": "UP_TNET_GI_50M" -- Upload: 50 Mbps profile applied to this customer's UNI }, "downstream-bandwidth-profile": { "downstream-bandwidth-profile-uuid": "TejNMS-DN_TNET_GI_50M" -- Download: MUST prefix 'TejNMS-' for Tejas downstream profiles }, -- ▲▲ QoS PARAMETERS END ▲▲ "additional-information": [ { "value-name": "outer-pbit-operation", "value": "COPY_PBIT" }, -- Copies upstream dot1p priority bits into SVLAN header -- This preserves traffic class info in the MPLS core { "value-name": "ingress-operation", "value": "Swap" }, { "value-name": "ingress-vlan1", "value": "3301" } -- Swaps CVLAN 2542 to SVLAN 3301 at OLT uplink ] }] }] RESPONSE: { "statusCode": 200, "msg": "ConnectivityService EndPoint added and activated Successfully" }

NOTE: COPY_PBIT — this is a critical QoS parameter for Tejas. It tells the Tejas OLT to copy the IEEE 802.1p priority bits from the customer's frame into the outer SVLAN header. Without this, the MPLS core routers would not know the traffic priority and could not enforce QoS in the network core.

5.5  — NSO Pushes QoS Policy to Router (NETCONF / CLI)

| NSO → Router | NSO pushes MPLS EXP marking and traffic class policy-map to GP/Mandal router (ASR 907/920) |
| --- | --- |

NSO pushes the router configuration via NETCONF to the GP Router (ASR920) at the customer's location. The router is the entry point into the T-Fiber MPLS core. This is where traffic is marked with MPLS EXP bits so that core routers know how to prioritise it.

5.5.1  Full Router Configuration Pushed by NSO

! ============================================================ ! L2VFI Configuration — Creates the Hub & Spoke L2VPN service ! ============================================================ l2 vfi GIBB_ORD000355 manual vpn id 3301 -- VPN identifier for GI-BB service bridge-domain 3301 mtu 9100 -- Jumbo frame support neighbor 172.17.236.1 encapsulation mpls -- Pseudowire to HUB ZCR router (NOC507RTRN00001) ! ============================================================ ! Service Instance — Maps customer CVLAN to bridge domain ! ============================================================ interface GigabitEthernet0/0/8 service instance 3301 ethernet encapsulation dot1q 2542 -- Customer CVLAN 2542 rewrite ingress tag push dot1q 3301 symmetric -- Tags upstream frame with SVLAN 3301 bridge-domain 3301 ! ============================================================ ! QoS Policy — MPLS EXP marking + policing per traffic class ! ============================================================ policy-map GIBB-IN class class-default set qos-group 2 set mpls experimental imposition 2 -- ALL GI-BB data marked EXP=2 police cir 200000000 bc <BC_VALUE> conform-action transmit exceed-action drop -- Traffic above 200 Mbps CIR dropped interface GigabitEthernet0/0/8 service-policy input GIBB-IN -- Apply QoS inbound on customer port

5.5.2  Traffic Class to MPLS EXP Mapping — GI-BB

— Logical Inventory: 'MPLS EXP (QoS) = 2 — Data queue marking for GI-BB traffic in MPLS network.'

| Traffic Type | dot1p (pbit) | CIR (Mbps) | MPLS EXP | Router Queue | Meaning |
| --- | --- | --- | --- | --- | --- |
| SIGN — Signalling | 7 (highest) | 20 Mbps | 7 | Priority / Strict | SIP, H.323, network control. Highest priority. Guaranteed delivery. |
| CS — Circuit Switched | 6 | 20 Mbps | 6 | Expedited | Voice calls. Second highest. Real-time traffic, delay-sensitive. |
| DATA — Internet | 5 | 20 Mbps | 2 ★ | Data / Best Effort | HTTP, HTTPS, streaming, file downloads. Customer internet traffic. Marked EXP=2 in core. |
| OAM — Management | 4 | 40 Mbps | 4 | Management | SNMP, TR-069, OAM probes, management plane. 40 Mbps guaranteed. |

NOTE: ★ MPLS EXP=2 is the fixed marking for all GI-BB DATA traffic in the T-Fiber MPLS core. This is set by the router policy-map GIBB-IN applied by NSO. All data traffic — regardless of what the customer's device marks — exits the GP router with EXP=2.

5.5.3  Hub ZCR Router Configuration (NSO also pushes this)

NSO also configures the Hub router (ZCR — ASR 9000) to accept the pseudowire from this new customer's spoke router. The hub router terminates all spoke pseudowires for GI-BB.

! Hub Router — ZCR (ASR 9000) — NOC507RTRN00001 L2VPN bridge group GIBB bridge-domain 3301 mtu 9100 interface Bundle-Ether15.3301 ! vfi GIBB_HUB neighbor <SPOKE1_LOOPBACK> pw-id 3301 pw-class PW1 neighbor <SPOKE2_LOOPBACK> pw-id 3301 pw-class PW1 -- New spoke (this customer's GP router) added here neighbor <NEW_GP_ROUTER_LOOPBACK> pw-id 3301 pw-class PW1

5.6  — SCCM Callback to BSS and Live QoS State

| AFTER | NSO confirms all devices configured — SCCM sends callback to BSS — service is ACTIVE |
| --- | --- |

5.6.1  SCCM Callback to BSS

After NSO successfully pushes configuration to all devices (router and OLT), SCCM sends a callback notification to BSS. BSS marks the customer ACTIVE and triggers billing.

| POST | https://172.28.10.9:30098/rest/thirdparty/notification/servicecompletion |
| --- | --- |

REQUEST (SCCM → BSS): { "service_order_id": "ORD000355", "unique_id": "CKT-GI-BB-SCCM-S000754", -- Circuit ID "status_code": 2000, -- 2000 = Full success "deviceid": "1241,1242", -- Device IDs configured "playbook_status": "NA" -- Direct NSO push, no Ansible } BSS Response: Service Completion Notification Received Successfully

5.6.2  QoS State After Successful Provisioning

| Where | QoS Active State | Enforcement Mechanism |
| --- | --- | --- |
| Nokia OLT — Upstream (from ONT) | 50 Mbps hard limit. Frames above 51200 kbps dropped at OLT. | UP_TNET_GI_50M bandwidth profile policing on ONT Eth port. |
| Nokia OLT — Downstream (to ONT) | 50 Mbps shaped. Customer cannot receive more than 51200 kbps. | DN_TNET_GI_50M shaper profile on ONT Eth port. |
| Tejas OLT — Upstream | 50 Mbps hard limit per UNI (customer port). | UP_TNET_GI_50M profile in AddUNI call. |
| Tejas OLT — Downstream | 50 Mbps shaped. | TejNMS-DN_TNET_GI_50M profile in AddUNI call. |
| GP Router — Ingress | All frames entering from customer port marked MPLS EXP=2. Policed at 200 Mbps CIR. | policy-map GIBB-IN applied inbound on Gig0/0/8. |
| MPLS Core — All Core Routers | GI-BB data traffic forwarded in EXP=2 queue. Lower priority than EXP>2 traffic. | MPLS EXP=2 marking set by GP router policy-map. |
| Hub ZCR Router — Egress | Traffic from all spokes aggregated. Forwarded toward BNG/internet. | L2VPN bridge-domain 3301 terminates all pseudowires. |

5.6.3  Complete API Sequence Summary — QoS Perspective

| # | From | To | API | QoS Data Carried |
| --- | --- | --- | --- | --- |
| 1 | BSS | SCCM | Step 1 | sub_service_id=9.5.2.1 → tells SCCM which OLT type and mode → selects correct AMS template. |
| 2 | BSS | SCCM | Step 2 | qos-config[pbit+cir+traffic-type], shape-value, hub-hostname, customer-vlans. ALL QoS parameters for router. |
| 3 | BSS | SCCM | Step 3 | action=1 — triggers NSO execution. No QoS data, just commit signal. |
| 4 | SCCM | NSO | Internal | NSO YANG model built by SCCM containing all service parameters including bandwidth profile names and router policy parameters. |
| 5 | NSO | Nokia AMS | SOAP POST | ontEthPortQosProfiles_BandwidthProfileUsQueue1=UP_TNET_GI_50M, ShaperProfileDsQueue0=DN_TNET_GI_50M — links 50 Mbps QoS to ONT port. |
| 5 | NSO | Tejas EMS | REST POST | upstream-bandwidth-profile-uuid=UP_TNET_GI_50M, downstream-bandwidth-profile-uuid=TejNMS-DN_TNET_GI_50M, outer-pbit-operation=COPY_PBIT. |
| 6 | NSO | ASR 907/920 | NETCONF | policy-map GIBB-IN: set mpls experimental imposition 2, police cir 200000000, pbit class entries (SIGN/CS/DATA/OAM). |
| 7 | NSO | ZCR Hub | NETCONF | Adds new spoke pseudowire neighbor to Hub's L2VPN bridge-domain 3301. |
| 8 | SCCM | BSS | Callback | status_code=2000, unique_id=CKT-GI-BB-SCCM-S000754. BSS marks customer ACTIVE. QoS is live. |

6.  GI ILL — TDIA Service Configuration

T-DIA ILL is a dedicated internet access service offering for Enterprise and Government Institutions, delivered over T-FIBER's IP/MPLS backbone with GPON and fiber last-mile connectivity.

# 6.1. Network Architecture Overview

The T-FIBER network is a greenfield IP/MPLS based architecture deployed across Telangana under the Bharatnet initiative. The core is built on Cisco routers with Nokia and Tejas GPON as the access layer. End-to-end reachability is achieved using BGP Label Unicast (BGP-LU).

## 6.1.2 Network Hierarchy

| Layer | Node Type | Platform | Role |
| --- | --- | --- | --- |
| Core | Zonal Core Router (ZCR) | Cisco ASR9K | MPLS Core, Route Reflector, ILL HUB, ISP Peering |
| Aggregation | Mandal Router (MAR) | Cisco ASR9K | Mandal Ring aggregation, MPLS P-router |
| Access | GP Router | Cisco ASR920/907 | L3VPN PE Spoke, Customer GW |
| OLT | GPON OLT | Nokia / Tejas | Passive Optical Line Termination |
| Customer | ONT / CPE | Nokia ONT / Customer Router | Last-mile termination at customer premises |

## 6.1.3 T-DIA ILL Topology (HUB & Spoke L3VPN)

The ILL service uses a HUB & Spoke L3VPN topology where:

- Each Enterprise/Government customer gets one L3VPN Spoke on the GP/Mandal router

- A single HUB exists at the Route Reflector ZCR (or one HUB per Zonal Router)

- The ISP upstream link terminates on the ZCR

- Traffic flows: Customer CPE → ONT → OLT → GP Router (Spoke PE) → Mandal Ring → ZCR (HUB) → ISP → Internet

## 6.1.4 Route Targets and Route Distinguishers

| RT/RD | Value | Node | Description |
| --- | --- | --- | --- |
| RT Export (Spoke) | 141688:33 | GP/Mandal Router | Customer routes exported from Spoke PE |
| RT Import (Spoke) | 141688:34 | GP/Mandal Router | Spoke imports default route from HUB |
| RT Export (HUB) | 141688:34 | ZCR | HUB exports default/ISP routes to all Spokes |
| RT Import (HUB) | 141688:33 | ZCR | HUB imports all Spoke customer prefixes |
| RD (Spoke) | Router-ID:XX | GP/Mandal Router | Unique RD per customer VRF instance |
| RD (HUB PoC) | 172.17.236.1:6 | ZCR (ASR9K) | HUB VRF RD used in lab verification |

## 6.1.5 VLAN Plan

| Service Type | S-Tag Range | C-Tag | Remarks |
| --- | --- | --- | --- |
| Enterprise L3VPN (T-DIA ILL) | 3000 – 3200 | N/A | GPON last mile; local to router |
| GI L3VPN (T-DIA ILL) | 3301 – 3499 | N/A | Government Institutions |
| Nokia OLT C-tag (ONT side) | 2000 (pvid) | 2700–2800 | ONT port PVID; swapped to S-tag at NNI |
| Infrastructure VLAN | 2 – 1000 | N/A | Management and infrastructure |
| Customer Tag | N/A | 1001–2699 | Customer VLAN space |
| Reserved | 3500 – 4096 | N/A | Future use |

Note: VLANs are significant only locally to the router (L3VPN) and can be reused across different GP/Mandal/Zonal routers if they are free on that specific router.

# 6.2. End-to-End Provisioning Flow

The T-DIA ILL service provisioning is fully automated via the SCCM + NSO stack. The flow covers seven stages from customer order intake to service verification.

## Phase 1: Order Intake & Feasibility (BSS/OSS)

| Step | System | Action |
| --- | --- | --- |
| 1 | Customer / BSS | Enterprise or Government customer raises an ILL service request. Bandwidth tier, last-mile type, CPE type, and public IP requirement are specified. |
| 2 | BSS | BSS receives and validates the order. BSS holds the billing system and communicates to SCCM-NSO via North Bound REST APIs. |
| 3 | OSS | OSS performs feasibility check using customer Lat/Long coordinates against device inventory. OSS maintains device inventory and service inventory for all network elements. |
| 4 | BSS → SCCM | Validated order handed to SCCM via North Bound API. OSS also connects to SCCM with device/inventory data. |

## Phase 2: SCCM Orchestration

SCCM (Service Configuration and Compliance Manager) is the middleware between OSS/BSS and NSO. It enforces service logic, input validation, topology discovery, and southbound API to NSO.

### 2a. Input Variable Assembly

SCCM combines User Defined (UD) parameters (provided by the operator) with System Defined (SD) parameters that SCCM derives automatically:

| Variable | UD/SD | Value / Description |
| --- | --- | --- |
| VRF-name | SD | TDIA-ILL |
| rt-export / rt-import | SD | 141688:33 / 141688:34 |
| internet-vlan (S-tag) | SD | 3000–3200 (Enterprise) ¦ 3301–3499 (GI) |
| BDI interface number | SD | Auto-assigned (e.g. BDI3302) |
| ipv4-wan-address | SD | /30 or /29+ from TFiber public pool |
| routing-protocol | SD/UD | connected ¦ static ¦ eBGP |
| qos-name | SD | TDIA-ILL-IN / TDIA-ILL-OUT |
| mtu | SD | 9100 (IOS-XE) ¦ 9216 (IOS-XR) |
| ont-serial-number | UD | Provided by operator during provisioning |
| ds-bandwidth-profile | UD | Nokia: UP_100M ¦ Tejas: 102400 kbps |
| cust-as-no / remote-as-no | UD | Required for eBGP PE-CE scenario only |
| bfd-required | UD | Optional – bidirectional forwarding detection |

### 2b. Topology Discovery & Service Package Selection

SCCM identifies the full end-to-end topology based on the endpoint device, then selects the appropriate NSO service package:

| NSO Service Package | Scenario |
| --- | --- |
| TDIA-DIRECT-SINGLE-ASR920 | Fiber last mile directly on ASR920 (single port) |
| TDIA-DIRECT-SINGLE-ASR907 | Fiber last mile directly on ASR907 (single port) |
| TDIA-DIRECT-SINGLE-ASR9K | Fiber last mile directly on ASR9K (single port) |
| TDIA-NOKIA-SINGLE-ASR920 | Nokia GPON OLT + ASR920 GP Router |
| TDIA-NOKIA-SINGLE-ASR907 | Nokia GPON OLT + ASR907 GP Router |
| TDIA-NOKIA-SINGLE-ASR9K | Nokia GPON OLT + ASR9K MAG/ZCR |
| TDIA-TEJAS-SINGLE-ASR920 | Tejas GPON OLT + ASR920 GP Router |
| TDIA-TEJAS-SINGLE-ASR907 | Tejas GPON OLT + ASR907 GP Router |
| TDIA-TEJAS-SINGLE-ASR9K | Tejas GPON OLT + ASR9K MAG/ZCR |
| TDIA-DIRECT-ASR920-MULTIPORT | Dual fiber LAG on ASR920 (redundant last mile) |
| TDIA-DIRECT-ASR907-MULTIPORT | Dual fiber LAG on ASR907 (redundant last mile) |
| TDIA-DIRECT-ASR9K-MULTIPORT | Dual fiber LAG on ASR9K (redundant last mile) |

### 2c. SCCM → NSO API & Provisioning Flow

- SCCM constructs the NSO service payload and sends via RESTCONF (ports 8080/8888) or CLI (port 4569)

- NSO checks CDB synchronisation with devices – if out of sync, performs sync-from first

- NSO computes device-specific configs and pushes atomically (all-or-nothing transaction)

- Flow A (Success): NSO confirms push → SCCM triggers Playbook post-check → verification PDF generated → BSS notified

- Flow B (Failure): If any device fails, NSO rolls back all changes across all devices → SCCM reports failure to user

Important – Out-of-Band Configuration Warning Any configuration applied directly to devices via CLI (outside NSO) is NOT visible to NSO's service model until explicitly reconciled. If NSO's CDB and device configuration differ, dry-run outputs during subsequent MACD operations will not be as expected.

# 6.3. NSO Solution Design

## 6.3.1 NSO Architecture

Cisco NSO (Network Service Orchestrator) has two main integrated layers:

- Service Manager: Manages high-level service definitions. Uses YANG service model defining T-DIA ILL attributes. Service-to-device mapping is implemented via Python/Java templates.

- Device Manager: Manages device configurations transactionally. Uses Network Element Drivers (NEDs) modelled in YANG for each device type. NEDs support CLI, NETCONF, and REST-based device interfaces.

## 6.3.2 NSO Connectivity Details

| System | VIP | Active / Standby | Ports |
| --- | --- | --- | --- |
| NSO (Primary) | 172.28.2.150 | 172.28.2.109 / 172.28.2.118 | 4569 (CLI), 4570 (HA), 8080 (RESTCONF), 8888 (SSL), 2022-23 |
| NSO (Secondary) | 172.28.2.133 | 172.28.2.110 / 172.28.2.119 | Same as above |
| NSO (Tertiary) | 172.28.105.120 | 172.28.2.111 / 172.28.2.120 | Same as above |
| Nokia AMS PKG-A | 172.28.39.11 / .12 | – | 8443 (HTTPS) |
| Nokia AMS PKG-B | 172.28.43.71 / .73 | – | 8443 (HTTPS) |
| Nokia AMS PKG-C | 172.28.43.101 / .103 | – | 8443 (HTTPS) |
| Tejas EMS PKG-A | 172.28.7.85 | – | 446 (HTTPS) |
| Tejas EMS PKG-B | 172.28.12.215 | – | 446 (HTTPS) |
| Tejas EMS PKG-C | 172.28.13.215 | – | 446 (HTTPS) |

## 8.3.3 NSO Configuration Pushed – Cisco Router (IOS-XR HUB / ZCR)

### HUB VRF Configuration

vrf TDIA-ILL

address-family ipv4 unicast

import route-target 141688:33

export route-target 141688:34

address-family ipv6 unicast

import route-target 141688:33

export route-target 141688:34

### HUB Upstream Interface (to ISP)

interface Bundle-EtherXX

description # TO UPSTREAM ISP – ILL HUB #

vrf TDIA-ILL

mtu 9216  |  ipv4 mtu 9216

service-policy input TDIA-ILL-IN

ipv4 address X.X.X.X X.X.X.X   ! As per WAN IP from upstream ISP

bundle minimum-active links 1  |  load-interval 30  |  dampening

### HUB BGP Configuration (eBGP to ISP)

router bgp 141688

vrf INTERNET

rd X.X.X.X:X

default-information originate

address-family ipv4 unicast

aggregate-address 103.162.4.0/23   ! TFiber public IP range

redistribute connected | redistribute static

neighbor 42.104.117.194              ! Vodafone ISP

remote-as 55410

ebgp-multihop 255  |  update-source Loopback50

route-policy TDIA-ILL-IN in  |  route-policy TDIA-ILL-OUT out

remove-private-AS

### HUB Route Policies

prefix-set TDIA-ILL_DEFAULT-IN

0.0.0.0/0 eq 32          ! Accept default route from ISP

end-set

route-policy TDIA-ILL-IN

if destination in TDIA-ILL_DEFAULT-IN then pass

if destination in TDIA-ILL_ALLOW-IN then pass

else drop

end-policy

route-policy TDIA-ILL-OUT

if destination TDIA-ILL_PREFIX_OUT in then pass   ! TFiber public prefixes

if as-path in TDIA-ILL_ASPATH_OUT then pass       ! Customer ASN prefixes

else drop

end-policy

### HUB QoS Policy

policy-map TDIA-ILL-IN

class class-default

set qos-group 2

set mpls experimental imposition 2

## 8.3.4 NSO Configuration Pushed – Cisco Router (IOS-XE Spoke / GP Router)

### Spoke VRF Configuration

vrf definition TDIA-ILL

description *** For TDIA-INTERNET-ILL ***

rd X.X.X.X:XX

address-family ipv4

route-target export 141688:33

route-target import 141688:34

address-family ipv6

route-target export 141688:33

route-target import 141688:34

### Physical Interface → Bridge Domain

interface GigabitEthernet0/0/6

mtu 9100  |  no ip address  |  no negotiation auto

service instance 3302 ethernet

description ** INTERNET_ILL-TESTING **

encapsulation dot1q 3302

rewrite ingress tag pop 1 symmetric

bridge-domain 3302

### BDI Interface (L3 Termination)

interface BDI3302

vrf forwarding TDIA-ILL

ip address X.X.X.X Y.Y.Y.Y   ! /30 or /29+ public IP

service-policy input TDIA-ILL-IN

arp timeout 30

### Spoke BGP Configuration

router bgp 141688

address-family ipv4 vrf TDIA-ILL

redistribute connected

redistribute static

### Spoke QoS Policy

Policy-map TDIA-ILL-IN

Class default

set qos-group 2

set mpls experimental imposition 2

# 6.4. GPON Last-Mile Configuration

## 6.4.1 Nokia OLT Configuration

### QoS Profiles

configure qos profiles ingress-qos HSI dot1-p0-tc 2 ... dot1-p7-tc 2

configure qos profiles bandwidth UP_100M committed-info-rate 0 assured-info-rate 0 excessive-info-rate 102400 delay-tolerance 1

configure qos profiles shaper DN_100M committed-info-rate 0 excess-info-rate 102400 type singletokenbucketgpon committed-burst-size 0

### VLAN & Bridge Configuration (Pizza Box OLT)

configure vlan id 3302 mode residential-bridge name L3VPN_S3302 in-qos-prof-name name:HSI new-broadcast enable

configure qos interface 1/1/1/1/1/1/1 upstream-queue 2 bandwidth-profile name:UP_100M bandwidth-sharing uni-sharing

configure qos interface 1/1/1/1/1/1/1 queue 1 shaper-profile name:DN_100M

configure bridge port 1/1/1/1/1/1/1 vlan-id 2000 network-vlan 3302 vlan-scope local

configure bridge port 1/1/1/1/1/1/1 pvid 2000

configure interface port uni:1/1/1/1/1/1/1 admin-up

configure bridge port nt:xfp:1 vlan-id 3302

### Nokia Chassis OLT – Uplink VPLS Configuration

configure service vpls 3302 customer 1 v-vpls vlan 3302 create

configure service vpls 3302 description L3VPN

configure service vpls 3302 sap lag-1:3302 create

configure service vpls 3302 sap lag-1:3302 no shutdown

configure service vpls 3302 sap lt:1/1/1:3302 create

configure service vpls 3302 sap lt:1/1/1:3302 no shutdown

configure service vpls 3302 no shutdown

## 6.4.2 Tejas OLT Configuration

### Service Creation (via Tejas EMS)

| Parameter | Value |
| --- | --- |
| Service Name | L3VPN-V3302 |
| Service Type | ELAN |
| Forwarding Type | 1q(CVLAN) |
| GPON Profile | GpoNProFile-L3VPN |
| Input ONT Tag | UNTAGGED |
| Outer VLAN Range (C-tag) | 2700 – 2800 |
| Network S-tag | 3302 |
| VT Operation (NNI) | Swap C-tag 2700 → S-tag 3302 |
| QoS Bandwidth | Assured = 102400 kbps ¦ Maximum = 102400 kbps |

# 6.5. PE-CE Routing Scenarios

Six deployment scenarios are supported for T-DIA ILL, driven by customer CPE capability and last-mile type. NSO automatically selects the applicable configuration template via SCCM inputs.

## Scenario 1 – GP/Mandal Router as GW for Customer LAN

Last mile: GPON (non-WiFi ONT in bridge mode). The BDI interface on GP/Mandal router gets a /30 or /29+ public IP subnet. This IP acts as the default gateway for all customer LAN devices.

interface BDI3302

vrf forwarding TDIA-ILL

ip address X.X.X.X Y.Y.Y.Y   ! /30 or /29+ public subnet

service-policy input TDIA-ILL-IN

## Scenario 2 – Static Routing: PE to Customer CPE

Customer has an L3 switch or router behind the ONT. BDI gets a /30 WAN IP. A static route is configured on the PE toward the customer's LAN via the CPE WAN IP.

interface BDI3302

vrf forwarding TDIA-ILL

ip address X.X.X.X 255.255.255.252   ! /30 WAN IP

service-policy input TDIA-ILL-IN

ip route vrf TDIA-ILL Y.Y.Y.Y 255.255.255.X CPE-WAN-IP   ! Customer LAN static route

## Scenario 3 – WiFi ONT (Router Mode) as NAT+GW

WiFi ONT operates in router mode. One public IP from /30 is assigned to the WiFi ONT WAN interface. The other /30 IP is assigned to the GP/Mandal router BDI (acting as GW for the WiFi ONT).

interface BDI3302

vrf forwarding TDIA-ILL

ip address X.X.X.X 255.255.255.252   ! /30: 2nd IP configured on WiFi ONT

service-policy input TDIA-ILL-IN

## Scenario 4 – WiFi Router as NAT+GW

Similar to Scenario 3 but with a separate WiFi Router (customer-owned or TFiber-provided). One public IP from /30 goes to the WiFi Router WAN port; BDI IP is the GW.

## Scenario 5 – Customer CPE on Fiber Last Mile (Single/Dual LAG)

Customer has own CPE connected directly via fiber. BW policing (CIR) and shaping applied on the GP/Mandal router interface. Both input QoS + output shaping policies applied on BDI.

interface BDI3302

vrf forwarding TDIA-ILL

ip address X.X.X.X 255.255.255.252   ! /30 WAN IP

service-policy input TDIA-ILL-IN   ! CIR + QoS

service-policy output TDIA-ILL-OUT  ! CIR shaping

ip route vrf TDIA-ILL Y.Y.Y.Y 255.255.255.X CPE-WAN-IP

Example QoS policies for 100 Mbps:

Policy-map TDIA-ILL-IN

Class default

set qos-group 2  |  set mpls experimental imposition 2

police cir 100000000 bc 2000000 conform-action transmit exceed-action drop

Policy-map TDIA-ILL-OUT

Class default

shape average 100000000 1000000

## Scenario 6 – Customer eBGP with TFiber (Customer Owns Public IPs)

Customer has own public IP pool and AS number. PE-CE eBGP is configured with strict prefix filtering. Customer IP pools are announced toward upstream ISP. Route objects must be created in the Internet Routing Registry (IRR).

router bgp 141688

vrf TDIA-ILL

neighbor Y.Y.Y.Y   ! Customer CPE BGP peer

remote-as ZZZZZ  |  description *** EBGP with Customer ***

route-policy CUSTOMER-ILL-IN in

route-policy CUSTOMER-ILL-OUT out

prefix-set TDIA-ILL_ALLOW-IN

X.X.X.X/XX   ! Customer public IP pools to accept

end-set

route-policy TDIA-ILL-IN

if destination in TDIA-ILL_DEFAULT-IN then drop   ! Block default from customer

if destination in TDIA-ILL_ALLOW-IN then pass

else drop

end-policy

# 6.6. IP Address Management

## 6.6.1 Public IP Allocation

NSO/SCCM manages all IP address allocation for T-DIA ILL services. TFiber's public IP block 103.162.4.0/23 is aggregated at the ZCR HUB and announced to the upstream ISP via eBGP.

| Allocation Type | Subnet Size | Description |
| --- | --- | --- |
| WAN Link (P2P) | /30 | 2 usable IPs: PE BDI + Customer CPE/ONT WAN |
| Customer LAN (small) | /29 | 6 usable IPs; GW on PE BDI, rest for customer devices |
| Customer LAN (larger) | /28 or larger | Enterprise customers needing more public IPs |
| TFiber Public Pool | 103.162.4.0/23 | Aggregated and announced to ISP from ZCR HUB |
| PoC Test Subnet | 103.162.5.252/30 | Used for ILL PoC verification in test lab |

## 6.6.2 Internal IP Ranges (SCCM/NSO Infrastructure)

| Pool | Purpose |
| --- | --- |
| 172.22.0.0/16, 172.23.0.0/16 | External service IPs |
| 172.24.0.0/16, 172.25.0.0/16 | Internal service IPs |
| 172.28.0.0/16 | NSO, SCCM, OLT management IPs (in-band) |

# 6.7. QoS Design

## 6.7.1 QoS Classification

All T-DIA ILL traffic is classified into the DATA queue (QoS group 2, MPLS EXP 2) at the ingress of the GP/Mandal router spoke and honored throughout the MPLS core network.

## 6.7.2 Ingress QoS – Spoke (IOS-XE)

class-map match-all ILL_Services_IN

match Vlan 3302                ! Matches VLAN on GPON last mile

Policy-map ILL_Services_IN

Class ILL_Services_IN

set qos-group 2

set mpls experimental imposition 2

## 6.7.3 Ingress QoS – HUB (IOS-XR)

class-map match-all ILL_Services_IN

match all                      ! All traffic on HUB VRF interface

Policy-map ILL_Services_IN

Class ILL_Services_IN

set qos-group 2

set mpls experimental imposition 2

## 6.7.4 Bandwidth Enforcement

| Last Mile Type | Enforcement Point | Method |
| --- | --- | --- |
| GPON (Nokia/Tejas) | OLT | CIR=PIR=Subscribed BW via OLT bandwidth profile; no policing on GP router |
| Fiber (single/dual LAG) | GP/Mandal Router | Input policing (CIR + burst) + Output shaping on BDI/port |

# 6.8. Service Verification Procedures

After NSO pushes the configuration and Playbook post-checks run, the following verification steps are executed to confirm end-to-end ILL service availability:

## Step 1 – HUB Routing Table Verification (ZCR)

RP/0/RSP0/CPU0:NOC507RTRN00001#show route vrf INTERNET

Expected: Default 0.0.0.0/0 via ISP (42.104.117.194), TFiber public 103.162.4.0/23 locally connected, Spoke /30 subnets via BGP nexthop through MPLS.

## Step 2 – Spoke Routing Table Verification (GP Router)

NOCTESTGAC002#show ip route vrf INTERNET_ILL

Expected: Default 0.0.0.0/0 via HUB nexthop (172.17.224.2), TFiber public subnets via BGP, local BDI3302 connected (103.162.5.252/30). Gateway of last resort correctly set.

## Step 3 – ARP Table Verification

NOCTESTGAC002#sho arp vrf INTERNET_ILL

Expected: BDI3302 shows both PE IP (.253) and customer/laptop IP (.254) with their MAC addresses resolved. Interface: BDI3302.

## Step 4 – P2P IP Ping Verification

NOCTESTGAC002#ping vrf INTERNET_ILL 103.162.5.254

Expected: 5/5 success (100%), RTT approximately 4/4/4 ms. Confirms Layer 3 adjacency between PE and customer device.

## Step 5 – Internet Connectivity Ping

NOCTESTGAC002#ping vrf INTERNET_ILL 8.8.8.8

Expected: 5/5 success (100%), RTT approximately 28/30/32 ms. Confirms full path from Spoke through MPLS HUB to ISP to public internet.

## Step 6 – Speed Test (GPON 100 Mbps Profile)

Execute Speedtest.net from customer device (103.162.5.254):

- Download: ~97.87 Mbps (of 100 Mbps subscribed)

- Upload: ~95.43 Mbps

- Provider shown as: Telangana Fiber Grid Corporation

- Confirms OLT bandwidth profile (102400 kbps EIR) is correctly applied

## Step 7 – Looking Glass Verification (NTT AS2914)

From NTT Data Looking Glass (gin.ntt.net), Router: New York, NY – US:

Query: ping 103.162.5.254

Expected: 5/5 success (100%), RTT 281/289/295 ms. Confirms the /30 subnet is globally reachable via TFiber's public IP announcement through the upstream ISP.

Traceroute path: NTT backbone → Singapore POP → India → Vodafone India (42.104.116.132) → TFiber ZCR. Traceroute stops at ILL GW as TFiber disables further tracing internally.

# 6.10. SCCM & NSO System Requirements

| Component | Description | VMs | vCPU | RAM (GB) | Storage (GB) | Version |
| --- | --- | --- | --- | --- | --- | --- |
| SCCM | Worker (PHP/Apache) | 2 | 16 | 64 | 300 | 8.0.91 / 2.4.52 |
| SCCM | RabbitMQ | 3 | 16 | 64 | 300 | 3.9.12 |
| SCCM | Web Server (PHP/Apache) | 2 | 16 | 64 | 300 | SCCM v4.1.1 |
| SCCM | DB (MySQL) | 2 | 16 | 64 | 1500 | 8.0.27 |
| SCCM | Discovery | 1 | 16 | 32 | 300 | v1.0.0 |
| SCCM | Playbook | 1 | 16 | 32 | 300 | v1.1.1 |
| SCCM | Load Balancer (HAProxy) | 2 | 16 | 32 | 300 | 2.2.9 |
| NSO | 2 Physical Servers | – | 36 | 256 | 3600 | NSO 5.7 |

Host OS (Hypervisor): VMware ESXi 7.0.3  |  Guest OS (VMs): Ubuntu 20.04  |  NSO Host OS: RHEL 8.1

7. Re-Try and provision mechanism parameter to check , number of retries.

NSO Transaction Retry and Provisioning Mechanism

NSO uses optimistic concurrency to improve parallelism by allowing transactions to run in parallel. When conflicts occur (rare situations where data read by one transaction is modified by another before completion), NSO employs automatic retry mechanisms for most scenarios.

Covered in this section: retry parameters, conflict detection, and handling mechanisms based on NSO's concurrency model.

7.2. Understanding Transaction Conflicts

7.2.1 What is Optimistic Concurrency?

NSO's optimistic concurrency works on the premise that data conflicts are rare. Transactions run in parallel, and NSO verifies just before commit that all accessed data is still valid. If conflicts exist, the system rejects the transaction.

Since NSO uses optimistic concurrency, which greatly improves parallelism. With this approach, NSO avoids the need for serialization and a global lock to run user code which would otherwise limit the number of requests the system can process in a given time unit.

7.2.2 When Do Conflicts Occur?

A conflict is when a piece of data, that has been read by one transaction, is changed by another transaction before the first transaction is committed.

For a transaction to experience a conflict, both of the following must be true:

- It reads some data that is changed after being read and before the transaction is completed

- It commits a set of changes in NSO

Important Notes:

- Read-only transactions or transactions where nothing is really changed will never conflict

- Multiple write-only transactions won't conflict even when updating the same data nodes (last write wins)

- Multiple transactions reading the same shared data can proceed without conflict

7.3. Conflict Detection

7.3.1 Developer Log Information

When a transaction conflict occurs, NSO logs an entry in the developer log, often found at logs/devel.log or a similar path.

Example Log Entry:

<INFO> 23-Aug-2022::03:31:17.029 linux-nso ncs[<0.18350.3>]: ncs writeset collector:

check conflict tid=3347 min=234 seq=237 wait=0ms against=[3346] elapsed=1ms

-> conflict on: /mysvc-dns read: <<"10.1.2.2">> (op: get_delem tid: 3347)

write: <<"10.1.1.138">> (op: write tid: 3346 user: admin) phase(s): work

write tids: 3346

Key Information Provided:

- Transaction IDs (3347 conflicted with 3346)

- Conflicting data path (/mysvc-dns)

- Value read by first transaction ("10.1.2.2")

- Value written by second transaction ("10.1.1.138")

- User who initiated the other transaction (admin)

- Phase where conflict occurred (work)

- Low-level operations that resulted in the conflict

Important: The transaction that did the read operations gets the error and causes the log entry, while the other transaction doing the write operations already completed successfully.

7.3.2 Error Messages

Python Exception:

ncs.error.Error exception

confd_errno = ncs.ERR_TRANSACTION_CONFLICT

Error text:

"Conflict detected (70): Transaction 3347 conflicts with transaction 3346 started by

user admin: /mysvc:mysvc-dns read-op get_delem write-op write in work phase(s)"

Java Exception:

com.tailf.conf.ConfException

errorCode = com.tailf.conf.ErrorCode.ERR_TRANSACTION_CONFLICT

7.3.3 Conflict Phases

The phase tells which part of the transaction encountered a conflict:

| Phase | Description | When It Occurs |
| --- | --- | --- |
| work | Changes in open transaction before apply | Direct read in code before calling apply() or applyTrans() |
| pre-transform | Service YANG model validation | Validating service input parameters according to the service YANG model |
| transform | Service mapping execution | During service (FASTMAP) or another transform invocation |
| validation | Final configuration validation | YANG validation of final configuration |
| rollback | Rollback file creation | During rollback file creation |

Note: It is possible for a conflict to occur in more than one place, such as the phase transform,work denoting a conflict in both the service mapping code as well as the initial transaction.

Phase information is useful when troubleshooting automatic transaction retries: When the phase includes work, automatic retry is not possible.

7.4. Transaction Commit Stages

Understanding the complete commit process helps identify where conflicts may occur:

1. Service mapping       → In transaction (before global lock)

2. Validation            → In transaction (before global lock)

3. Transforms            → In transaction (before global lock)

4. Take global lock      → Acquire transaction lock

5. Conflict check        → NSO verifies all data reads are still valid

6. Write to CDB          → If no conflicts detected

7. Disseminate to devices → Prepare and commit phases

8. Call kickers/subscribers → Trigger related actions

9. Release global lock   → Transaction complete

7.4.1 Data Tracking

NSO tracks all data reads and writes from transaction start until lock and conflict check, including:

- Service mapping callbacks and XML templates

- Transform and custom validation hooks

- YANG validation (automatic)

- Rollback creation (automatic)

Important: Service mapping, validation, and transforms all happen in the transaction before taking a (global) transaction lock. If reads do not overlap with writes from other transactions, the conflict check passes.

7.5. Automatic Retry Mechanism

7.5.1 When Automatic Retry is Possible

NSO can automatically retry transactions only when NSO controls the logic and can run it again.

Automatic Retry Available:

- transform phase: Service mapping code

- validation phase: Configuration validation

- pre-transform phase: Service YANG validation

- rollback phase: Rollback creation

Automatic Retry NOT Available:

- work phase: External agent logic (Python scripts, NETCONF clients, etc.) - NSO is not in control of and is not aware of the logic in the external agent

7.5.2 How Automatic Retry Works

- Checkpoints: NSO keeps checkpoints for each transaction to restart from the conflicting phase and save itself redoing the work from the preceding phases if possible

- Memory Management: NSO automatically checks if the transaction checkpoint read/write-set grows too large. This allows for larger transactions to go through without memory exhaustion

- Checkpoint Skip Impact:

  - When all checkpoints are skipped: no transaction retries are possible, and the transaction fails

  - When later-stage checkpoints are skipped: the transaction retry will take more time

- Service Conflict Tracking: NSO tracks the conflicting services in order to not schedule them concurrently in the future

This automatic retry behavior is enabled by default.

7.5.3 Service Conflict Optimization

In case of conflicts during service mapping, NSO optimizes the process even further. It tracks the conflicting services to avoid scheduling them concurrently in the future.

7.6. Configuration Parameters

7.6.1 Location

Path: /services/global-settings

Services retries can be configured further or even disabled under this path.

7.6.2 Viewing Service Conflicts

Command:

admin@ncs# unhide debug

admin@ncs# show services scheduling conflict | notab

Example Output:

services scheduling conflict mysvc-servicepoint mysvc-servicepoint

type           dynamic

first-seen     2022-08-27T17:15:10+00:00

inactive-after 2022-08-27T17:15:09+00:00

expires-after  2022-08-27T18:05:09+00:00

ttl-multiplier 1

Field Descriptions:

| Field | Description |
| --- | --- |
| type | static (declared in YANG) or dynamic (learned from conflicts) |
| first-seen | When conflict was first detected |
| inactive-after | When conflict became inactive |
| expires-after | When dynamic entry will expire |
| ttl-multiplier | Time-to-live multiplier |

7.6.3 Dynamic Conflict Behavior

- NSO tracks conflicts automatically

- Conflicts expire after a certain time

- NSO reverts to default scheduling after expiry unless new conflicts occur

Important: Since a given service may not always conflict and can evolve over time, NSO reverts to default scheduling after expiry time.

7.7. Static Conflict Declaration

7.7.1 YANG Model Configuration

You can declare known conflicts in the service YANG model using the conflicts-with parameter under the servicepoint definition:

list mysvc {

uses ncs:service-data;

ncs:servicepoint mysvc-servicepoint {

ncs:conflicts-with "mysvc-servicepoint";

ncs:conflicts-with "some-other-servicepoint";

}

// ...

}

7.7.2 Benefits

- Ensures NSO will never schedule and execute this service concurrently with another service using the specified servicepoint

- Adds a non-expiring static scheduling conflict entry

- Avoids unnecessary occasional retry when the dynamic scheduling conflict entry expires

- Essential for older, non-thread-safe service code that cannot be easily updated to avoid threading issues

7.7.3 Self-Conflict Declaration

Declaring a conflict with itself (e.g., ncs:conflicts-with "mysvc-servicepoint" on the same servicepoint) is especially useful when you have older, non-thread-safe service code that cannot be easily updated to avoid threading issues.

7.8. CLI and JSON-RPC Automatic Rebase

7.8.1 Rebase Mechanism

For the NSO CLI and JSON-RPC (WebUI) interfaces:

- A commit of a transaction that results in a conflict will trigger an automatic rebase and retry

- Only when the resulting configuration is the same despite the conflict

- If the rebase does not resolve the conflict, the transaction will fail

- The conflict can, in some CLI cases, be resolved manually

7.8.2 Developer Log (trace level)

A successful automatic rebase and retry will generate something like the following pseudo log entries:

<INFO> … check for read-write conflicts: conflict found

<INFO> … rebase transaction

…

<INFO> … rebase transaction: ok

<INFO> … retrying transaction after rebase

7.9. Manual Retry Implementation

7.9.1 When Manual Retry is Required

When a transaction fails to apply due to a read-write conflict in the work phase, NSO rejects the transaction and returns a corresponding error. In such a case, you must start a new transaction and redo all the changes.

Why is this necessary?

If code reads data that changes mid-process, the code needs to restart from the beginning or you can end up with a broken system. To guard against such a scenario, NSO needs to be conservative and return an error.

Scenarios requiring manual retry:

- Work-phase conflicts

- CDB subscriber code

- Standalone scripts

- Action implementations

7.9.2 Retry Helpers

Since there is a chance of a transaction failing to apply due to a conflict, robust code should implement a retry scheme. You can implement the retry algorithm yourself, or you can use one of the provided helpers.

Python Helpers:

- Maapi.run_with_retry() method

- @retry_on_conflict() function decorator

Java Helper:

- Maapi.ncsRunWithRetry() method

7.10. Python Manual Retry Examples

7.10.1 Example Code That Can Conflict

with ncs.maapi.single_write_trans('admin', 'system') as t:

root = ncs.maagic.get_root(t)

# Read a value that can change during this transaction

dns_server = root.mysvc_dns

# Now perform complex work... or time.sleep(10) for testing

# Finally, write the result

root.some_data = 'the result'

t.apply()

If /mysvc-dns changes while the code is executing, the t.apply() line fails.

7.10.2 Method 1: Using run_with_retry()

Refactored Code:

def do_provisioning(t):

"""Function containing the actual logic"""

root = ncs.maagic.get_root(t)

# First read some data, then write some too.

# ...

# Finally, return True to signal apply() has to be called.

return True

# Need to replace single_write_trans() with a Maapi object

with ncs.maapi.Maapi() as m:

with ncs.maapi.Session(m, 'admin', 'python'):

m.run_with_retry(do_provisioning)

With Additional Parameters:

m.run_with_retry(lambda t: do_provisioning(t, one_param, another_param))

7.10.3 Method 2: Using @retry_on_conflict() Decorator

from ncs.maapi import retry_on_conflict

@retry_on_conflict()

def do_provisioning():

# This is the same code as before but in a function

with ncs.maapi.single_write_trans('admin', 'python') as t:

root = ncs.maagic.get_root(t)

# First read some data, then write some too.

# ...

# Finally, call apply().

t.apply()

do_provisioning()

Major Benefit: When the code is already in a function, only a decorator needs to be added.

7.10.4 Using with Action Classes

class MyAction(ncs.dp.Action):

@ncs.dp.Action.action

@retry_on_conflict()

def cb_action(self, uinfo, name, kp, input, output, trans):

with ncs.maapi.single_write_trans('admin', 'python') as t:

...

Important Notes:

- The order of decorators is important

- The decorator is only useful when you start your own write transaction in the wrapped function

- This is because the old transaction cannot be used any longer in case of conflict

7.11. Java Manual Retry Examples

7.11.1 Original Code

public class MyProgram {

public static void main(String[] arg) throws Exception {

Socket socket = new Socket("127.0.0.1", Conf.NCS_PORT);

Maapi maapi = new Maapi(socket);

maapi.startUserSession("admin", InetAddress.getByName(null),

"system", new String[]{},

MaapiUserSessionFlag.PROTO_TCP);

NavuContext context = new NavuContext(maapi);

int tid = context.startRunningTrans(Conf.MODE_READ_WRITE);

// Your code here that reads and writes data.

// Finally, call apply.

context.applyClearTr();

}

}

7.11.2 Using ncsRunWithRetry()

The same functionality is available in Java as the Maapi.ncsRunWithRetry() method. Where it differs from the Python implementation is that it expects the function to be implemented inside a MaapiRetryableOp object.

8. Additionally Second configuration of PON port not working to be resolved while already existing customer is in use, We cant go live without this option. This was working when Cisco has done the Demo now its not working.

Initialization (setCommonVariables function)

The variable is populated when specific conditions are met:

- Service is GI-BB

- Topology is NOT Hub (meaning it's a Spoke)

- The ref_id starts with 9.5 or 9.4 (specific service reference IDs)

It queries the database for a previous Spoke topology service order for the same endpoint device that has status='3' (active).

Usage Pattern Throughout the Code

The variable is checked in multiple functions to determine if this is an existing customer adding a new order:

- getVPNId() - If committed order exists, reuses VPN ID from previous order instead of generating new one

- getRDValue() - Reuses Route Distinguisher from previous order

- getVRFName() - Reuses VRF name from previous committed order

- getDeviceMode() - Uses device mode from previous order

- getCircuitid() - Reuses circuit ID from previous order

- getnokiafreeontid() - Uses CVLAN/SVLAN values from previous committed order

Core Logic

When !empty($this->committed_spoke_serviceOrder) is true, the code calls:

getPreviousGIBBCommitedOrderVariableValue($parameterName)

This function retrieves the parameter value (vpn-id, rd, VRF-name, etc.) from the previous committed order and reuses it for the new order, instead of generating or allocating new values.

Purpose

This ensures that when an existing GI-BB customer adds a new Spoke service, they maintain consistency by reusing key network parameters from their previous service order rather than creating duplicates or conflicting values.