The Service Order Creation process in SCCM is performed through a sequence of three API calls (Step 1,2,3). These APIs are executed sequentially to initialize the service order

Step 1:

**Step 1 – Service Order Initialization**

**Purpose:**

The Service Order Initialization API is the first step in the SCCM service provisioning workflow. It creates a new service deployment request and generates a unique Transaction ID. This Transaction ID serves as a reference identifier and is used by subsequent APIs (Step 2 and Step 3) to process the same service order request.

**Description:**

Upon receiving the service order details, SCCM validates the request and creates a transaction entry in the system. A unique Transaction ID is then generated and returned in the API response. This Transaction ID links all subsequent service order creation and provisioning activities.

**Input:**

Basic service order information such as Service Order ID, Customer ID, Service Type, Location, VLAN, and other service attributes.

**Output:**

A unique Transaction ID generated by SCCM.

**Outcome:**

* Service order request is successfully initialized.

* Transaction record is created in SCCM.

* Transaction ID is returned to the calling application.

* The generated Transaction ID is used in Step 2 for service order data submission and Step 3 for service provisioning.

Step 1 payload and response





Step 2 – Service Order Data Submission

Purpose:
The Service Order Data Submission API is the second step in the SCCM service provisioning workflow. It uses the Transaction ID generated in Step 1 and submits all service-specific configuration details required for service creation and provisioning.

Description:
Upon receiving the Transaction ID and service configuration details, SCCM validates the submitted information and associates it with the corresponding service order transaction. The request includes endpoint device details, bandwidth profiles, ONT information, OLT details, and PE device connectivity information required for provisioning the service.

Input:

- Transaction ID generated in Step 1

- Endpoint device count

- Service-specific ID

- Upstream and downstream bandwidth profiles

- ONT details (Model, Serial Number, PON Port, Tagging Information)

- OLT hostname

- PE hostname and PE port

- Other service-specific parameters

Output:
Service configuration details are validated and linked to the Transaction ID for further processing in the provisioning workflow.

Outcome:

- Service order details are submitted against the Transaction ID generated in Step 1.

- Endpoint device and service configuration information are processed by SCCM.

- ONT, OLT, bandwidth profile, and PE connectivity details are validated.

- The service order transaction is updated with provisioning-related information.

- The Transaction ID remains active and is used in Step 3 to initiate service provisioning or scheduling.

Step 2 payload and response


step 3:


step 3 payload and response

Step 3 – Service Provisioning

Purpose:
The Service Provisioning API is the final step in the SCCM service provisioning workflow. It uses the Transaction ID from the previous steps to initiate the service provisioning process and trigger the required downstream configuration activities.

Description:
Upon receiving the Transaction ID, SCCM retrieves the service order information submitted in Steps 1 and 2 and initiates the provisioning workflow. The system validates the service data and executes the required provisioning actions. Once the provisioning request is successfully initiated, SCCM returns a unique service identifier associated with the provisioned service.

Input:

- Transaction ID generated in Step 1

- Action value indicating the provisioning operation

- Service order information previously submitted in Step 2

Output:

- Service provisioning status

- Unique service identifier generated for the provisioned service

Outcome:

- SCCM retrieves the service order details associated with the provided Transaction ID.

- Service provisioning workflow is initiated.

- Required service configuration processing is triggered.

- A unique service identifier is generated for the provisioned service.

- SCCM returns a successful response indicating that the provisioning request has been accepted and processed.

- Generated Service Identifier: A unique service identifier (CKT-L3VPN-GI-BB-SCCM-Tik100566) is generated and returned upon successful initiation of the service provisioning process.This unique identifier can be used to track the provisioned service in subsequent validation and operational workflows.

RESTCONF API Integration with NSO CFS

Purpose

The RESTCONF APIs act as the integration interface between SCCM and NSO CFS. After successful completion of the service provisioning workflow, SCCM generates RESTCONF payloads and forwards them to NSO CFS for automated service orchestration and network configuration deployment.

Description

After successful execution of Step 1 (Service Order Initialization), Step 2 (Service Order Data Submission), and Step 3 (Service Provisioning), SCCM consolidates the information collected throughout the workflow, including service order details, customer information, VLAN configuration, bandwidth profiles, OLT/ONT details, PE device details, VRF configuration, routing information, and QoS policies.

Using this information, SCCM generates two separate RESTCONF payloads and sends them to NSO CFS through the RESTCONF interface:

MPLS L3VPN RESTCONF Payload (Router configuration)

This payload contains MPLS L3VPN service configuration details, including Circuit ID, VPN ID, VRF definitions, interface configuration, VLAN information, QoS policies, routing parameters, and PE device details. The payload is consumed by NSO CFS to orchestrate MPLS L3VPN service deployment on the network infrastructure.

GPON RESTCONF Payload (Nokia/Tejas) (OLT,ONT configuration)

This payload contains GPON access service configuration details, including OLT information, ONT details, ONT serial number, VLAN mapping, bandwidth profiles, subscriber information, and other service-specific parameters. The payload is consumed by NSO CFS to orchestrate GPON service provisioning on Nokia or Tejas OLT infrastructure.

Outcome

- SCCM generates RESTCONF payloads using the data collected during the service provisioning workflow.

- Separate RESTCONF payloads are created for MPLS L3VPN services and GPON access services.

- RESTCONF APIs are triggered towards NSO CFS.

- NSO CFS receives and processes the MPLS L3VPN and GPON service payloads.

- NSO CFS initiates the corresponding service orchestration workflows.

- Service configurations are prepared for deployment on the target MPLS and GPON network infrastructure.

MPLS L3VPN Service Orchestration Request via RESTCONF to NSO CFS


MPLS L3VPN RESTCONF Payload

Purpose

The MPLS L3VPN RESTCONF API is used to transfer MPLS L3VPN service configuration details from SCCM to NSO CFS for service orchestration and configuration deployment on the target MPLS network infrastructure.

Description

After successful completion of Step 1 (Service Order Initialization), Step 2 (Service Order Data Submission), and Step 3 (Service Provisioning), SCCM generates an MPLS L3VPN RESTCONF payload using the information collected throughout the service provisioning workflow.

The payload contains all MPLS service-specific configuration details required for service deployment, including service identifiers, VPN information, PE device details, interface configuration, VLAN information, VRF definitions, QoS policies, and routing parameters. SCCM sends this payload to the NSO CFS RESTCONF endpoint, where NSO CFS processes the request and initiates MPLS L3VPN service orchestration.

Input

The payload is generated using:

- Transaction ID

- Service ID

- Circuit ID

- VPN ID

- Sub Service ID

- Customer VLAN Information

- PE Device Information

- Interface Configuration

- VRF Configuration

- QoS Configuration

- Routing Parameters

- WAN IP Address Information

Output

An MPLS L3VPN RESTCONF payload is generated and forwarded to NSO CFS through the RESTCONF interface for MPLS service orchestration.

Outcome

- SCCM generates the MPLS L3VPN RESTCONF payload using the service order information collected during the provisioning workflow.

- The payload contains service, interface, VLAN, VRF, QoS, and routing configuration details required for MPLS L3VPN deployment.

- The RESTCONF API is triggered towards NSO CFS.

- NSO CFS receives and processes the MPLS L3VPN payload.

- NSO CFS initiates the MPLS L3VPN service orchestration workflow.

- MPLS L3VPN configuration is prepared for deployment on the target PE network devices.

GPON Service Orchestration Request via RESTCONF to NSO CFS


GPON RESTCONF Payload (Nokia/Tejas)

Purpose

The GPON RESTCONF API is used to transfer GPON access service configuration details from SCCM to NSO CFS for service orchestration and configuration deployment on the target Nokia or Tejas GPON network infrastructure.

Description

After successful completion of Step 1 (Service Order Initialization), Step 2 (Service Order Data Submission), and Step 3 (Service Provisioning), SCCM generates a GPON RESTCONF payload using the information collected throughout the service provisioning workflow.

The payload contains all GPON service-specific configuration details required for service deployment, including OLT information, ONT details, ONT serial number, VLAN configuration, bandwidth profiles, subscriber information, and service provisioning parameters. SCCM sends this payload to the NSO CFS RESTCONF endpoint, where NSO CFS processes the request and initiates GPON service orchestration on the target Nokia or Tejas OLT infrastructure.

Input

The payload is generated using:

- Transaction ID

- Service ID

- Circuit ID

- OLT Information

- ONT Information

- ONT Serial Number

- ONT Card and Port Details

- Customer VLAN Information

- Network VLAN Information

- Upstream Bandwidth Profile

- Downstream Bandwidth Profile

- Subscriber Information

- Service-Specific Parameters

Output

A GPON RESTCONF payload is generated and forwarded to NSO CFS through the RESTCONF interface for GPON service orchestration.

Outcome

- SCCM generates the GPON RESTCONF payload using the service order information collected during the provisioning workflow.

- The payload contains OLT, ONT, VLAN, bandwidth profile, and subscriber-specific configuration details required for GPON service deployment.

- The RESTCONF API is triggered towards NSO CFS.

- NSO CFS receives and processes the GPON RESTCONF payload.

- NSO CFS initiates the GPON service orchestration workflow.

- GPON service configuration is prepared for deployment on the target Nokia or Tejas OLT infrastructure.

Service Order Status Verification in SCCM
After successful completion of Step 1 (Service Order Initialization), Step 2 (Service Order Data Submission), Step 3 (Service Provisioning), and subsequent RESTCONF-based service orchestration in NSO CFS, the service configurations are processed by NSO and applied on the target network infrastructure.

Upon successful completion of the orchestration workflow, NSO updates the provisioning status back to SCCM. SCCM then updates the service order status to Committed, indicating that the service provisioning workflow has been successfully completed.

MPLS L3VPN Service Validation in NSO CFS

Purpose

This step is used to verify that the MPLS L3VPN service configuration received from SCCM through the RESTCONF API has been successfully processed and created as a service instance in NSO CFS.

Description

After SCCM generates the MPLS L3VPN RESTCONF payload and sends it to NSO CFS, NSO CFS processes the payload and creates the corresponding MPLS L3VPN service instance.

By logging into the NSO CFS portal and navigating to the MPLS L3VPN Service Manager, the created service can be searched and viewed using the Service ID. Selecting the service and opening the associated Circuit ID allows verification of the service attributes populated from the SCCM RESTCONF payload.

The displayed service information in NSO CFS represents the configuration data received from SCCM and confirms that the service request has been successfully onboarded into the NSO orchestration layer.

Validation Points

Verify that the MPLS L3VPN service instance is available in the NSO CFS Service Manager.

Verify that the Service ID is displayed as:

Tik100566

Verify that the Circuit ID is displayed as:

CKT-L3VPN-GI-BB-SCCM-Tik100566

Verify that the service details displayed in NSO CFS match the values received through the SCCM RESTCONF payload, including:

- Circuit ID

- Sub Service ID

- VPN ID

- Site ID

- Topology Name

- Routing Protocol

- PE Type

Output

The MPLS L3VPN service instance and associated service attributes are displayed in the NSO CFS portal for validation and further orchestration activities.

Outcome

- NSO CFS receives the MPLS L3VPN RESTCONF payload generated by SCCM.

- A corresponding MPLS L3VPN service instance is created in NSO CFS.

- The service can be searched and accessed using the Service ID.

- Service attributes displayed in NSO CFS match the values received through the SCCM RESTCONF payload.

- The successful creation of the service instance confirms that the data sent from SCCM through the RESTCONF interface has been processed by NSO CFS.

- The service instance is available for subsequent synchronization, deployment, and lifecycle management activities within NSO CFS.

MPLS L3VPN Service Instance Validation in NSO CFS Using SCCM RESTCONF Configuration Data

Click on Circuit id and check payload
After the MPLS L3VPN service instance is created in NSO CFS, open the service and click on the Circuit ID. The Configuration Editor displays the detailed service configuration that was received from SCCM through the RESTCONF payload.
Outcome

- The MPLS L3VPN configuration received through the SCCM RESTCONF payload is visible in NSO CFS.

- Device configuration details are successfully associated with the service instance.

- VRF configuration is created and available for verification.

- Static routing configuration is populated in the service instance.

- QoS configuration is applied and available for validation.

- The displayed configuration confirms that the data sent from SCCM through the RESTCONF interface has been successfully processed and stored in NSO CFS.

Interface Configuration Verification in NSO CFS

Description:
After opening the MPLS L3VPN service instance and selecting the required interface row, the Configuration Editor displays the detailed interface configuration received from SCCM through the RESTCONF payload. This view can be used to verify that the interface-related attributes have been successfully populated in NSO CFS.

Outcome:

- Interface configuration received through the SCCM RESTCONF payload is visible in NSO CFS.

- Connected device details are successfully associated with the service instance.

- Interface parameters such as Interface ID, Transaction ID, IP Type, WAN IP Address, and VRF Name are available for verification.

- Service policy and VLAN-related configuration details are populated within the interface configuration.

- The displayed interface configuration confirms that the data sent from SCCM through the RESTCONF interface has been successfully processed and stored in NSO CFS.

MPLS L3VPN Interface Configuration Verification in NSO CFS from SCCM RESTCONF Payload

After opening the MPLS L3VPN service instance and selecting the required interface row (e.g., BDI3301 or Port-channel21),
Outcome:

- Interface configuration received from the SCCM RESTCONF payload is available in NSO CFS.

- Verify that the selected interface is associated with the correct connected device (NOCTESTGAC001).

- Verify that the Transaction ID (5776) matches the service provisioning request.

- Verify interface attributes such as Interface ID, Device Mode, IP Type, VRF Name, WAN IP Address, and Service Policy.

- Verify Customer VLAN information associated with the interface.

- Verify that VLAN ID 3301 is populated as received from the SCCM RESTCONF payload.

- Verify that the Service Instance Number is associated with the interface configuration.

- The displayed interface configuration confirms that the data sent from SCCM through the RESTCONF interface has been successfully processed and stored in NSO CFS.

MPLS L3VPN Interface Configuration Validation in NSO CFS from SCCM RESTCONF Payload





Device Configuration Verification in NSO CFS
After opening the MPLS L3VPN service instance and selecting the required device row, the Configuration Editor displays the detailed device configuration. This view can be used to validate that the routing, VRF, and device-specific parameters

Outcome:

- Device configuration received from the SCCM RESTCONF payload is available in NSO CFS.

- Verify that the device NOCTESTGAC001 is associated with the service instance.

- Verify that the Transaction ID (5776) matches the service provisioning request.

- Verify routing parameters such as BGP ID, Provider AS Number, and IP Type.

- Verify that the VRF Name (GI_BB_SERVICES_NSO) is correctly populated.

MPLS L3VPN Device Configuration Validation in NSO CFS from SCCM RESTCONF Payload




QoS Configuration Verification in NSO CFS
After opening the MPLS L3VPN service instance and navigating to the qos-config section, select the required device row. The Configuration Editor displays the detailed QoS configuration . This view is used to validate that the QoS policies, class maps, and VLAN-related parameters have been successfully populated in NSO CFS.

Outcome:

- QoS configuration received from the SCCM RESTCONF payload is available in NSO CFS.

- Verify that the device NOCTESTGAC001 is associated with the QoS configuration.

- Verify that the QoS Name (COS_BRONZE) is correctly populated.

- Verify that the Transaction ID (5776) matches the service provisioning request.

- Verify that the Class Map Name (GI_VLAN_3301) is configured.

- Verify that the Policy Map Name (Ingress-policy-GI) is applied.

- Verify that the QoS Group Value (1) is populated.

- Verify that the Class Map VLAN (3301) is associated with the service.

- The displayed QoS configuration confirms that the QoS parameters sent from SCCM through the RESTCONF interface have been successfully processed and stored in NSO CFS.

MPLS L3VPN QoS Configuration Validation in NSO CFS from SCCM RESTCONF Payload

VRF Configuration Verification in NSO CFS
After opening the MPLS L3VPN service instance and navigating to the vrf-definitions section, select the required device row. This view is used to validate that the VRF-related parameters have been successfully populated in NSO CFS.

Outcome:

- VRF configuration received from the SCCM RESTCONF payload is available in NSO CFS.

- Verify that the device NOCTESTGAC001 is associated with the VRF configuration.

- Verify that the VRF Name (GI_BB_SERVICES_NSO) is correctly populated.

- Verify that the Transaction ID (5776) matches the service provisioning request.

- Verify that the Address Family (both) is configured for the VRF.

- Verify that the Route Distinguisher (172.17.128.1:93) is populated.

- Verify that the Import Route Target (141688:32) is configured.

- Verify that the Export Route Target (141688:31) is configured.

- The displayed VRF configuration confirms that the VRF parameters sent from SCCM through the RESTCONF interface have been successfully processed and stored in NSO CFS.

MPLS L3VPN VRF Configuration Validation in NSO CFS from SCCM RESTCONF Payload



GPON Service Instance Validation in NSO CFS

After the GPON RESTCONF payload is processed by NSO CFS, navigate to the Nokia Service Manager and verify that the GPON service instance has been created. Search for the Service ID and open the service configuration to validate the parameters received from the SCCM RESTCONF payload.

Verify that the following Service ID: Tik100566-GPON-180626120031
Outcome

- Verify that the GPON Service ID Tik100566-GPON-180626120031 is created in NSO CFS.

- Verify that the Circuit ID CKT-L3VPN-GI-BB-SCCM-Tik100566 is associated with the service.

- Verify that the Nokia OLT device nokia-ams-pkga is populated.

- Verify that the Template Name ORG_SERV_U_D_CVLAN is associated with the service instance.

- Verify that the service status is displayed as ACTIVE.

- Verify that the ONT Serial Number TJNW95D2CD14 is populated.

- Verify that the Upstream Bandwidth Profile UP_TNET_GI_50M is populated.

- Verify that the Downstream Bandwidth Profile DN_TNET_GI_50M is populated.

- Verify that the Subscriber IDs are populated as received from the SCCM RESTCONF payload.

- Verify that the Customer VLAN 2537 is configured.

- Verify that the Network VLAN GI-Internet_S3301 is configured.

GPON Service Instance Validation in NSO CFS from SCCM RESTCONF Payload

Device Verification in NSO RFS

Description:
After successful service orchestration from NSO CFS, verify the target device in the NSO RFS Device Manager. Search for the device NOCTESTGAC001 and open the device configuration to validate that the service-related configuration has been associated with the device and is available for deployment and synchronization operations.

Outcome:

- Verify that the device NOCTESTGAC001 is available in the NSO RFS Device Manager.

- Verify that the device IP Address 172.17.128.1 and Port 22 are configured.

- Verify that the device type is displayed as Cisco IOS CLI.

- Verify that service-related entries are visible under the Service List section.

- Verify that MPLS L3VPN service configurations associated with the device are listed.

Target Device Validation in NSO RFS for MPLS L3VPN Service Deployment