- Contents

Contents	3

List of Tables	5

1 Introduction	6

1.1 Preface	6

1.2 Audience	6

1.3 Scope	6

1.4 Related Documents	6

2 Cisco ASR 9910 Acceptance Test Plan	7

2.1 Overview	7

2.2 Test Checklist	7

2.3 Node Details	7

2.3.1 ASR 9910 Test Result	7

3 ASR 9910 Logical Test Cases	9

3.1 Router Hostname Verification	9

3.2 IP Address Configurations Verification	10

3.3 IS-IS Configuration Verification	11

3.4 MPLS Configuration Verification	12

3.5 BGP Configuration Verification	13

3.6 QoS Configuration Verification	14

3.7 Segment Routing Configuration Verification	15

3.8 NTP Configuration Verification	16

3.9 ISE Configuration Verification	17

3.10 EPNM Configuration Verification	18

3.11 Syslog Configuration Verification	19

3.12 ACL Configuration Verification	20

3.13 MTU Configuration on interfaces Verification	20

3.14 Validation of unused port	21

3.15 Licenses Verification	22

3.16 Validation of procured SFP	23

3.17 Software verification and Version Check	24

3.18 Hardware and Power Check	25

3.19 RP Cards and High Availability	26

3.20 ISIS status validation	27

3.21 MPLS status validation	28

3.22 NTP synchronization with master NTP validation	29

3.23 BGP status validation	30

3.24 BFD status validation	31

3.25 IP reachability with ISE Server validation	32

3.26 IP reachability with EPNM Server validation	33

3.27 IP reachability with FTP Server validation	34

3.28 IP reachability with Syslog Server validation	35

3.29 IP reachability with TNOC validation	36

3.30 Validation of CRC errors on interfaces	37

3.31 Validation of CPU & Memory Utilization	38

3.32 Router 72 hours up time/stability	39

3.33	Vlan information	40

1 Appendix: Acronym Listing or Full Glossary	41

Document Acceptance	42

- List of Tables

Table 2-1: ASR 9910 - Test Result	7

Table 2 Router Hostname Verification	9

Table 3 IP Address Configurations Verification	10

Table 4 IS-IS Configuration Verification	11

Table 5  MPLS Configuration Verification	12

Table 6 BGP Configuration Verification	13

Table 7QoS Configuration Verification	14

Table 8  Segment Routing Configuration Verification	15

Table 9  NTP Configuration Verification	16

Table 10 ISE Configuration Verification	17

Table 11 EPNM Configuration Verification	18

Table 12 Syslog Configuration Verification	19

Table 13 ACL Configuration Verification	20

Table 14  MTU Configuration on interfaces Verification	21

Table 15 Validation of unused port	22

Table 16 Licenses Verification	23

Table 17 Validation of procured SFP	24

Table 18 Software verification and Version Check	25

Table 19 Hardware and Power Check	26

Table 20 RP Cards and High Availability	27

Table 21   ISIS status validation	28

Table 22 MPLS status validation	29

Table 23 NTP synchronization with master NTP validation	29

Table 24  BGP status validation	30

Table 25 BFD status validation	31

Table 26 IP reachability with ISE Server validation	32

Table 27  IP reachability with EPNM Server validation	33

Table 28  IP reachability with NNMi (OSS) Server validation	34

Table 29  IP reachability with Syslog Server validation	35

Table 30 IP reachability with TNOC validation	36

Table 31 Validation of CRC errors on interfaces	37

Table 32  Validation of CPU & Memory Utilization	38

# Introduction

## Preface

The objective of this document is to provide the Acceptance Test Plan (ATP) for Cisco ASR 9910 router which will act as Zonal & DC WAN router in the T-FIBER network.

The tests will demonstrate that Cisco ASR 9910 router has been correctly installed and configured and it will operate in a manner which will enable T-FIBER to accept it as a working system.

## Audience

The document is intended for:

T-Fiber Project Engineering Team

L&T (T-Fiber Engineering & Operations)

Cisco CX Team

## Scope

The scope of this ATP document is limited to the verification of the configuration and operation of ASR 9910 router.

## Related Documents

High Level Design (HLD)

Low Level Design (LLD)

GCT

# Cisco ASR 9910 Acceptance Test Plan

## Overview

The purpose of the ATP tests in this document is to assess the overall logical functionality of Cisco ASR 9910 router deployed for T-FIBER network.

Any issues that arise because of the ATP Test execution will be entered in the Project Issues Log.

## Test Checklist

The following table provides a master checklist of the ATP test cases to be carried out. Test results should be recorded on the individual Test sheets and the master schedule.

## Node Details

| Hostname | Application | IP Address | Software Version |
| --- | --- | --- | --- |
| Cisco Datacenter DCWAN-1 NOC507RTRN00001 | ASR-9910 | 172.28.64.127 | Version 24.4.2 |

### ASR 9910 Test Result

Table 2-1: ASR 9910 - Test Result

| Test No. | Test Description | Pass/Fail Result |
| --- | --- | --- |
| Router Configuration Verification |  |  |
|  | ##### Router hostname configuration | Pass R Fail ☐ |
|  | ##### IP address configuration on interfaces | Pass R Fail ☐ |
|  | ##### IS-IS configuration | Pass R Fail ☐ |
|  | ##### MPLS (LDP) configuration | Pass R Fail ☐ |
|  | ##### BGP configuration | Pass R Fail ☐ |
|  | ##### QoS configuration | ##### Pass R Fail ☐ |
|  | ##### Segment Routing configuration | ##### Pass R Fail ☐ |
|  | ##### NTP configuration | ##### Pass R Fail ☐ |
|  | ##### ISE configuration | ##### Pass R Fail ☐ |
|  | ##### EPNM configuration | ##### Pass R Fail ☐ |
|  | ##### Syslog configuration | ##### Pass R Fail ☐ |
|  | ##### ACL configuration | ##### Pass R Fail ☐ |
|  | ##### MTU configuration on interfaces | ##### Pass R Fail ☐ |
|  | ##### Validation of unused port | ##### Pass R Fail ☐ |
| Router Hardware, Software and License Verification |  |  |
|  | ##### Validation of Procured Licenses | ##### Pass R Fail ☐ |
|  | ##### Validation of Procured SFP | ##### Pass R Fail ☐ |
|  | ##### Software verification and Version Check | ##### Pass R Fail ☐ |
|  | ##### Validation of Hardware and Power | ##### Pass R Fail ☐ |
| ##### Router Resiliency Tests |  |  |
|  | ##### Validation of RSP and High Availability | ##### Pass R Fail ☐ |
| ##### Router Operation verification |  |  |
|  | ##### Validation of ISIS status | ##### Pass R Fail ☐ |
|  | ##### Validation of MPLS (LDP) status | ##### Pass R Fail ☐ |
|  | ##### Validation of NTP synchronization with master NTP | ##### Pass R Fail ☐ |
|  | ##### Validation of BGP Session with neighbors | ##### Pass R Fail ☐ |
|  | ##### Validation of BFD Session with neighbors | ##### Pass R Fail ☐ |
|  | ##### Validation of IP reachability with ISE Server | ##### Pass R Fail ☐ |
|  | ##### Validation of IP reachability with EPNM Server | ##### Pass R Fail ☐ |
|  | ##### Validation of IP reachability with FTP Server | ##### Pass R Fail ☐ |
|  | ##### Validation of IP reachability with Syslog Server | ##### Pass R Fail ☐ |
|  | ##### Validation of IP reachability with NOC | ##### Pass R Fail ☐ |
|  | ##### Validation of CRC errors on interfaces | ##### Pass R Fail ☐ |
|  | ##### Validation of CPU & Memory Utilization | ##### Pass R Fail ☐ |

# ASR 9910 Logical Test Cases

Section - Device Configuration Verification

## Router Hostname Verification

| Test No. | 3.1 | Revision: | 1.0 | Author: | Cisco CX | Date: |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Category: |  | ASR 9910 Test Cases |  |  | Standard/Optional: | Standard |  |
| Test Title: |  | - Router hostname configuration |  |  |  |  |  |
| Test Purpose: |  | To verify the hostname configured on the router |  |  |  |  |  |
| Procedure: |  | - Will execute the command on CLI: show run hostname - Above command output will be parsed and Hostname will be extracted from the output |  |  |  |  |  |
| Checks: |  |  |  |  |  |  |  |
| Hostname configured on the router will be matched with hostname |  |  |  |  |  | Pass R Fail ☐ |  |
| Expected Results: |  | Hostname configured on Router should matched with hostname as per standard |  |  |  |  |  |
| Passed: (initial) |  |  |  |  |  |  |  |
| Failed: (initial) |  |  |  |  |  |  |  |
| Reason for Failure: |  |  |  |  |  |  |  |
| Remarks: |  | Verified and Logs captured |  |  |  |  |  |

Table 2 Router Hostname Verification

## IP Address Configurations Verification

RFP cause No : CR.RQ.035

| Test No. | 3.2 | Revision: | 1.0 | Author: | Cisco CX | Date: |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Category: |  | ASR 9910 Test Cases |  |  | Standard/Optional: | Standard |  |
| Test Title: |  | - IP address configuration on interfaces |  |  |  |  |  |
| Test Purpose: |  | To verify that IP addresses configured on router |  |  |  |  |  |
| Procedure: |  | - Will execute the command on CLI: show ipv4/v6 interfaces brief - Above command output will be parsed and Interfaces and IP addresses will be extracted from the output |  |  |  |  |  |
| Checks: |  |  |  |  |  |  |  |
| IP addresses configured on router respective interfaces |  |  |  |  |  | Pass R Fail ☐ |  |
| Expected Results: |  | IP addresses configured on respective interfaces |  |  |  |  |  |
| Passed: (initial) |  |  |  |  |  |  |  |
| Failed: (initial) |  |  |  |  |  |  |  |
| Reason for Failure: |  |  |  |  |  |  |  |
| Remarks: |  | Verified and Logs captured |  |  |  |  |  |

Table 3 IP Address Configurations Verification

## Router 72 hours up time/stability

| Test No. | 3.32 | Revision: | 1.0 | Author: | Cisco CX | Date: |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Category: |  | ASR 9910 Test Cases |  |  | Standard/Optional: | Standard |  |
| Test Title: |  | Node uptime should be more than 72 hours |  |  |  |  |  |
| Test Purpose: |  | To verify that recommended software is installed on the router |  |  |  |  |  |
| Procedure: |  | Will execute the command on CLI: show version Will execute the command on CLI: sh version Above command output will be parsed and show uptime of Node |  |  |  |  |  |
| Checks: |  |  |  |  |  |  |  |
| Node will show up time duration |  |  |  |  |  | Pass R Fail ☐ |  |
| Expected Results: |  | Router will show uptime details should be more than 72 hrs |  |  |  |  |  |
| Passed: (initial) |  |  |  |  |  |  |  |
| Failed: (initial) |  |  |  |  |  |  |  |
| Reason for Failure: |  |  |  |  |  |  |  |
| Remarks: |  | Verified and Logs captured |  |  |  |  |  |

Table 33 node uptime more than 72 hours

| Test No. |  | 3.32 | Revision: | 1.0 | Author: | Cisco CX | Date: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Category: |  |  | ASR 9910 Test Cases |  |  | Standard/Optional: | Standard |  |
| Test Title: |  |  | Vlan configuration details |  |  |  |  |  |
| Test Purpose: |  |  | To verify that vlan information |  |  |  |  |  |
| Procedure: |  |  | Will execute the command on CLI: show run interface <BE.XX> Above command output will be parsed and show vlan tagged information |  |  |  |  |  |
|  | Checks: |  |  |  |  |  |  |  |
|  | Node will show up time duration |  |  |  |  |  | Pass R Fail ☐ |  |
| Expected Results: |  |  | Router will show vlan information |  |  |  |  |  |
| Passed: (initial) |  |  |  |  |  |  |  |  |
| Failed: (initial) |  |  |  |  |  |  |  |  |
| Reason for Failure: |  |  |  |  |  |  |  |  |
| Remarks: |  |  | Verified and Logs captured |  |  |  |  |  |

## Vlan information

Table 34 Vlan information details

# Appendix: Acronym Listing or Full Glossary

| Term | Definition |
| --- | --- |
| DCN | Data Center Network |
| ATP | Acceptance Test Plan |
| FTP | File Transfer Protocol |
| BGP | Border Gateway Protocol |
| EPNM | Evolved Programmable Network Manage |
| NTP | Network Time Protocol |
| PI | Prime Infrastructure |
| ISE | Identity Service Engine |
| ACL | Access control list |
| MPLS | Multiprotocol Label Switching |
| LDP | Label Distribution Protocol |
| QoS | Quality of Service |

- Document Acceptance

| Organization | MSI Representative | TPA Representative | T-Fiber Representative |
| --- | --- | --- | --- |
| Signature |  |  |  |
| Name | Ranganadham Srinivasa Rao | Umamaheshwar Gaddam | Sanjay Kumar Kaligotla |
| Designation | (SME IP/MPLS – L&T) | TPA - Expert | Asst Director Noc (T-FIBER) |
| E-Mail ID | Ranganadham.Srinivasarao@Ltts.Com | Umamaheshwar.Gaddam@Arcadis.Com | AD-UT-Tfiber@ Telangana.Gov.In |
| Contact | +91 8885582200 | +919848002082 | +91 95051 77477 |
| Dated |  |  |  |