SCCM ACCEPTANCE TEST PLAN

LSA Deployment — GPON Service Provisioning (CFS / RFS Cluster)

CLI and Web UI Test Procedures — Tejas EMS Integration

Revision History

| Rev. No. | Date of Submission | Description of Revision | Prepared By | Date Submitted for Review | Review Completed Date |
| --- | --- | --- | --- | --- | --- |
| 1.0 | 07-Jul-2026 | Initial version | — | 07-07-2026 | — |

Approval

| Prepared By | Reviewed By | Approved By |
| --- | --- | --- |

# Table of Contents

Table of Contents	1

1. Introduction	3

Site / Node Details	3

2. Test Case Summary	3

3. NSO Service Configuration and Lifecycle Management	5

Test Case 1: Provisioning a New Service Instance	5

Test Case 2: Committing the Service Configuration	5

Test Case 3: Deleting a Service Instance	6

4. The Cluster Architecture — Nodes, Roles, and Topology	7

4.1 Cluster Node Reference Table	7

5. User Interface and System Verification	8

5.1 Web Portal and EMS Verification	8

Test Case 4: RFS Cluster Lock/Unlock — Baseline (All Nodes Unlocked)	8

Test Case 5: RFS Cluster Lock/Unlock — Partial Lock (RFS-18 & RFS-20 Locked)	15

Test Case 6: RFS Cluster Lock/Unlock — Alternate Partial Lock (RFS-18 & RFS-19 Locked)	19

# 1. Introduction

This Acceptance Test Plan (ATP) documents the tests performed on the Cisco NSO (Network Services Orchestrator) platform to evaluate the automation, provisioning, and management functionality provided for the T-Fiber network. It covers the LSA (Layered Service Architecture) deployment across the RFS (Resource Facing Service) and CFS (Customer Facing Service) layers.

Testing included verification of NSO system status, user access, device inventory, package operational status, service configuration, service commit operations, service deletion, and service visibility across CFS, RFS, and Tejas EMS. Additional tests validated service behavior under different RFS cluster lock and unlock scenarios.

## Site / Node Details

| Parameter | Value |
| --- | --- |
| Environment | 172.28.2.17 (CFS – Customer Facing Service) |
| RFS Node | 172.28.2.18 (RFS – Resource Facing Service) |
| Cluster Nodes | RFS-18, RFS-19, RFS-20 |
| Deployment Type | LSA (Layered Service Architecture) – RFS / CFS |
| Field Management System | Tejas EMS |

# 2. Test Case Summary

The table below summarizes each verification step performed. The detailed procedure, expected result, and supporting evidence (screenshots) are documented against each test case in the sections that follow.

| # | Checkpoint | What It Confirms | Type | Result |
| --- | --- | --- | --- | --- |
| 1 | Provisioning a New Service Instance | The required service parameters (device, OLT, ONT, customer ID, VLANs, bandwidth profile) can be successfully configured in NSO. | CLI | PASS |
| 2 | Committing the Service Configuration | The configured service transaction commits successfully in NSO with no errors. | CLI | PASS |
| 3 | Deleting a Service Instance | An existing service instance can be removed from NSO and the deletion commits successfully. | CLI | PASS |
| 4 | Cluster Node Role & Status Verification | RFS-18 (Leader) and RFS-19 / RFS-20 (Followers) are correctly configured and Unlocked. | Web UI | PASS |
| 5 | RFS Cluster Lock/Unlock — Baseline (All Nodes Unlocked) | With all RFS nodes Unlocked, a provisioned service is visible across CFS, RFS, and Tejas EMS, and is fully removed from all three after deletion. | Web UI | PASS |
| 6 | RFS Cluster Lock/Unlock — Partial Lock (RFS-18 & RFS-20 Locked) | Service provisioning and visibility across CFS, RFS, and Tejas EMS remain unaffected when RFS-18 and RFS-20 are Locked and RFS-19 is Unlocked. | Web UI | PASS |
| 7 | RFS Cluster Lock/Unlock — Alternate Partial Lock (RFS-18 & RFS-19 Locked) | Service provisioning and visibility across CFS, RFS, and Tejas EMS remain unaffected when RFS-18 and RFS-19 are Locked and RFS-20 is Unlocked. | Web UI | PASS |

# 3. NSO Service Configuration and Lifecycle Management

This section covers the lifecycle of a service instance within NSO — from initial configuration through commit and eventual deletion — all performed via the command-line interface (CLI).

### Test Case 1: Provisioning a New Service Instance

| Description | Verify that a service instance can be configured in NSO by supplying the required service parameters — device name, OLT details, ONT details, customer ID, VLANs, and bandwidth profile. |
| --- | --- |
| Procedure | 1. Enter the service configuration mode: services SERVICE_TEMPLATE SERVICE_INSTANCE_ID 2. Configure the required service parameters, including: • Device name and package • OLT IP address • ONT serial number, model, and port • C-VLAN and S-VLAN • Customer ID • Bandwidth profile • Provisioning mode and VLAN tagging • PON port details |
| Expected Result | The configuration is accepted and the provided parameters are displayed within the NSO configuration session. |
| Result | ☐ OK ☐ Not OK |
| Logs / Snapshots |  |

### Test Case 2: Committing the Service Configuration

| Description | Verify that a configured service transaction can be committed in NSO. |
| --- | --- |
| Procedure | 1. Execute: commit |
| Expected Result | The service configuration transaction is committed successfully. |
| Result | ☐ OK ☐ Not OK |
| Logs / Snapshots | Confirmation that the order committed successfully. |